3GPP TSG-SA2 Meeting #165	S2-2409941

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	KI#4: Event Exposure by SCP

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

This contribution discusses whether an SCP can provide event exposure to help with the detection of signalling storms. Related changes are proposed in S2-2409942 and S2-2409943.

# Discussion

The conclusions for key issue 4 in TR 23.700-94 contain the following

-	The following Input Data are agreed to be collected from NF, MDAF, AF and/or via NRF to support generation of the analytics by NWDAF:

-	Information related to control plane signalling exchange, including:

-	Request Type of control messages (and related posterior messages), message counts, success and failures, failure cause, fault information, response times for N1 and/or N2 interfaces and 5GC SBIs, taking information as described in clause 6.35.2.2 and table 6.39.1-1.

NOTE 6:	Whether SCP entity itself can be a source of input data will be discussed and specified in normative work.

This contribution discusses whether an SCP can be a source of input data.

The SCP is defined IN TS 23.501 as follows:

### 6.2.19	SCP

The Service Communication Proxy (SCP) includes one or more of the following functionalities. Some or all of the SCP functionalities may be supported in a single instance of an SCP:

-	Indirect Communication (see clause 7.1.1 for details).

-	Delegated Discovery (see clauses 7.1.1 and 6.3.1 for details).

-	Message forwarding and routing to destination NF/NF service.

-	Message forwarding and routing to a next hop SCP.

..

NOTE 2:	Load balancing, monitoring, overload control functionality provided by the SCP is left up to implementation.

…

### 7.1.1	General

…

NF services may communicate directly between NF Service consumers and NF Service Producers, or indirectly via an SCP. Direct and Indirect Communication are illustrated in Figure 7.1.1-1. For more information, see Annex E and clauses 6.3.1 and 7.1.2. Whether a NF Service Consumer (e.g. in the case of requests or subscriptions) or NF Service Producer (e.g. in the case of notifications) uses Direct Communication or Indirect Communication by using an SCP is based on the local configuration of the NF Service Consumer/NF Service Producer. An NF may not use SCP for all its communication based on the local configuration.

NOTE:	The SCP can be deployed in a distributed manner.

In Direct Communication, the NF Service consumer performs discovery of the target NF Service producer by local configuration or via NRF. The NF Service consumer communicates with the target NF Service producer directly.

In Indirect Communication, the NF Service consumer communicates with the target NF Service producer via a SCP. The NF Service consumer may be configured to perform discovery of the target NF Service producer directly, or delegate the discovery of the target NF Service Producer to the SCP used for Indirect Communication. In the latter case, the SCP uses the parameters provided by NF Service consumer to perform discovery and/or selection of the target NF Service producer. The SCP address may be locally configured in NF Service consumer.

Observation 1: In deployments with SCP, the SCP can be a central entity to monitor signalling exchange. It may already contain functionality to detect load and handle overload.

The SCP could thus be a good source for input data for signalling storm related analytics and can possibly also avoid that input data  need to be collected from many different NFs of different types.

The SCP handles messages it passes in a generic manner and is agnostic of service specific logic implied by those messages.

Observation 2: Signalling observations done by the SCP need to avoid service specific aspects.

However, information related to source/destination NFs of messages, related service, ratio of success/failure responses, number/frequency of specific messages can be available at an SCO

From TS 23.502:

Table 5.2.7.1-1: NF services provided by the NRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_NFManagement | NFRegister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, AF (NOTE 2), TSCTSF, EASDF, MFAF, ADRF, DCSF, MF, MRF, MRFP |
|  | NFUpdate | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, TSCTSF, EASDF, MFAF, ADRF, DCSF, MF, MRF, MRFP |
|  | NFDeregister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR, SCP, 5G DDNMF, DCCF, NSACF, MB-SMF, TSCTSF, EASDF, MFAF, ADRF, DCSF, MF, MRF, MRFP |
|  | NFStatusSubscribe | Subscribe/Notify | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
|  | NFStatusNotify |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
|  | NFStatusUnSubscribe |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, TSCTSF, DCCF |
| Nnrf_NFDiscovery | Request | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, AF (NOTE 2), DCCF, MBSF, 5G DDNMF, TSCTSF, MRF |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, UDM, NWDAF, I-CSCF, S-CSCF, IMS-AS, HSS, DCCF |
| Nnrf_Bootstrapping | Get | Request/Response | NRF, any NF or SCP |

Table 5.2.12.1-1: NF services provided by UDR

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Data Management (DM) | Query | Request/Response | UDM, PCF, NEF |
|  | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Update | Request/Response | UDM, PCF, NEF |
|  | Subscribe | Subscribe/Notify | UDM, PCF, NEF |
|  | Unsubscribe |  | UDM, PCF, NEF |
|  | Notify |  | UDM, PCF, NEF |
| GroupIDmap | Query | Request/Response | NRF, SCP |

Observation 3: The SCP is already a consumer of many services

From TS 23.501:

#### 6.2.6.3	SCP profile

SCP profile maintained in an NRF includes the following information:

-	SCP ID.

-	FQDN or IP address of SCP.

-	Indication that the profile is of an SCP (e.g. NF type parameter set to type SCP).

-	SCP capacity information.

-	SCP load information.

-	SCP priority.

-	Location information for the SCP (see locality in clause 6.1.6.2.2 of TS 29.510 [58]).

-	Served Location(s) (see servingScope in clause 6.1.6.2.2 of TS 29.510 [58]).

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	SCP Domain the SCP belongs to. If an SCP belongs to more than one SCP Domain, the SCP will be able bridge these domains, i.e. sending messages between these domains.

NOTE:	Service definition defines optional and mandatory parameters, see TS 23.502 [3].

Observation 4: Mechanisms for the discovery of an SCP via an NRF are available. They might require enhancements to discover services potentially offered by an SCP

Summary of Proposals

Proposal:  Define an SCP event exposure service to allow an SCP to expose events related to a service-agnostic description of characteristics of signalling messages it passes

The events are expected to be consumed by an NWDAF to provide statistics and/or analytics related to signalling storms.