SA WG2 Meeting #S2-165	S2-2409959

14 - 18 October, 2024, Hyderabad, India

Title:	LS Response on Multiple Queries related to untrusted AF influence on Traffic Routing

Response to:	S2-2409587/C3-244442

Release:	Rel-19

Work Item:	TEI19, eEDGE_5GC, 5GS_Ph1_CT

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks for CT3 for the LS (C3-244442) on Multiple Queries related to untrusted AF influence on Traffic Routing. SA2 would like to provide the answers to the questions as following:

Question 1:	Whether there are any impacts of these optimizations on the traffic influence functionality from architectural and/or procedural point of view?

SA2 Answer: In SA2 specification there is no restriction that the AF can only request one set of traffic filters and the corresponding traffic routing requirement. Whether to support multiple set of traffic filters and the corresponding traffic routing requirement within one AF request is per stage 3 decision. However as described in calsue 4.3.6 of TS23.502, The AF traffic influence request information shall be stored in the UDR even if the target is for ongoing PDU Session.

Question 2:	Whether CT3 can proceed with this optimization for future PDU session(s) as a pure stage 3 enhancement in Rel-19?

SA2 Answer: See above.

Question 3:	Whether the outcome of the Traffic Influence request like success or failure be notified to an AF by NEF so that it can take appropriate actions. If yes, can SA2 indicate the signaling path, the success or failure notification will take?  Like, would it be the existing UP path change notification procedure itself?

SA2 Answer: The User Plane Management Events are defined in clause 4.3.6.3 of TS23.502. There is no requirement to notify the outcome of the Traffic Influence request like success or failure to the AF.

Question 4:	Is it within the scope of CT3 to extend the existing notifications to convey both success and failure from NEF to an untrusted AF?

SA2 Answer: see answer above.

2. Actions:

To CT3

ACTION:

SA2 kindly asks CT3 to take the information above into account.

3. Date of Next SA2 Meetings:

SA2#166		Nov 18-22, 2024 				Orlando , US

SA2#166-Adhoc-e	Jan 20-24, 2025 				E-meeting