3GPP TSG-WG2 Meeting #165	S2-2410022

# Hyderabad, India, October 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5692 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for satellite access with regenerative payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The FS_5GSAT_Ph3_ARCH study has concluded to support satellite access with regenerative payload (i.e. satellite with full eNB on-board). While most exsting functionality defined for satellite access in rel-18 applies also to regenerative accessand can be used with no impact to 23.501, a few specific aspects should however be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for regenerative payload. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.1, 5.4.11.X (new), 5.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.4.11	Support for integrating NR satellite access into 5GS

#### 5.4.11.1	General

This clause describes the specific aspects for NR satellite access, including transparent satellite payload (i.e. where the gNB is on ground), and regenerative satellite payload (i.e. with gNB on-board), as defined in TS 38.300 [27]. The description for satellite access in this specification is applicable to both transparent satellite payload and regenerative satellite payload, unless otherwise stated.

**** Next Change ****

#### 5.4.11.x	N2 interface management for satellite access with regenerative payload

The NG Removal procedure defined in TS 38.413 [34] can be used to remove the NG interface between an gNB and an AMF in a controlled manner, e.g. when the gNB on-board a LEO satellite is leaving the service area of an AMF.

Editor’s note: Further updates to this clause are FFS, pending RAN3 outcome.

**** Next Change ****

### 5.7.4	Standardized 5QI to QoS characteristics mapping

Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 |  | 50 | 300 ms (NOTE 11, NOTE 13) | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) |  | 7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 12) |  | 20 | 100 ms (NOTE 10, NOTE 13) | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) |  | 15 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 (NOTE 14) |  | 25 | 50 ms (NOTE 13) | 10-2 | N/A | 2000 ms | V2X messages (see TS 23.287 [121]). A2X messages (see TS 23.256 [136]) |
| 71 |  | 56 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | 10-6 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 |  | 56 | 500 ms (NOTE 11, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 |  | 56 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | Non-GBR | 10 | 100 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | IMS Signalling |
| 6 | (NOTE 1) | 60 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.), AI/ML model download for image recognition (e.g. for model topology) (see TS 22.261 [2]) |
| 7 |  | 70 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming, AI/ML model download for image recognition (e.g. for model weight factors) (see TS 22.261 [2]) |
| 8 |  | 80 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 10 |  | 90 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| 69 (NOTE 9, NOTE 12) |  | 5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 (NOTE 12) |  | 55 | 200 ms (NOTE 7, NOTE 10) | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 |  | 65 | 50 ms (NOTE 10, NOTE 13) | 10-2 | N/A | N/A | V2X messages (see TS 23.287 [121]) |
| 80 |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 82 | Delay-critical GBR | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 83 |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]). Split AI/ML inference - DL Split AI/ML image recognition, (see TS 22.261 [2]) |
| 86 |  | 18 | 5 ms (NOTE 5) | 10-4 | 1354 bytes | 2000 ms | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 87 |  | 25 | 5 ms (NOTE 4) | 10-3 | 500 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 88 |  | 25 | 10 ms (NOTE 4) | 10-3 | 1125 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]), split AI/ML inference - UL Split AI/ML image recognition, (see TS 22.261 [2]) |
| 89 |  | 25 | 15 ms (NOTE 4) | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| 90 |  | 25 | 20 ms (NOTE 4) | 10-4 | 63000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex J are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 4: A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 5: A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 6: A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 7: For Mission Critical services, it may be assumed that the UPF terminating N6 is located "close" to the 5G_AN (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence a static value for the CN PDB of 10 ms for the delay between a UPF terminating N6 and a 5G_AN should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that 5QI-65 and 5QI-69 are used together to provide Mission Critical Push to Talk service (e.g. 5QI-5 is not used for signalling). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC_IDLE and RRC_INACTIVE mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This 5QI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this 5QI value. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. In case of satellite access with regenerative payload, as described in TS 38.300 [27], the gNB may be configured with a different value for the CN PDB depending on the satellite orbit type (LEO, MEO, GEO, etc), considering the additional delay between the gNB and the 5GC. NOTE 14: This 5QI is only used for transmission of V2X messages as defined in TS 23.287 [121] and transmission of A2X messages as defined in TS 23.256 [136]. NOTE 15: For "live" uplink streaming (see TS 26.238 [76]), guidelines for PDB values of the different 5QIs correspond to the latency configurations defined in TR 26.939 [77]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PER combinations are needed these configurations will have to use non-standardised 5QIs. NOTE 16: These services are expected to need much larger MDBV values to be signalled to the RAN. Support for such larger MDBV values with low latency and high reliability is likely to require a suitable RAN configuration, for which, the simulation scenarios in TR 38.824 [112] may contain some guidance. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms,~21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |

NOTE:	It is preferred that a value less than 64 is allocated for any new standardised 5QI of Non-GBR resource type. This is to allow for option 1 to be used as described in clause 5.7.1.3 (as the QFI is limited to less than 64).

**** End of Changes ****