3GPP TSG-WG SA2 Meeting # 165	S2-2410029

# Hyderabad, IN, 14 - 18 October 2024										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5696 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce MWAB authorization aspect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce MWAB authorization aspect in clause 5.49.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions related to MWAB authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.49.3	Authorization aspects

In order to operate as MWAB, the authorization and control of the MWAB-gNB part and the MWAB-UE part are performed separately.

The authorization and control of the MWAB-UE are performed based on the slice management function as specified in clause 5.15 and are performed by the PLMN/SNPN that the MWAB-UE selects to register and provides the backhaul IP connectivity. During the MWAB-UE registration and PDU session establishment procedures, the AMF serving the MWAB-UE verifies and controls the S-NSSAIs/DNNs for the PDU session(s) for backhaul links based on the subscription data information of the MWAB-UE (i.e. the MWAB-UE is allowed to use certain S-NSSAIs/DNNs to setup PDU sessions for backhaul IP connectivity). The MWAB-UE is informed with allowed S-NSSAIs during registration procedure.

The existing location and time availability for network slice function is used to control if the MWAB-UE is allowed to provide IP connectivity for backhaul links in certain location and time. For time-based control, existing time-based network slice subscription control is reused (including the enhancements in clause 5.15.16). For location-based control, the existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5.and 5.6.5a) can be reused.

The authorization and configuration of the MWAB-gNB is managed by the OAM system of the MWAB Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The MWAB-gNB establishes a secure and trusted connection to the OAM server using the IP connectivity provided by the MWAB-UE via the wireless backhaul links as indicated above.

If the MWAB operation is not authorized (e.g. due to location or time limitation, or other local policy configuration), the MWAB-gNB ensures that all UEs connected via MWAB-gNB are moved into other cells based on mobility procedures as specified in clause 5.49.5.

NOTE 1:	It is assumed that authorization handling for MWAB-gNB part in the OAM system and the allowed S-NSSAIs handling for the MWAB-UE is synchronized based on network management system. When MWAB-gNB moves the connected UEs to other cells, the IP connectivity provided by the MWAB-UE wireless backhaul links are maintained to perform the needed mobility procedures.

When the AMF of the MWAB-UE initiates the removal of S-NSSAIs, the AMF informs the MWAB-UE the new allowed S-NSSAIs and may trigger the PDU session release based on local configured time if there is such, for the PDU sessions related to the S-NSSAIs for wireless backhaul links.

# * * * * End of changes * * * *