SA WG2 Meeting #S2-165	S2-2410059

14 - 18 October, 2024, Hyderabad, India

Title:	[Draft] Reply LS on AIML data collection

Response to:	LS on AIML data collection (S2-2409600/RP-242389)

Release:	Rel-19

Work Item:	NR_AIML_Air, AIML_CN

Source:	SA2

To:	RAN

Cc:	RAN1, RAN2, RAN3, TSG SA, SA3, SA5

Contact person:	Jaewoo Kim

jaewoo1.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

SA2 thanks RAN for the LS on AIML data collection.

SA2 discussed about the possible options 1a, 1b, 2, and 3 for UE data collection for UE-side model training.

For option 1a, since the UE data collection is outside the MNO, Controllability of MNO on data transfer and Visibility of data contents in MNO cannot be satisfied.

For option 1b, mechanism of Data Collection from the UE Application defined in Clause 6.2.8 of TS 23.288 can be leveraged so that the UE transfers training data to the server for data collection. Controllability of MNO on data transfer can be achieved by reusing the DCAF architecture in TS 26.531 and the Server for data collection inside the MNO. And Visibility of data contents in MNO can be achieved by defining data format like format of data reports submitted to the Data Collection AF defined in TS 26.532.

For option 2, NAS message has limited message size. This has limited extendability and can cause signalling congestion. Using UP solution of option 2 can be considered similar to option 1b.

Considering the impact on the existing specification and to ensure the requirements agreed by RAN, SA2 suggests to use a solution based on option 1b, which leverages the existing data collection from the UE application defined in TS 23.288.

2	Actions

To RAN:

ACTION: SA2 kindly asks RAN to take the above information into account.

3	Dates of next TSG SA WG 2 meetings

SA WG2#166			18 – 22 November 2024		Orlando, USA

SA WG2#166-Ad Hoc-e		20 – 24 January 2025		e-meeting (Online)