3GPP TSG-SA2 Meeting #165	S2-2410062

# Hyderabad, India, October 14 – 18, 2024	(revision of S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1249 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and Discovery procedure for VFL When AF is VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusions for KI#2 of TR 23.700-84 and discussions in SA2#164 meeting, the 5GC need to be enhanced to support Registration and Discovery of VFL entities when the AF is VFL server. P#2.2.3 The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s) from NRF for VFL training process. This paper propose a separate procedure for Registration and Discovery procedure for VFL when NWDAFs are the VFL client with an AF is VFL Server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the Registration and Discovery procedure procedure for VFL when NWDAFs are the VFL client with an AF is VFL Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support VFL when the AF is a VFL server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.1, 6.2H.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF, and how AF as a VFL server can register and also discover NWDAFs via NRF.

* * * * Start of 2nd Change * * * *

6.2H.2.X	Registration and Discovery procedure for Vertical Federated Learning among NWDAFs and AF with AF as the VFL server

Figure 6.2H.2.X-1: Registration and Discovery procedure for Vertical Federated Learning when AF is the VFL server and NWDAF(s) are the VFL client

1.	VFL Server AF and VFL Client NWDAFs register to the NRF as described in clause 6.2H.2.1 steps 1 to 3.

Steps 2 to 7 are the NWDAF Discovery procedures when the AF is the VFL server.

2.	The AF as the VFL server determines the ML Model requires VFL based on internal AF process.

If the VFL Server AF is untrusted, the AF requests to discover the NWDAFs as VFL client via the NEF. The following criteria might be used: Analytics ID, and Service Area.

NOTE 1:	In case of a trusted AF, steps 2, 6, and 7 in Figure 6.2H.2.X-1 can be skipped and the AF performs step 3 to 5 to discover the client NWDAF(s) directly with the NRF without involving the NEF.

NOTE 2:	Step 2 (for untrusted AF) or step 3 (for trusted AF) in Figure 6.2H.2.X-1 may be triggered at the VFL Server AF by VFL Server AF itself or a request from a consumer.

Editor's note:	The details of service operation used in step 2 and step 7 are FFS.

3-5.	The NEF discovers other NWDAF(s) as VFL client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, VFL capability type (i.e. VFL client), and Service Area.

6.	For an untrusted AF, the NEF anonymizes NWDAF instance ID(s) into Temporary NWDAF ID(s).

Editor's note:	How to handle Temporary NWDAF ID(s) is FFS.

7.	For an untrusted AF, The NEF respond to VFL Server AF with Temporary NWDAF ID(s) as Clients.

* * * * End of Changes * * * *