3GPP TSG-SA2 Meeting #165	S2-2410077

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1250 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample alignment and sample update for Vertical Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion of KI#2 5GC support for Vertical Federated Learning, the description of sample alignment procedure for VFL is to be described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces procedure description for VFL sample alignment and sample update. The procedure is described in a generic way between VFL client and VFL server and is applicable in both scenarios of AF and NWDAF acting as VFL server. Specific differences are described in the procedure text. The details for the procedure is explained in the accompanying discussoin paper in S2-2410078 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL sample alignment procedure is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### * * * * First change (all new)* * * *

#### 6.2G.2.X Sample alignment and sample update procedure for vertical federated learning

VFL Server

VFL Client

0. VFL client discovery and selection

1 VFL Sample Alignment Request

2 VFL Sample Alignment Confirm

3. VFL Training start

Figure 6.2G.2.X-1 VFL Sample alignment procedure

The figure shows the generic procedure for sample alignments between a VFL server and VFL client. It is applicable for both AF as the VFL server and NWDAF as the VFL server case.

0.  A sample alignment procedure for VFL takes place after the server has performed VFL client discovery and selection.

1. The VFL server sends VFL sample alignment request to the VFL client. The request includes VFL traning session identifier, Analytics ID, initial list of samples (UE IDs). If the AF is the VFL server, this request may be sent through NEF. The VFL server may include optional re-training parameters if the sample alignment request is used for a sample update during the VFL training session.

2. The VFL client confirms the samples and responds with a VFL sample alignment confirm. This includes the status of sample alignment request, accepted list of samples and optionally rejected list of samples. A VFL client may reject a sample due to lack of data or lack of quality of data for the sample.

3. If the VFL server determines that required analytics quality can be achieved with the confirmed samples, it proceeds with further steps in VFL training. Otherwise, it may restart the procedure with a different sample set or different VFL client.

VFL Server

VFL Client

1. Trigger condition for sample update

2 VFL Sample Update Request

3 VFL Sample Update Confirm

4. VFL Training continues

Figure 6.2G.2.X.-2 VFL sample update procedure triggered by VFL client

The figure shows the generic sample update procedure triggered by a VFL client.

1. A trigger condition for sample update occurs in the VFL client. In the case of VFL with AF as the server and NWDAF as VFL Client, it may be due to UE moving out of Area of Interest or change in user consent to allow UE participation in the VFL training process. In the case of VFL with NWDAF as the server and AF as VFL client, this could be when UE no longer has an application session.

2. VFL client sends a VFL sample update request. The request includes VFL Session identifier, sample UE ID(s) to be removed and optionally new sample UE ID(s) to be added. The request may also indicate a previous checkpoint in the training process from which the training needs to restart.

3. VFL server takes the information into account and responds with a VFL sample update confirm. This confirmation includes the VFL Session identifier, updated list of samples and the re-training information including the checkpoint for training restart.

4. VFL training continues from the agreed checkpoint.

#### * * * * End of changes* * * *