3GPP TSG-SA2 Meeting #165	S2-2410096

# 14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1388 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Traffic detection and QoS handling of multiplexed data flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the Conclusions for KI#4 in TR 23.700-70, traffic detection and QoS handling of multiplexed data flows shall be supported in TS 23.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add handling of additional packet filters provided to the UE by SMF - Add new clause to support KI#4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Traffic detection and differentiated QoS is not supported for multiplexed media flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.9, 6.1.3.27.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change* * * *

6.1.3.9	Handling of packet filters provided to the UE by SMF

Traffic mapping information is signalled to the UE by the SMF in the Packet Filter Sets of QoS rules as defined in TS 23.501 [2].

The network shall ensure that the traffic mapping information signalled to UE reflects the QoS Flow binding of PCC rules, except for those extending the inspection beyond what can be signalled to the UE. The PCC rules may restrict what traffic is allowed compared to what is explicitly signalled to the UE. The PCF may, per service data flow filter, indicate that the SMF is required to explicitly signal the corresponding traffic mapping information to the UE, e.g. for the purpose of IMS precondition handling at the UE. In absence of that indication, it is an SMF decision whether to signal the traffic mapping information that is redundant from a traffic mapping point of view.

For QoS Flow for services with no uplink IP flows, there is no need to provide any UL filter to the UE that effectively disallows any useful packet flows in uplink direction.

The default QoS rule will either contain a Packet Filter Set that allows all UL packets or a Packet Filter Set that is generated from the UL SDF filters (and from the DL SDF filters if they are available) which have an indication to signal corresponding traffic mapping information to the UE.

NOTE:	If multiple PCC rules with an indication to signal corresponding traffic mapping information to the UE are bound to the QoS Flow associated with the default QoS rule, it is up to SMF implementation which one will be chosen to generate the default QoS rule. If the PCC rule that is chosen to generate the default QoS rule is removed/deactivated, another PCC rule bound to the QoS Flow associated with the default QoS rule will be used instead and the default QoS rule would be updated accordingly.

In the case of interworking with E-UTRAN connected to EPC, the specific aspects of the handling of packet filters at the SMF are described in clause 4.11.1 of TS 23.502 [3].

For PCC rules share the same legacy packet filter but have different additional Packet Filters, if the SMF has received the UE capability information for supporting additional packet filter, the QoS rules are enhanced to include the additional Packet Filter. If SMF did not receive the UE capability information for supporting additional Packet Filter, SMF generates an explicitly signalled QoS rule (with the legacy packet filter only) for these PCC rules.

# * * * * 2nd change* * * *

6.1.3.27.x	Traffic detection and differentiated QoS Flow mapping for multiplexed media flows

Traffic detection and differentiated QoS Flow mapping for multiplexed media flows is defined in clause 5.37.x of TS 23.501 [2].

UE indicates the support of additional packet filter to SMF at PDU Session establishment and the SMF provides this UE capability information to PCF.

For media flows multiplexed on the same end-to-end transport layer connection, when AF requests a data session with a specific QoS (as described in clause 6.1.3.22), the AF can provide additional Packet filters along with legacy packet filters to identify a media flow. The PCF generates the PCC rules under consideration of the UE capability for the additional Packet Filter and the QoS requirements provided by the AF. If the PCF did not receive the UE capability information, the PCF still generates a PCC rule per media flow but set the same QoS parameters for the PCC rules of media flows share the same legacy packet filter, so that the UPF can treat the UL media flows separately (e.g. for charging and policing purposes). Reflective QoS should not be applied for the PCC rule with additional Packet Filter.

# * * * * End of changes * * * *