3GPP TSG-WG SA2 Meeting #165	S2-2410126

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1477 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of UE-satellite-UE communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AGW reallocation is key to UE-SAT-UE communication - Case#1: In UE-SAT-UE activation procedure: the AGW is reallocated from the ground to the satellite that serves the UE at both the originating and terminating sides. - Case#2: If UE-SAT-UE continues when gNB (satellite) changes: the AGW on the satellite change side is reallocated from the source gNB satellite to the target gNB satellite. - Case#3: If UE-SAT-UE terminates when gNB (satellite) changes: the AGW is reallocated from the satellite back to the ground at both sides. This CR outlines a generalized AGW reallocation procedure for these three scenarios.. The logic of Case#1: Normal call setup can reuse existing procedures, with optional AGW NAT traversal, selecting a ground AGW initially, followed by AGW reallocation to the satellite during UE-SAT-UE activation. The logic of Case#2 and Case#3: If an Xn handover occurs, AGW is reallocated to the target satellite to optimize the path. If an N2 handover occurs, AGW is reallocated to the ground to ensure continuity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4 new clauses regarding procedures: - A new clause to depict AGW reallocation procedures - A new clause to depict how to reuse the AGW reallocation procedure in the activation of UE-SAT-UE communication - A new clause to depict how to reuse the AGW reallocation procedure in satellite change with UE-SAT-UE continues - A new clause to depict how to reuse AGW reallocation procedure in satellite change with UE-SAT-UE terminates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AY.6.1 (new), AY.6.2.1 (new), AY.6.2.2 (new), AY.6.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AY.6 Procedures

### AY.6.1 Activation of UE-satellite-UE communication

This procedure is applied when a UE-SAT-UE is activated, for which the session is re-allocated to the AGWs on the satellites.

Editor’s note:	the depicted procedures regarding AGW reallocation can be simplified/removed once AY.6.3 is agreed.

Figure AY.6.1 Activation of UE-satellite-UE communications

1). UE1 sends SIP INVITE to P-CSCF-1.

Standard call set-up procedures phase:

The standard call setup procedures outlined in clause 5.6.2 for the originating side and clause 5.7.2 for the terminating side can be reused. If an AGW is involved, the procedure described in clause G.4.3 for transport address reservation and configuration can also be applied.

NOTE 1: 	the standard set-up procedure means the call is set with ground AGW(s). The interactions with ground AGW(s) are optional, i.e. when at the terminating side if the called party is behind a NAT and/or when at the originating side if the session initiator is behind a NAT.

2). 5GC1 (PCF1) to P-CSCF-1: Npcf_Policy Authorization Notify (access info, UE1’s satellite ID)

P-CSCF-1 obtains the access information and the UE1’s serving satellite ID from 5GC1 and detects it is a satellite access type.

3). P-CSCF-1 to P-CSCF-2: SIP INVITE (SDP_offer [c=Tr1_A], UE1’s satellite ID).

4). 5GC2 (PCF) to P-CSCF-2: Npcf_Policy Authorization Notify (access info, UE2’s satellite ID)

P-CSCF-2 obtains the access information and the UE1’s serving satellite ID from 5GC2 and detects it is a satellite access type.

6). P-CSCF-2 to P-CSCF-1: 183 Progress (SDP_offer [c=Tr3_B], UE2’s satellite ID).

UE-SAT-UE communication activation phase:

7). P-CSCF-1 re-selects AGW-2 on the satellite

The P-CSCF-1 verifies the connectivity status of the serving satellite(s) between the originating and terminating UEs based on UE1 and UE2’s serving satellite IDs. Once connectivity is confirmed, the P-CSCF-2 selects a new AGW onboard the same gNB-serving satellite (serving UE1), i.e. AGW-2.

8). Skipped, as UE1’s transport address has been received in the INVITE message

9). Same as step 3 in clause AY.6.3

10). P-CSCF-1 to P-CSCF-2: SIP UPDATE (SDP_offer [c=Tr2_A], UE1’s satellite ID).

As a new AGW is selected, the transport address re-negotiation is needed. To trigger it, P-CSCF-1 sends a SIP UPDATE message to P-CSCF-2. The new reserved transport address c=Tr2_A from AGW-2 (satellite), and satellite ID are also included.

11). P-CSCF-2 re-selects AGW-4 on the satellite

The P-CSCF-2 verifies the connectivity status of the serving satellite(s) between the originating and terminating UEs based on UE1 and UE2’s serving satellite IDs. Once connectivity is confirmed, the P-CSCF-2 selects a new AGW onboard the same gNB-serving satellite (serving UE2), i.e. AGW-4. The connectivity can also be confirmed at Step 4).

12-16). Same as steps 6-10 in clause AY.6.3

17). P-CSCF-2 to P-CSCF-1: SIP 200 OK (UPDATE)(SDP_answer [c=Tr2_Tr4_B], UE2’s satellite ID)

18-19). Same as steps 12-13 in clause A.Y.6.3

20). P-CSCF-1 to UE-1: 183 Progress (SDP_answer [c=Tr2_Tr4_B])

21). P-CSCF to 5GC (PCF): Npcf_Policy Authorization Notify (U-S-U indication)

At both the originating side, and the terminating side, as the P-CSCF determines to activate the UE-SAT-UE communication, the 5GC will be informed via Npcf_PolicyAuthorization_Notify with the U-S-U indication and the new transport address reserved with the new AGW onboarding the satellite. The SMF will insert a UL CL/L-PSA as described in clause 4.3.5.4 in TS 23.502 [x] and update the UL CL filters. This step can be executed after Step 9).

16). Same as step 16 in clause AY.6.3

# * * * * Second change * * * *

### AY.6.2 Mobility support of UE-satellite-UE communication

#### AY.6.2.1 UE-SAT-UE continues

This procedure is applied when a UE-SAT-UE is decided to be continued when one of the UE’s gNB onboarded serving satellite changes, for which the session is reallocated to a new AGW on the satellite at the satellite changing UE side. This can be triggered by both the originating side and the terminating side.

Editor’s note:	the depicted procedures regarding AGW reallocation can be simplified/removed once AY.6.3 is agreed.

Figure AY.6.2.1 Procedure of UE-satellite-UE communication continus

Pre-conditions:

0a). UE1 is handed over from a source gNB onboarding satellite to a target gNB onboarding satellite. At the UE1 side, the uplink and downlink data are transmitted via AGW-1 onboard the source satellite.

0b). PCF to P-CSCF-1: Npcf_Policy Authorization Notify (satellite ID, access info)

When the handover is completed, the PCF updates P-CSCF-1 with the new satellite ID and access information to notify P-CSCF-1 that UE1’s serving satellite has changed. If there is no user plane fallback indication, it means that on the 5GC side, the UE-SAT-UE can be continued.

1). The P-CSCF-1 verifies the connectivity status (based on the new satellite ID) between serving satellite(s) of both the originating and terminating sides and once confirmed, the P-CSCF-1 decides to continue UE-SAT-UE communication and selects a new AGW that onboards the same target gNB onboarded satellite, i.e. AGW-2.

2a). P-CSCF-1 to UE-1: SIP NOTIFY (send SIP RE-INVITE indication).

2b). UE1 to P-CSCF-1: SIP RE-INVITE (SDP_offer [c=A])

As a new AGW is selected, the transport address re-negotiation between UE1 and UE2 is needed. To trigger the transport address re-negotiation, P-CSCF-1 sends a SIP NOTIFY message to UE-1 to enable UE-1 to send a RE-INVITE message with an SDP offer. The transport address in the SDP offer can be the same as the one in the SIP INVITE message.

3). Same as step 3 in clause AY.6.3

4). P-CSCF-1 to P-CSCF-2: SIP RE-INVITE (SDP Offer [c=Tr2_A], new satellite ID)

The P-CSCF-1 modifies and forwards the SDP offer with the new reserved transport address, and includes the new satellite ID to P-CSCF-2.

5). If a different satellite ID and a different transport address are received (i.e. UE-SAT-UE communication continues with satellite change at UE1 side), and the connectivity status (based on the new satellite ID) between serving satellite(s) of both the originating and terminating sides is confirmed, the P-CSCF-2 selects AGW-4. The AGW-4 is the UE2-serving AGW.

6-10). Same as steps 6-10 in clause AY.6.3

11). P-CSCF-2 to P-CSCF-1: SIP 200 OK (UPDATE) (SDP_answer [c=Tr2_Tr4_B], satellite ID)

The P-CSCF-2 also includes the satellite ID to notify P-CSCF-1 that the UE-SAT-UE communication is continuing at the UE2 side.

12-14). Same as steps 12-13 in clause AY.6.3

15). P-CSCF-1 to PCF (5GC1): Npcf_Policy Authorization Notify (U-S-U indication, Tr2_A)

As the P-CSCF-1 determines to continue the UE-SAT-UE communication, the 5GC1 will be informed via Npcf_PolicyAuthorization_Notify with the U-S-U continue indication and step 3) reserved transport address with the new AGW. The SMF will change a UL CL/L-PSA as described in clause 4.3.5.6 in TS 23.502 [x] and update the UL CL filter. This step can be executed after Step 3).

16). Same as step 16 in clause AY.6.3

# * * * * Third change * * * *

#### AY.6.2.2 UE-SAT-UE terminates (fall back to ground)

This procedure is applied when a UE-SAT-UE is decided to be terminated, for which the session is reallocated to the AGWs on the ground. This can be triggered by both the originating side and the terminating side.

Figure AY.6.2.2 Procedure of UE-satellite-UE communication fall back to the ground

Editor’s note:	the depicted procedures regarding AGW reallocation can be merged with AY.6.2.1 once AY.6.3 is agreed.

Pre-conditions:

Same as pre-condition as AY.6.2.1, the difference is:

If a user plane fallback indication is included, it means that on the 5GC side, the UE-SAT-UE cannot be continued.

1). P-CSCF-1 decides to terminate UE-SAT-UE communication and selects a new AGW on the ground, i.e. AGW-2.

2-3). Same as steps 2-3 in clause AY.6.2.1

4). P-CSCF-1 to P-CSCF-2: SIP RE-INVITE (SDP Offer [c=Tr2_A])

As a new AGW-2 on the ground is selected, only the transport address is included in the RE-INVITE message to P-CSCF-2.

5). As there is no satellite ID included in the SIP RE-INVITE (UE-SAT-UE is fallen back to the ground at UE1 side), the P-CSCF-2 selects AGW-4 on the ground.

6-10). Same as steps 6-10 in clause AY.6.2.1

11). P-CSCF-2 to P-CSCF-1: SIP 200 OK (SDP_answer [c=Tr4_B])

As a new AGW-4 on the ground is selected, only the transport address is included in the 200 OK message to P-CSCF-1.

12-14). Same as steps 6-10 in clause AY.6.2.1

15). Can be skipped, as UE-SAT-UE is terminated.

16). Same as step 16 in clause AY.6.3

# * * * * Fourth change * * * *

### AY.6.3 AGW reallocation in UE-SAT-UE communications

This procedure is applied when AGW reallocation in UE-SAT-UE communication is needed. This can happen at the terminating side if the called party is behind a NAT and/or at the originating side if the session initiator is behind a NAT. Both cases are handled in the P-CSCF and the IMS Access Gateway as described in this clause.

In case of activation of UE-SAT-UE communication, the AGW-1 and AGW-2 can be regarded as AGWs on the ground and on the satellite separately for the originating UE, the AGW-3 and AGW-4 can be regarded as AGWs on the ground and on the satellite separately for the terminating UE.

In the case of when one of the UE’s gNB onboarded serving satellite changes in UE-SAT-UE communication, and the UE-SAT-UE communication is decided to be continued at the satellite changing UE side, the AGW-1 and AGW-2 can be regarded as AGW on the source satellite and AGW on the target satellite separately for the originating UE, the AGW-4 can be regarded as the AGW on the satellite that is serving the terminating UE.

In the case of when one of the UE’s gNB onboarded serving satellite changes in UE-SAT-UE communication, and the UE-SAT-UE communication is decided to be terminated at the satellite changing UE side, the AGW-1 and AGW-2 can be regarded as AGWs on the satellite and on the ground separately for the originating UE, the AGW-3 and AGW-4 can be regarded as AGWs on the satellite and on the ground separately for the terminating UE.

Figure AY.6.3 AGW reallocation procedures

NOTE 1:	The Transport address refers to both the IP address and Ports (see definition in clause 3.1).

NOTE 2:	The Tri stands for the NAT traversal by AGW-i, where i stands for the index of AGW.

1). 	The P-CSCF-1 re-selects AGW-2 to serve UE1 in the UE-SAT-UE communication.

2). The P-CSCF-1 obtains a SIP message with an SDP offer, where the transport address of UE1’s outgoing media flow is c=A. This step can happen before AGW re-selection.

3). P-CSCF-1 reserves UE1’s outgoing termination T2’s transport address Tr2_A from the AGW-2, where Step 2-4 as described in clause 6.2.1 in TS 23.334 [x] can be reused.

4). P-CSCF-1 to P-CSCF-2: SDP Offer (c=Tr2_A)

The P-CSCF-1 modifies and forwards the SDP offer with the new reserved transport address. Step 5-6 as described in clause 6.2.1 in TS 23.334 [x] can be reused.

5). The P-CSCF-2 (re)-selects AGW-4.

The (re)-selected AGW-4 can be the UE2-serving AGW depending on whether a new AGW is needed at the UE2 side.

6). P-CSCF-2 reserves UE2’s outgoing termination T2’s transport address Tr4_Tr2_A from the AGW-4. Same as step 3).

7). P-CSCF-2 to UE2: SDP Offer (c=Tr4_Tr2_A)

The P-CSCF-2 modifies and forwards SDP offer with the new reserved transport address. Same as step 4).

8). UE2 to P-CSCF-2: SDP answer (c=B)

The P-CSCF 2 receives a SIP message with an SDP answer according to the normal SIP message routing procedures, where the transport address of UE2’s outgoing media flow is c=B.

9). P-CSCF-2 with AGW-4: UE2’s outgoing termination T2, transport addresses (Tr4_Tr2_A, B) configuration

The P-CSCF 2 requests AGW-4 to configure the UE2’s outgoing media flow via Configure AGW Connection Point procedure. The configuration binds (Tr4_Tr2_A, B) as UE2’s outgoing T2. Steps 8-10 as described in clause 6.2.1 in TS 23.334 [x] can be reused.

10). P-CSCF-2 to AGW-4: UE2’s incoming termination T1, transport address Tr4_B retrieval, transport addresses (Tr2_A, Tr4_B) configuration

The P-CSCF 2 requests a transport address for the UE2’s incoming media flow, and AGW-4 reserves Tr_B for the media flow, sends it back to the P-CSCF-2, and configures UE2’s incoming T1 (Tr2_A, Tr4_B). Steps 11-13 in clause 6.2.1 in TS 23.334 [x] can be reused.

11). P-CSCF-2 to P-CSCF-1: SDP answer (c=Tr4_B)

The P-CSCF-2 modifies and forwards SDP answer with the new reserved transport address.

12). P-CSCF-1 to AGW-2: UE1’s outgoing termination T2, transport addresses (Tr2_A, T4r_B) configuration

13). P-CSCF-1 to AGW-2: UE1’s incoming termination T1, transport address Tr2_Tr4_B reservation, transport addresses (A, Tr2_Tr4_B ) configuration

14). P-CSCF-2 to P-CSCF-1: SDP answer (c=Tr2_Tr4_B)

The P-CSCF 1 modifies and forwards SDP offer with the new reserved transport address Tr2_Tr4_B.

15). P-CSCF updates the 5GC user plane filters if applicable.

16). P-CSCF terminates the session with the old AGW-1 and the  old AGW-1 releases the resources, as described in clause G.4.4.

# * * * * End of change * * * *