3GPP TSG-SA2 Meeting #165	S2-2410145

# Hyderabad, India, 14 October - 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1252 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Preparation procedure including sample alignment for AF-initiated VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following conclusion with NOTEs in TR 23.700-84 clause 8.2 needs to be addressed: P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples. NOTE 3: The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase. NOTE 4: How NEF assists sample alignment in P#2.3.3 will be determined in normative phase. NOTE 5: For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2H is updated to address the above conclusion, including the preparation procedure including sample alignment for AF-initiated VFL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-84 (AIML_CN) KI#2 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2H	Vertical Federated Learning among NWDAFs and AFs

6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF. Also, this clause specifies how the preparation (including sample alignment), training, and interference procedures can be performed in the following scenarios:

-	AF-initiated VFL between AF and NWDAF(s) within a single PLMN.

-	NWDAF-initiated VFL among NWDAFs or AF and NWDAF(s) within a single PLMN.

6.2H.2	Procedures

6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning among NWDAFs and/or AFs with NWDAF as the VFL server

Figure 6.2H.2.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1-3.	VFL Server/Client NWDAF and VFL Client AF register to NRF with its NF profile, which includes NF Type (as defined in clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF/AF, Service Area, Time interval supporting FL as described in clause 5.2.

Editor's note:	Whether the FL capability will be extended or a new VFL capability will be defined is FFS.

Editor's note:	Whether the VFL server/client needs to register into NRF any interoperability information is FFS.

Editor's note:	Whether to use Vendor ID(s) or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

For an untrusted AF, the NEF registers at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes other information related to the AF capabilities.

Editor's note:	The content of the AF VFL-related capability is FFS.

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF as the VFL server determines that the ML Model requires VFL based on operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from a consumer.

If the NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, Time Period of Interest, and Service Area.

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, and Service Area.

6.2H.2.X	Preration procedure for Vertical Federated Learning among NWDAFs with AF as the VFL server

This clause specifies the preparation (including sample alignment) procedure for AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2C.2.1-1: Preparation (including sample alignment) procedure for Vertical Federated Learning

- The AF as VFL server discovers and select candidate VFL clients(s) by the discovery procedure described in clause 6.2H.2.1.

  - The AF as VFL server sends VFL preparation request to the candidate VFL client(s) with the ML Preparation Flag. This is done either directly using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service for a trusted AF, or indirectly via NEF using Nnef_VFLPreparation_Subscribe service for an untrusted AF, to check if the VFL client(s) can meet the ML Model training requirement (e.g. Analytics ID, ML Model interoperability information, Sample alignment requirement, Data Availability requirement, VFL Availability time requirement (time span needed for the VFL process), etc.). Sample alignment requirement includes  a list of samples targeted for VFL training as defined by the AF. Data Availability requirement includes a list of Event IDs of the local data necessary for training, ensuring different features for the same samples across each involved NF in the context of VFL feature alignment. It may also include the dataset statistical properties, the time window of the data samples, and the minimum number of data samples. The NEF is only involved for an untrusted AF and maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs). The NEF may sends VFL preparation request to each candidate VFL client using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service when multiple candidate VFL clients are involved.

Editor’s Note:	Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS.

Editor's note:	Whether to use ML Model interoperability information or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is to be determined during the normative phase.

- VFL client(s) checks if it can meet the ML Model training requirement and decides whether to join the VFL process based on operator policy and/or implementation. Example criteria used by VFL client(s) may be based on whether it can satisfy the sample alignment requirement, its data availability, and time availability, and ML Model Interoperability information.

  - VFL client(s) invokes directly Nnwdaf_MLModelTraining_Notify, Nnwdaf_MLModelTraining_Subscribe response, or Nnwdaf_MLModelTrainingInfo_Request response service operation for a trusted AF, or  indirectly via the NEF Nnef_VFLPreparation_Subscribe response service operation for an untrusted AF to indicate to the VFL server whether it will join the VFL process. The response may include a reason if it cannot join the VFL process. When multiple candidate VFL clients are involved for an untrusted AF, the NEF may aggregate the response from each candidate VFL client and send a VFLPreparation_Subscribe response message including the down-selected candidate VFL client(s) that fullfil the ML Model training requirement to the untrusted AF.

- VFL server determines the final VFL client(s) to be involved in the VFL procedures based on the information received from step 6 of the discovery procedure described in clause 6.2H.2.1 and other information available received in step 7.

END CHANGE