SA WG2 Meeting #S2-165	S2-2410175

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5063 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Femto provisioning parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clause 4.15.6.3h, the parameter “Validity time” is described as “If no expiry time is present, the provided CAG information will be valid until UE leaves the CAG cells or the next update is performed.” But it is unclear why “UE leaves the CAG cells” is related to “Validity time”. If no expiry time is present, the CAG(s) in the table is always valid to the UE even it moves out of the CAG cell coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the condition that UE leaves the CAG cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear why “UE leaves the CAG cells” is related to “Validity time” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3h |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.3h	5G Femto CAG provisioning information parameters

The 5G Femto information provisioning parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2. The AF request can be driven by the 5G Femto CAG owner or an authorized administrator. The 5G Femto information provisioning includes the parameters listed in Table 4.15.6.3h-1. According to the received 5G Femto information provisioning parameters at NEF, the NEF updates the UDM with the provided information. The UDM updates the allowed CAG list of the given UE(s) and notifies AMF via Nudm_SDM_Notification Notify message as defined in clause 4.15.6.2. Then, AMF initiates UE Configuration Update procedure to the UE(s) to update the allowed CAG list of the UE(s) along with the provided validity timer as described in clause 4.2.4.2.

NOTE:	The existing CAG concept defined for PNI-NPN in 5.30.3.3 of TS 23.501 [2] is re-used for 5G Femto information provisioning.

Table 4.15.6.3h-1: Description of 5G Femto information provisioning parameters

| 5G Femto information provisioning Parameters | Description |
| --- | --- |
| UE ID(s) | Indicates the unique identifier for the user to be updated with provided CAG information. It can be GPSI or MSISDN. |
| CAG ID(s) | Indicates the CAG ID(s) to be included to the existing UE Allowed CAG list. |
| Validity time | Indicates the expiry time if the CAG information is only valid temporarily. If no expiry time is present, the provided CAG information will be valid until the next update is performed. |

END OF CHANGES