SA WG2 Meeting #S2-165	S2-2410182

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5736 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access control of the UE served by MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_VMR2 conclusion (clause 8.3 of 23.700-06.), this CR proposes to add the Access control of the UE served by MWAB to 23.501 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Access control of the UE served by MWAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Access control of the UE served by MWAB is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.6.1 (new), 5.49.6.2 (new), 5.49.6.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.49.6	Control of UE access to MWAB

#### 5.49.6.1	General

According to operator’s policy, the access control may be applied to UE access to MWAB-gNB.

If the MWAB-gNB is serving a PNI-NPN, the CAG mechanism defined in the clause 5.49.6.2 is used to control the access of UE via MWAB. If the MWAB-gNB is serving a SNPN, the SNPN access control mechanism defined in the clause 5.49.6.3 is used.

If the MWAB-gNB is serving a PLMN, PNI-NPN or SNPN, the existing mechanism e.g. forbidden Tracking Area can be reused by 5GS to manage access control to MWAB-gNB.

#### 5.49.6.2	Control of UE access to an MWAB serving PNI-NPN

The existing CAG mechanism defined in clause 5.30.3 can be used for managing UE's access to MWAB cell, with the following additional considerations:

-	When the MWAB is allowed to operate as an MWAB node for a PLMN, the MWAB is configured, either with the MWAB owner PLMN OAM or (pre-)configuration mechanism, with a CAG identifier which is unique within the scope of this PLMN. If the MWAB is (pre-)configured with the PLMN list in which the MWAB is allowed to operate as MWAB, the corresponding CAG Identifier per PLMN is also configured in the MWAB.

NOTE 1:	The CAG for MWAB is supported as part of the PNI-NPN concept described in clause 5.30.3.

-	NG-RAN and 5GC support the UE access control based on the CAG identifier associated with the MWAB cell and the allowed CAG identifiers for the UE that supports CAG functionality.

-	Time duration restriction may be provided together with the CAG Identifier(s) for the MWAB(s) that the UE can access. The Allowed CAG list will be provided to UE and AMF for enforcement, to make sure that UE not accessing the MWAB cell outside of the time duration. For example, if the time when a certain CAG is allowed for a UE is up, the CAG for the UE is revoked from the network.

NOTE 2:	Control of the MWAB access to the serving network is based on normal mobility restriction management based on subscription data form MWAB-UE.

#### 5.49.6.3	Control of UE access to an MWAB serving SNPN

If the MWAB-gNB is serving a SNPN, the existing SNPN control mechanism defined in clause 5.30.2.5 can be used to manage the UE's access to MWAB-gNB.

END OF CHANGES