SA WG2 Meeting SA2#165	S2-2410198

14-18, October 2024, Hyderabad, IN

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5741 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UIA support of maximum number of Device IDs and traffic association |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One of the objectives of the UIA_ARC WID is to support the optional restriction within the 5GS on maximum number of simultaneously active Device ID per UE/5G-RG for devices whose traffic need to be differentiated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to support this featrue, description about the following aspects are added in TS 23.501, including - General about non-3GPP device identifier - Association between non-3GPP device traffic and non-3GPP device identifier - Restriciton of the maximum number of simultaneously active Device ID per UE/5G-RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction of max number of simultaneously active Device ID can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

### 5.X	Support of Non-3GPP Device Identifier

#### 5.X.1	General

This clause describes the specific aspects for the support of non-3GPP Device Identifier.

To identify a non-3GPP device connecting behind a UE or 5G-RG, a non-3GPP Device Identifier, unique within each UE or 5G-RG, can be allocated by the operator or a third party. The UE or 5G-RG binds the identifier with a non-3GPP device when connected via the UE or 5G-RG, allowing 5G System to distinguish each individual non-3GPP device connected through the same UE or 5G-RG.

The Identifier is used by the network to control the traffic to/from UE or 5G-RG when the traffic is associated with the non-3GPP device.

#### 5.X.A	Association of Non-3GPP Device Traffic and Non-3GPP Device Identifier

The 5GC can associate traffic from individual non-3GPP devices with a Device Identifier and use this identifier to retrieve the appropriate QoS settings for the traffic. Device Identifiers, along with their corresponding QoS policies linked to a UE or 5G-RG subscription, are provisioned in the UDR. This provisioning can be carried out by the AF.

Once a UE or 5G-RG binds the Device Identifier to a non-3GPP device, QoS differentiation can be applied either within a PDU session by using different QoS Flows, or between PDU sessions. When non-3GPP devices share a PDU session, QoS flows are provisioned on the UE or 5G-RG through PDU Session Modification procedures.

The UE or 5G-RG may send the Device Identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message. The SMF will then forward this information to the PCF, which will consider it when making policy decisions based on the Device Identifier data retrieved from the UDR. For non-3GPP devices, the user plane address can vary depending on the PDU session type. For IP PDU sessions, this address can be the UE IP address and/or port ranges (in the case of IPv4) or an IPv6 address. For Ethernet PDU sessions, the user plane address may include the device’s MAC address and/or a VLAN tag ID associated with its traffic.

#### 5.X.B	Maximum Number of Simultaneously Active Non-3GPP Device Identifier

The UE or 5G-RG, with the support of non-3GPP Device Identifier, receives a list of provisioned non-3GPP Device Identifiers and the maximum number of simultaneously active non-3GPP Device Identifiers, based on the UE or 5G-RG’s non-3GPP device information stored in UDR. When supported, the UE or 5G-RG can bind a specific Device Identifier to a particular non-3GPP device.

The UE or 5G-RG shall enforce that a non-3GPP Device Identifier is only actively used by one non-3GPP device at any given time. Upon connection of a non-3GPP device intended to be associated with a non-3GPP Device Identifier, the UE or 5G-RG determines if any available non-3GPP Device Identifiers can be assigned to the newly connected non-3GPP device.

If the maximum number of simultaneously active non-3GPP Device Identifiers is reached, or the UE/5G-RG will not have any available Device Identifiers to assign, the UE or 5G-RG will either deny further non-3GPP device connections requesting non-3GPP Device Identifier or routes the traffic through the PDU Session that matches the default URSP rule based on the policy to support backward compatibility.

***** End of Changes *****