SA WG2 Meeting #165 	S2-2410202

# 14 - 18 October 2024, Hyderabad, India									       rev of S2-2408213

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5546 |  | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices connecting behind a UE/5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Summarize the feature description for the Objective#1 in SP-240971, based on the conclusion in clause 8.4 of TR 23.700-32. |  |  |  |  |  |  |  |  |
| Summary of change: |  | introduce two new clauses: - Summary the feature on general part - Summary the feature on “QoS differentiation for non-3GPP devices connecting behind a UE/5G-RG” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New UIA feature is not captured in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 5.X; (new) 5.X.1; (new) 5.X.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.X	Identifying non-3GPP devices connecting behind a UE/5G-RG.

5.X.1	General

5G system supports to identify the non-3GPP devices connecting behind a UE/5G-RG by using Device Identifier. The Device Identifier is a permanent identifier and will be a generic string. UE/5G-RG will bind the Device Identifier to a non-3GPP device. 5GC is able to associate the traffic from each individual non-3GPP device with a Device Identifier and use the Device Identifier to obtain QoS Settings for the traffic.

The UE/5G-RG shall support the traffic from non-3GPP devices that are not linked with a Device Identifier. Non-3GPP devices that do not need to be identified by 5GC can use the QoS Flow(s) established using Rel-18 Connectivity Group ID mechanisms.

NOTE 1:	The allocation of the Device Identifier is out scope of 3GPP. Authentication for non-3GPP device is performed by UE/5G-RG, which is not in the scope of 3GPP. The connection between non-3GPP devices and UE/5G-RG is not impacted.

NOTE 2:	How UE/RG binds Device Identifier to non-3GPP device is out of scope of 3GPP. When a non-3GPP device connects to the UE/5G-RG, the UE/5G-RG can, based on locally configured policies, bind the traffic from the non-3GPP device to a Device Identifier. For non-3GPP devices with Device Identifiers whose traffic need to be differentiated, in case of NAT, UE/5G-RG will ensure that it does not NAT the traffic from different non-3GPP devices to the same IP address and port number range combinations.

It is implementation to perform restriction on the max number of simultaneously active Device Identifiers per UE/5G-RG.

5.X.2	QoS differentiation for non-3GPP devices connecting behind a UE/5G-RG

QoS differentiation can be performed within a PDU Session (using different QoS Flows) or between PDU Sessions. In the case of non-3GPP devices connecting behind a UE/5G-RG sharing a PDU Session, QoS Flows are provisioned in the UE/5G-RG using PDU Session Modification procedures. Based on the application-layer mechanisms out of 3GPP scope between UE and AF, an AF session with required QoS may be implemented as per clause 4.15.6.6 of TS 23.502 [3] for the corresponding non-3GPP device.

Editor’s note:	Who (the UE/5G-RG and/or the AF) decides to request for change of QoS Flows will be discussed and documented during the normative stage.

For non-3GPP devices connecting behind a UE/5G-RG requiring QoS differentiation, Device Identifiers and their corresponding QoS/Policies associated with a UE/5G-RG subscription are provisioned into the UDR by the AF.

For non-3GPP devices behind a UE/5G-RG requiring QoS differentiation, UE/5G-RG may send the Device Identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message, and the SMF will forward the Device Identifier and user plane address to the PCF. The PCF will take it into account for policy decisions based on the Device Identifier information retrieved from the UDR. Device Identifier is used as a data key to retrieve the information from the UDR.

- When using an IP PDU Session Type, the user plane address is the UE IP Address(es) / Port Number(s) that carry the non-3GPP device's traffic.

- When using an Ethernet PDU Session Type, the user plane address is the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device's traffic.

For non-3GPP devices requiring QoS differentiation, UE/5G-RG may also send the Device Identifier of the non-3GPP device to the SMF in DHCPv6 message to associate the Device Identifier with the user plane address.

Editor’s note:	The details of this solution will be discussed and clarified as part of the normative stage.

* * * End of Changes * * *