3GPP TSG SA WG2 #165	            S2-2410224

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1400 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consistent handling of URSP across 5GS and EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During establishment of non-emergency PDN connection in EPC, if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C selects a S-NSSAI and provides it to the UE. This makes possibility that the SMF+PGW-C selects a S-NSSAI that does not match with the URSP rule for the UE, and it becomes an issue when the UE moves from EPS to 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE indicates the S-NSSAI according to the URSP rule during PDN connection establishment in EPC. - SMF+PGW-C takes into account the S-NSSAI indicated by the UE when select a S-NSSAI for the PDN connection. - When UE moves from EPS to 5GS and the UE received a S-NSSAI from the SMF+PGW-C which is different from the UE indicated to the SMF+PGW-C, the UE shall re-evaluate the URSP rule and proceed corresponding procedures. If there is no more traffic left for the PDN Connection (which is associated with the S-NSSAI not match with URSP rule), the UE may request release of the session to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation in 5GS with EPS interworking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.6.2.3	UE procedure for associating applications to PDU Sessions based on URSP

For every newly detected application/PIN the UE evaluates the URSP rules in the order of Rule Precedence and determines if the application/PIN is matching the Traffic descriptor of any URSP rule. When a URSP rule is determined to be applicable for a given application/PIN (see clause 6.6.2.1), the UE shall select a Route Selection Descriptor within this URSP rule in the order of the Route Selection Descriptor Precedence.

When a valid Route Selection Descriptor is found, the UE determines if there is an existing PDU Session that matches all components in the selected Route Selection Descriptor. The UE compares the components of the selected Route Selection Descriptor with the existing PDU Session(s) as follows:

-	For a component which only contains one value (e.g. SSC mode), the value of the PDU Session has to be identical to the value specified in the Route Selection Descriptor.

-	For a component which contains a list of values (e.g. Network Slice Selection), the value of the PDU Session has to be identical to one of the values specified in the Route Selection Descriptor.

-	When some component(s) is not present in the Route Selection Descriptor, a PDU Session is considered matching only if it was established without including the missing component(s) in the PDU Session Establishment Request.

-	When the Route Selection Descriptor includes a Time Window or a Location Criteria, the PDU Session is considered matching only if the PDU Session is associated with an RSD that has the same Time Window or a Location Criteria Validity Conditions.

When a matching PDU Session exists the UE associates the application/PIN to the existing PDU Session, i.e. route the traffic of the detected application/PIN on this PDU Session.

If the UE determines that there is more than one existing PDU Session which matches (e.g. the selected Route Selection Descriptor only specifies the Network Slice Selection, while there are multiple existing PDU Sessions matching the Network Slice Selection with different DNNs), it is up to UE implementation to select one of them to use.

NOTE 1:	When more than one PDU Sessions of SSC mode 3 to the same DNN and S-NSSAI exist due to PDU Session anchor change procedure as described in clause 4.3.5.2 of TS 23.502 [3], the UE can take the PDU Session Address Lifetime value into account when selecting the PDU Session.

If none of the existing PDU Sessions matches, the UE tries to establish a new PDU Session using the values specified by the selected Route Selection Descriptor. If the PDU Session Establishment Request is accepted, the UE associates the application/PIN to this new PDU Session. If the PDU Session Establishment Request is rejected, based on the rejection cause, the UE selects another combination of values in the currently selected Route Selection Descriptor if any other value for the rejected component in the same Route Selection Descriptor can be used. Otherwise, the UE selects the next Route Selection Descriptor, which contains a combination of component value which is not rejected by network, in the order of the Route Selection Descriptor Precedence, if any. If the UE fails to establish a PDU Session with any of the Route Selection Descriptors, it tries other URSP rules in the order of Rule Precedence with matching Traffic descriptors, except the URSP rule with the "match-all" Traffic descriptor, if any. The UE shall not use the UE Local Configuration in this case.

NOTE 2:	An application can match the Traffic descriptor of different URSP rules and be associated with different PDU Sessions simultaneously.

If a UE receives tuple(s) (PLMN ID, list of PSIs associated with the PLMN ID), the UE uses the URSP rules associated with the PSIs indicated in the tuple(s) as VPLMN specific URSP rules and the UE uses the URSP rules associated with the PSI not indicated in the tuple(s) as non-VPLMN specific URSP rules.

If a UE receives VPLMN specific URSP rules and non-VPLMN specific URSP rules (i.e. the URSP rules which are applicable to both HPLMN and VPLMN), the VPLMN specific URSP rules take precedence over the non-VPLMN specific URSP rules and Local UE Configuration and any other URSP rules provided to the UE. The UE determines VPLMN specific URSP rules to be used taking serving PLMN ID into consideration. If the UE does not find a match to the VPLMN specific URSP rules associated to serving PLMN ID, then the UE uses the VPLMN specific URSP rules associated to the equivalent serving PLMN ID, if any. Otherwise, the UE uses the non-VPLMN specific URSP rules.

The UE receives the updated URSP rules and (re-)evaluates their validities in a timely manner when certain conditions are met, for example:

-	the URSP is updated by the PCF;

-	the UE moves from EPC to 5GC;

-	change of Allowed NSSAI or Configured NSSAI;

-	change of LADN DNN availability;

-	change of PLMN;

-	UE registers over 3GPP or non-3GPP access;

-	UE establishes a connection with a ProSe Layer-3 UE-to-Network Relay;

-	UE establishes connection to a WLAN access.

Details of the conditions are defined by TS 24.526 [19].

NOTE 3:	When providing the updated URSP rules to the UE with a new DNN, the PCF can set the SMF selection management trigger in the AMF to contact the PCF at PDU Session establishment (as specified in clause 6.1.2.5) if the old DNN is requested by the UE.

The Route Selection Descriptor of a URSP rule shall be only considered valid if all of the following conditions are fulfilled:

1)	If any S-NSSAI(s) is present, the S-NSSAI(s) is in the Allowed NSSAI or in the Partially Allowed NSSAI for the non-roaming case and in the mapping of the Allowed NSSAI (or of the Partially Allowed NSSAI) to HPLMN S-NSSAI(s) for the roaming case.

2)	If any DNN is present and the DNN is an LADN DNN, the UE is in the area of availability of this LADN.

3)	If Access Type preference is present and set to Multi-Access, the UE supports ATSSS.

4)	If a Time Window is present and the time matches what is indicated in the Time Window.

5)	If a Location Criteria is present and the UE location matches what is indicated in the Location Criteria.

6)	If ProSe Layer-3 UE-to-Network Relay Offload indication is present and the UE supports the ProSe capability of 5G ProSe Layer-3 Remote UE.

7)	If ProSe Multipath Preference indication is present and the UE supports the ProSe capability of 5G ProSe Layer-3 Remote UE.

If none of the conditions in bullet 1) are met for all the S-NSSAI(s) in the RSD during the validation of the route selection descriptor, the UE shall attempt to meet the condition by requesting any of the S-NSSAI(s) in the RSD through a Mobility Registration Update procedure to attempt to add the S-NSSAI(s) to the Allowed NSSAI (or to the Partially Allowed NSSAI), as specified in clause 5.15.5.2.2 of TS 23.501 [2]. The UE attempts the Mobility Registration Update for a S-NSSAI only if the S-NSSAI is in the Configured NSSAI or, in the roaming case, in the mapping of the S-NSSAIs of the Configured NSSAI for the VPLMN to the corresponding S-NSSAI values of the HPLMN, and any other restrictions to prevent triggering Mobility Registration Update as defined in TS 24.501 [22].

If a matching URSP rule has no valid RSD, the UE tries other URSP rules in the order of Rule Precedence with matching Traffic descriptors, except the URSP rule with "match-all" Traffic descriptor. The UE shall not use the UE Local Configuration in this case.

When URSP rules are updated or their validity according to the conditions above change, the association of existing applications/PINs to PDU Sessions may need to be re-evaluated. The UE may also re-evaluate the application/PIN to PDU Session association due to the following reasons:

-	periodic re-evaluation based on UE implementation;

-	an existing PDU Session that is used for routing traffic of an application/PIN based on a URSP rule is released;

-	The expiration of Time Window in Route Selection Validation Criteria, i.e. the expiration of Time Window, or UE's location no longer matches the Location Criteria.

-	change of PLMN.

NOTE 4:	It is up to UE implementation to avoid frequent re-evaluation due to location change.

If the re-evaluation leads to a change of the application/PIN to PDU Session association, e.g. the application/PIN is to be associated with another PDU Session or a new PDU Session needs to be established, the UE may enforce such changes in a timely manner based on implementation, e.g. immediately or when UE enters CM-IDLE state.

If the UE selected and provided to SMF+PGW-C a S-NSSAI for the APN based on the URSP rule evaluation and received a S-NSSAI from the SMF+PGW-C that is different from the UE provided, the UE shall evaluate the URSP rules when the UE moves from EPC to 5GC. This re-evaluation leads to a change of the application to PDU Session association, and the application needs to be associated with another PDU Session or a new PDU Session needs to be established. The UE may initiate PDU Session Release procedure for the PDN Connection associated with the S-NSSAI different from the URSP rules, if there is no application traffic to be routed to that PDN Connection. See in clause 5.15.7 of TS 23.501 [2].

If the selected Route Selection Descriptor contains a Non-Seamless Offload indication and the UE has established a connection to a WLAN access, the UE routes the traffic matching the Traffic descriptor of the URSP rule via the WLAN access outside of a PDU Session.

If the selected Route Selection Descriptor contains a ProSe Layer-3 UE-to-Network Relay Offload indication and the UE has established a connection with a ProSe Layer-3 UE-to-Network Relay, the UE routes the traffic matching the Traffic descriptor of the URSP rule (including the URSP rule with the "match-all" Traffic descriptor) via the ProSe Layer-3 UE-to-Network Relay outside of a PDU Session.

The 5G-RG and FN-RG procedure for associating applications to PDU Sessions based on URSP is defined in clause 9.5.2 of TS 23.316 [27].

# * * * * End of changes * * * *