3GPP TSG-WG SA2 Meeting #165	S2-2410236

Hyderabad, Oct 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	A new solution: AIoT exposure services

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: a new solution to capture what AIoT related information can be exposed to external AFs

# 1. Introduction/Discussion

This contribution is prepared to propose a new solution for Ambient IoT.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.0.0.

FIRST CHANGE (All text is new)

## 6.x	Solution #x: AIoT exposure services

### 6.x.1	Description

This solution addresses the following aspect of key issue #3 on Support of Ambient IoT Services.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

This solution can be applied for both Topology 1 and 2.

There are many solutions captured in this TR covering NEF service exposure to enable an external AF to make requests to the 5GC for certain AIoT services (e.g., inventory or command). However, event exposure, which provides notifications when specific events occur, can also be considered as part of AIoT services. This can supply useful network information to the AF, which could, for example, be used to optimize the AF's internal logic.

Thus, this solution proposes several new events that are related to AIoT services and can be exposed through NEF to external AFs.

### 6.x.2	Procedures

The following events are proposed.

- AIoT UE reader overload

  - Which NF detects the event: This event is detected by the UE reader itself when it becomes overloaded and can no longer function as an AIoT UE reader due to various factors such as CPU, memory, or battery overload.

  - Description: This event can be useful when the AIoT UE reader is selected by the AF. Once the event is triggered by the AF-selected UE reader, the AF can decide to either select other available UEs that are less loaded or delegate the decision to the 5GC for subsequent communications.

- Interaction latency between AIoT UE reader and AIoT Devices

  - Which NF detects the event: This event is detected by the UE reader itself when it detects the latency is higher than a specific threshold that can be pre-configured or provided by the AF.

  - Description: This event can be useful when the AIoT UE reader is selected by the AF. The AF can take actions to mitigate the latency issues (e.g. switching to another reader or reattempting communication)

- AIoT UE reader positioning capability enabled

  - Which NF detects the event: This event is detected by the UE or BS reader itself whenever the AIoT reader's positioning capabilities are enabled. This can occur, for example, by changing the positioning settings for UEs, due to limited resources at the gNB, or by triggering a positioning request from the LMF or AIoTF to the gNB.

  - Description: This event helps the AF determine whether AIoT device positioning, based on the reader’s positioning capability, is feasible or not.

- Geo-fencing location for AIoT UE reader

  - Which NF detects the event: This event is detected by the AMF when a target UE moves in or out the intended location.

  - Description: This event can be useful when the AIoT UE reader is selected by the AF. When the event occurs, the AF can determine that the target reader UE has reached the desired location and can now be used as a reader, or that the target reader UE is no longer in the location, prompting the need to select another UE as the reader.

To support the aforementioned AIoT-related events, it is proposed to extend the existing NEF event exposure mechanism.

### 6.x.3	Impacts on services, entities and interfaces

NEF:

- Supports extending the existing exposure mechanism to offer the new AIoT related events.

AMF:

- Supports detecting and reporting the geo-fencing location event

AIoT reader:

- Supports detecting and reporting the events of overload, latency and positioning capability enabled

END OF CHANGES