3GPP TSG-WG SA2 Meeting #165	S2-2410248

Hyderabad, India, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Local offloading handling at I-SMF

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	eEDGE_5GC_ph3 / Rel-19

Abstract: This contribution discusses about the how to do the local offloading at the I-SMF.

1. Introduction

In S2#164 meeting for local offloading handling at the I-SMF, due to lack of time there are several open issues. In this paper we analyze those issue and give the related proposal.

2. Discussion

In the revised paper in S2#164 meeting there are some EN related to the local offloading handling. The open issue can be listed as: 1) Offloading policy clarification; 2) how to retrieve the policy from PCF; 3) how to configure the ULCL/L-PSA UPF controlled by the I-SMF;

Issue 1: Offloading policy clarification

The related EN in the revised paper is as following:

- Editor’s Note: It is FFS whether new information element is needed or using which existing IEs in AF request to let PCF generate Local Offloading Policy.

- Editor’s note: Whether the Offload Identifier for the Local Offloading Policy is FFS.

The above EN is related to how to generate the Local Offloading Policy and the usage of Offload ID.

The concept of the local offloading policy is inherited from VPLMN specific Offloading Policy. It is used to indicate which IP range(s) or FQDN(s) can (or cannot) be offload. Considering local offloading policy is associated all traffic UE is permitted (or forbidden) to be offloaded. It should not be restricted to traffic associated with one AF. Hence the Offloading Policy should not be generated by one AF. Also the offloading Policy can be policy set by operator and independent of any AF request, e.g. even without AF request, the local offloading policy should be still possible to be set. Hence the local offloading policy should not rely on to be set by the signalling from AF, i.e. without signalling mechanism the policy should also be possible to be available. In addition, we do not see the necessity to define a procedure to provision the local offloading policy. It can be same as the HR-SBO case, e.g. by configuration.

It is possible that the IP range(s)/FQDN(s) within the Offloading policy are valid only for some dedicated local DC. For example, the EAS(s) (special IP ranges/FQDN) are only deployed in DC-A. The traffic offloading is only necessary when the traffic is gone through via the DNAI-A. When the traffic is not gone through that DNAI-A, the traffic is not required to be offloading. Hence it suggested that the IP range(s) or FQDN(s) within the Offload Policy can be associated with the DNAI information.

To reduce the signaling load the Offload Identifier is introduced to avoid same policy information need be transferred multi times for different user. It is also possible to configure the Offload Identified at the SMF including I-SMF. Hence it is proposed to also introduce the Offload Identified in Local Offloading scenario same as the HR-SBO case.

Proposal 1: the setting of Offloading policy should not rely on the signalling from AF. How to provision the Offloading policy to PCF is not defined.

Proposal 2: The IP range(s) or FQDN(s) within the Offloading Policy can be associated with a DNAI information.

Proposal 3: The Offloading Policy can be associated with one Offload Identifier. And the Offload Identified can be configured at the SMF including I-SMF.

Issue 2: Retrieving policy from PCF

The related EN in the revised paper is as following:

- Editor’s Note:	It is FFS to further define Local Offloading Policy indication granularity.

- Editor’s Note: It is FFS whether the local offloading policy received from PCF is sent to the I-SMF via SMF transparently or not.

- Editor’s Note: 	It is FFS whether SMF transfers transparently Local Offloading Policy to I-SMF.

The above EN is related to how to retrieve policy from PCF and sent to I-SMF.

When the I-SMF is inserted, the I-SMF report the supported DNAI to the SMF. Based on that information, there are two proposals on how to retrieve the policy (e.g., AF traffic influence information, Local Offloading Policy) from PCF and send the policy information to I-SMF:

- Option 1, SMF filter-based solution, the SMF sends the Local Offloading support Indication to the PCF. The PCF sends all Policy information to the SMF. The SMF filters the policy information corresponding to DNAI supported by the I-SMF and sent the related information to the I-SMF.

- Option 2, PCF filter-based solution, the SMF sends both the Local Offloading support Indication and DNAI supported by the I-SMF to the PCF. The PCF filters the policy information corresponding to the DNAI supported by the I-SMF and send the policy information to SMF. SMF forwards the received Policy information to I-SMF transparently.

Table 1: options for policy provision to I-SMF

|  | SMF->PCF | PCF->SMF | SMF->I-SMF |
| --- | --- | --- | --- |
| Option1 | Local Offloading support indication | Policy information | SMF determines the corresponding information, and send related filtered policy information |
| Option2 | Local Offloading support indication, target DNAI | Filtered Policy information corresponding to the DNAI supported by the I-SMF | Transparently forward the received policy information. |

The difference on these two proposals is on which entity do the filtering. In the option 1 the SMF stores all the policy information and tailor the policy per the DNAI supported by the I-SMF. In the option 2, the PCF tailors the policy and SMF forwards the received policy to I-SMF transparently. If the filtering is done by PCF, the SMF forward policy transparently.

Considering the mobility case, it can happen that UE move across different supporting I-SMF area, or between non-supporting I-SMF and supporting I-SMF. Then following issue need be considered:

- For AF traffic influence information, in the non-supporting I-SMF area, the SMF gets all information and stores it. There are no reason why when the UE move into supporting I-SMF area, the information need be tailored by PCF and handled differently.

- For Local offloading policy information, if the information is tailored by PCF per different DNAI, then each time when the UE move across different supporting I-SMF area, the SMF always need interact with PCF to get different policy information. This cause additional signalling load. However, if we check the non-supporting I-SMF case for AF traffic influence policy handling, it does not require the PCF to tailor the information per DNAI supported by the I-SMF. Instead, PCF always send all information to SMF.

Per above analysis, we do not see the reason to adopt option 2. Option 1 is more aligned with the existing policy handling.

Proposal 4: For local offloading handling, the policy handling is as following:

- The SMF includes the local offloading support indication to PCF.

- The PCF sends all the policy information related to local offloading (e.g., AF traffic influence information, Local Offloading Policy) to the SMF.

- The SMF filters the policy information related to the DNAI supported by the I-SMF to the I-SMF.

Issue 3: how to configured UL-CL/L-PSA

The related EN in the revised paper is as following:

- Editor’s note: Whether I-SMF or SMF generates N4 rules is FFS.

The above EN is related to how to configure the ULCL/L-PSA UPF controlled by the I-SMF.

Considering the intention of the KI#1 is that the I-SMF should be able to handle the DNS query and insert the UL-CL/L-PSA by itself without the interaction with the SMF, i.e. minimize the impact to the central SMF. It seems natural that the ULCL/L-PSA should also be configured by the I-SMF to avoid that each time after the handling of DNS query, the I-SMF need report the related offloading traffic to the SMF, then SMF generates the N4 information and return it back again to the I-SMF. This also includes the case the I-SMF may configure the UL-CL/L-PSA based on the EDI information directly, which may be not known by the SMF as SMF is not required to retrieve that information. In case the UE moves within the I-SMF service area and traffic need be offloaded from via one DNAI to via another DNAI, the I-SMF should also be able to manage it by itself. Hence we see the requirement that the I-SMF need be able to configure the N4 rule for UPF controlled by it.

As part of the TR conclusion, it is also agreed that EDI and Policy for local offloading handling information are sent to or retrieved by I-SMF. So from the required information to be used for N4 rule configuration at the UPF controlled by the I-SMF, the I-SMF has already got that information. If we still require the SMF to configure the N4 rule, it makes no sense to send those information to I-SMF.

Based on above analyse we see the requirement and information required are all available for I-SMF to do the N4 rule configuration for the UPF controlled by it. We do not see the reason why it is still required the SMF to generate the N4 rule.

Also in the ETSUN procedure, the DNAI of interest is sent to I-SMF per the AF traffic influence information. Now if the AF traffic influence information are already sent to I-SMF, the I-SMF can directly determine whether ULCL/L-PSA need be inserted and in which place based on that information. Hence the DNAI of interest sent by SMF is not needed.

Proposal 5: For the handling of the UPF controlled by the I-SMF:

- I-SMF generate N4 rules for the UPF controlled by it. This can be based on the result of DNS query or for preparation of DNS query.

- The SMF does not need to return the interest of DNAI to the I-SMF.

3. Conclusion and proposal(s)

It is proposed to discuss and agreed above Proposal. The related accompanied paper (S2-2410249,10250,10251,10252) are also proposed on the concrete change.