SA WG2 Meeting #165	S2-2410268

14 - 18 October, 2024, Hyderabad, India

# Title:	Discussion on UE-side AIML data collection

# Source:	CATT

# Document for:	Discussion

Agenda Item:	19.15.2

Abstract:	This paper presents a discussion on solutions of UE-side data collection for UE-side model training related to LS RP-242389 from RAN TSG.

- Background

- LS RP-242389 from RAN TSG on UE-side data collection for UE-side model training

LS RP-242389 introduces the current discussion status on UE-side data collection for UE-side model training in RAN TSG/WGs, and asks related SA WGs (SA2, SA3, SA5) to provide opinions on potential solutions. Following is a brief summary:

RAN2 has discussed UE-side data collection for UE-side model training and Table 7.2.1.3.2-1 below captures the solutions identified and analyzed in RAN2.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

RAN2 could not reach consensus without input from SA groups on feasibility of MNO full controllability and full visibility (Note 5) for option 1b and couldn’t reach consensus on the feasibility of UP solution for option 2 and option 3 (Note 7).

TSG RAN kindly asks TSG SA to initiate the work and at least for SA2 to provide inputs on solutions 1b, 2, and 3 by RAN#106.

- Use cases for UE-side data collection for UE-side model training (as discussed in RAN WGs)

UE-side model can be used for UE-based or UE-assisted positioning, CSI prediction and beam management, as described in TR 38.843. To train the UE-side models, the UE or other model training entities (e.g. OTT server) need to collect data from the UE side (and possibly also from the network side). What types of UE-side data are required for UE-side model training for the above use cases is also under discussion in RAN WGs.

- Discussion

2.1	Feasibility of option 1b in terms of MNO controllability and visibility

In option 1b, UE collected data is sent by the UE over Application layer via User Plane to a server for data collection for UE-side model training, and then sent by the server to the OTT server.

The following analysis assumes that the server for data collection for UE-side model training is a trusted AF in the MNO domain. If the server for data collection for UE-side model training is outside the MNO domain, then MNO full controllability and full/partial visibility of UE data collection cannot be achieved without SLA.

On MNO controllability of UE data collection:

- Without operator configuration or interacting with other 5GC NFs and/or the NG-RAN, the server cannot perform UE data collection with MNO full controllability, e.g. when and what data are allowed to be collected and reported by the UE.

What data are allowed to be collected and reported by the UE should be subject to:

- use cases of UE-side models, e.g. UE-based or UE-assisted positioning, CSI prediction, beam management as described in TS 38.843;

- operator policy, regulation requirements, SLA between the operator and the OTT server;

- user contents, etc.

Considering a), it requires the input from NG-RAN to decide what data the UE is allowed to collect and report, while how the server interacts with the NG-RAN to get the input is FFS.

If the server does not interact with the NG-RAN to get such input, the server can only trust the UE to comply with the data collection instructions from the NG-RAN. Or in other words, the server is not be able to reject/drop the packets of UE collected data or terminate the data transfer if the data packets contain information which is not allowed to be collected and reported by the UE.

The MNO may configure the server on UE data collection, e.g. what data are allowed to be collected and reported by the UE, based on b), but it would be hard to capture the input from the NG-RAN in a standardized and dynamic way (e.g. adjusting the configuration based on NG-RAN inputs) via operation configuration.

- Since the UE collected data is sent by the UE over Application layer, the data is transparent to the NG-RAN and UPF, so neither the NG-RAN nor the UPF have controllability of UE data collection, e.g. when and what data are collected and reported by the UE.

On MNO visibility of UE data collection:

- The server can access the Application layer payload of the data packet, i.e. UE-collected data, but the server cannot be aware of the payload content is UE collected data or comprehend the payload content, e.g. what data are being collected and reported by the UE, if there is no User Plane protocol defined for UE data collection for UE-side model training.

- Since the UE collected data is sent by the UE over Application layer, the data is transparent to the NG-RAN and UPF, so neither the NG-RAN nor the UPF has the visibility of UE data collection, e.g. what data are being collected and reported by the UE.

2.2	Feasibility of UP solution for option 2

In UP solution for option 2, UE collected data is sent by the UE first to an 5GC NF via User Plane, and then sent by the 5GC NF to a server for data collection for UE-side model training and/or to the OTT server.

On MNO controllability of UE data collection:

- If the 5GC NF (as UE data termination entity in the CN in User Plane) is the UPF, the UPF cannot initiate UE data collection. UE data collection has to be initiated by a 5GC NF with control plane functionality (e.g. the server for data collection) and then the User Plane can be established between the NG-RAN, UPF and the server for data collection under the control of the SMF. But as discussed in 2.1, the server for data collection cannot decide e.g. when and what data are allowed to be collected and reported by the UE, without operator configuration or interacting with other 5GC NFs and/or the NG-RAN.

- If the 5GC NF (as UE data termination entity in the CN in User Plane) is the LMF (only for AI/ML positioning case) or a newly defined 5GC NF which supports communication with UE via User Plane to collect UE-side data, the LMF or the newly defined 5GC NF may be able to control UE data collection, based on operator configuration or interacting with other 5GC NFs (e.g. the server for data collection) and/or the NG-RAN. However, this needs further study.

On MNO visibility of UE data collection:

If there is no User Plane protocol defined for UE data collection for UE-side model training, the MNO full/partial visibility of UE data collection cannot be achieved.

2.3	Feasibility of UP solution for option 3

In UP solution for option 3, UE collected data is sent by the UE is sent to the OAM via the gNB using User Plane (UP tunnel), and then sent by the OAM to a server for data collection for UE-side model training and/or to the OTT server.

There is no UP tunnel between gNB and OAM defined so far, nor the UP tunnel between the UE and OAM.

But anyway the feasibility is up to SA5 to discuss and decide.

2.4	Consideration on other options (i.e. CP solution for option 2 and 3)

The CP solution for option 2, though not requiring a new protocol to be defined for UE data collection as in UP solution, still requires big enhancements in NAS signalling, interactions between 5GC NFs (e.g. UE data exposure to the AF) and possible interactions between 5GC NF and NG-RAN.

The CP solution for option 3 can reuse the current MDT mechanism, ensuring MNO full controllability and full visibility of UE-collected data in gNB and OAM, and requires no enhancements in 5GC. So it seems the easiest way forward in Rel-19 for UE-side data collection for UE-side model training.

- Conclusions and proposals

Base on the discussion, conclusions and proposals on UE-side data collection for UE-side model training are made as follows:

-	For option 1b (if the server for data collection for UE-side model training is a trusted AF in the MNO domain) and UP solution for option 2, MNO full controllability and full / partial visibility of UE-side data collection for UE-side model training cannot be supported by the existing specifications or small enhancements to the existing specifications. Further study is required, taking into account also the feedback from other WGs (e.g. SA5).

-	In option 1b, if the server for data collection for UE-side model training is outside the MNO domain, MNO full controllability and full/partial visibility of UE data collection cannot be achieved without SLA.

-	The feasibility of UP solution for option 3 is up to SA5 to discuss and decide.

-	The CP solution for option 2 also needs further study considering the impacts to NAS signalling, 5GC NF services and possible impacts to the interfaces between 5GC and NG-RAN.

-	To avoid the impacts to 5GC, the CP solution for option 3 is preferred for UE-side data collection for UE-side model training, if supported and specified in Rel-19.