3GPP TSG-WG SA2 Meeting #165	S2-2410272

Hyderabad, INDIA, Oct 14 – 18, 2024

Source:	China Mobile, NEC

Title:	Conclusions for KI#1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusions for KI#1 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to capture some conclusions for KI#1.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

## 8.1	Conclusion on Key Issue #1

### 8.1.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function is introduced to support Ambient IoT.

Editor’s Note:	Whether the new core network function also applies to topology 2 is FFS.

-    The new core network function shall be responsible for reader selection, ambient IoT devices management, and services management.

-    The new core network function shall connect with a BS Reader via AMF.

-    The SBI based standard interface between AMF and the new core network function shall be introduced in 5GC.

-    the NGAP protocol between gNB and AMF shall be enhanced to support ambient IoT service.

-    The AMF shall be enhanced to support ambient IoT services.

END OF CHANGES