SA WG2 Meeting #165	S2-2410276

14 - 18 October, 2024, Hyderabad, India

# Title:	Reply LS on AIML data collection

# Response to:	RP-242389/S2-2409600 LS on AIML data collection

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	Samsung [to be SA2]

To:	RAN, RAN2, SA1, SA3, SA5

CC:	N/A

Contact person:	Hyesung Kim

hs1207.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

SA2 thanks RAN for the LS on the AIML data collection. SA2 would like to provide views on the analysis of different data collection options for UE-side model training as follows:

SA2’s view on controllability of MNO on data transfer

- The term “controllability” needs further clarification for precise analysis. For given information, SA2 cannot provide comprehensive analysis on whether full controllability can be given, or what level of controllability is possible for the data collection options.

SA2 understanding is that the meaning of controllability is not stable as the RAN#105 requirements the LS (RP-242389) indicate that “FFS controllability on data collection”. Also, for SA2 perspective analysis, the detailed scenario for “initiating, terminating, and fully managing data transfer” are required. For example, which entity performs those operations, what kinds of management are required to achieve “fully managing data transfer”. Therefore, SA2 would like to ask the following question:

To RAN2: SA2 kindly asks further clarification on “Controllability of MNO on data transfer, including (i) detailed scenario for “initiating, terminating, and fully managing data transfer” in each use cases, (ii) which entity is required to perform those operations, and (iii) what kinds of management are required to achieve “fully managing data transfer”

- SA2 provides the following observation on controllability of each option, noting that it is not comprehensive analysis and SA2 only can provide precise input once the controllability is further clarified by RAN2.

  - For Option 1a, from SA2 perspective, the core network can offer a level of controllability on data transfer. For example, the MNO can utilize the existing QoS framework and policy control for managing data transfer for AIML data collection.

  - For Option 1b, the controllability of MNO on data transfer is very limited because the data goes through the UP tunnel toward the server. For such a UP tunnel path, the potential involvement of NF is restricted, especially for the server located outside of MNO domain.

  - For Option 2, the MNO can control various aspects on data transfer via CP tunnel as the data goes through the 5GC NF. The NF can be designed to control the initiation and termination of data transfer over CP from UE to the NF and decide when and whether to transfer the data, received from the UE, toward the server.

  - For Option 3, the MNO can control various aspects on data transfer via CP tunnel as the data goes through the OAM. The OAM can be designed to decide when and whether to transfer the data toward the server. However, Table 7.2.1.3.2-1 only indicates RRC procedure as solution for network controllability, which cannot cover comprehensive solution. If MNO wants to have controllability via OAM system, then its related interaction between OAM and gNB is required, which requires SA5 input.

Based on the above observation on the controllability, SA2 would like to suggest the following updates in Table 7.2.1.3.2-1 from SA2 perspective.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability Full controllability is not possible, FFS: level of controllability | Full controllability is not possible, FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) Example: per PDU sessions | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure or FFS additional procedure between OAM and gNB |

SA2’s view on possible options for visibility of data content in MNO and Data format

- The term “visibility” needs further clarification for precise analysis. Especially, for requirement for visibility, it is not clear which entity should have the visibility and what the partial visibility means in comparison with the full visibility. The definition of visibility and what level of visibility is required should be discussed first. However, it is out of SA2 scope as it belongs to the service requirements for the 5GS, which should be discussed in SA1. Accordingly, SA2 thinks that the definition of visibility and its related requirement should be discussed in SA1 and that for SA1’s convenience RAN2 needs to share further clarification on the visibility with SA1.

To SA1: SA2 asks the following questions on options for visibility of AIML data described in RP-242389

Q1) Whether is any requirement for visibility of AIML data collection specified? If yes, please provide details and how the possible options (Full visibility, Partial visibility, and No visibility) can be associated with the existing requirement

Q2) If no to Q1, what is SA1’s view on the definition of visibility for AIML data collection and on what level of visibility is required?

To RAN2: SA2 asks RAN2 for further clarification on the visibility including:

i) Which entity should have visibility

ii) Whether the visibility is considered as fully guaranteed from RAN2 perspective only if the data content is standardized, although it may not be decoded by the entities in the MNO domain

iii) Whether “Partial visibility for partially standardized data content” means, e.g., whether the data content is partially standardized, or whether the standardized data content can be partially decoded in the MNO domain, etc.

- For Option 2 and Option 3, the standardized data content may not be visible for the MNO. Although the first termination entity is inside the 5G CN or OAM domain, NF or OAM may not decode or decrypt the data. For example, the container for AIML data transfer from UE to the server can be defined so that the NF or OAM entity just deliver the container without being able to decode the data content in the container.

SA2’s view on feasibility of UP solution for option 2 and option 3

- For Option 2, SA2 cannot confirm if the UP tunnel is feasible for data collection with given information. From SA2 perspective, it is not clear and straightforward which NF needs to be the first termination entity for UP tunnel, and how the NF can initiate and terminate the data transmission over UP from UE, and requires substantial studies. What SA2 can confirm is that the controllability and visibility of Option 2 using UP tunnel might be lower than the one using CP tunnel.

- For Option 3, SA2 cannot confirm if the UP tunnel is feasible for data collection with given information. It requires SA5 inputs on UP tunnel establishment, its usage on AIML data collection, and potential impacts on 5GS to enable such things.

SA2’s view on secured data collection, data integrity, and confidentiality

- SA2 requires SA3’s views on whether the options can satisfy the requirement “The data collected is secured and data integrity and confidentiality for that data is ensured”. Especially, SA2 identifies potential security issues on how to address the given requirement for the data transfer path between CN/OAM and the OTT server outside the MNO domain.

SA2’s view on user data privacy, anonymity and user consent is respected

- SA2 requires SA3’s views on how user data privacy, anonymity and user consent can be respected for each data collection option.

SA2’s view on futureproof and extendibility

- For Option 1a, this option can be futureproof because it does not require additional standardization impact.

- For Option 1b, this option is less futureproof and extendible than Option 1b may require potential changes on the involved NF.

- For Option 2 and Option 3, both NAS and RRC based approaches are neither futureproof or extendable. NAS and RRC signaling would become congested due to increasing AIML data collection volume. The use of UP tunnel for Option 2 and Option 3 may offer better futureproof and extendibility, comparing with a CP-based solution, but it still requires new additional functionalit(ies).

2	Actions

To RAN, RAN2, SA1, SA3, and SA5

ACTION: 	SA2 kindly asks RAN and RAN2 to take the above information into account.

SA2 kindly asks RAN2 for further clarification on “Controllability of MNO on data transfer”, including (i) detailed scenario for “initiating, terminating, and fully managing data transfer” in each use cases, (ii) which entity is required to perform those operations, and (iii) what kinds of management are required to achieve “fully managing data transfer”.

SA2 also asks RAN2 for clarification on “visibility”: (i) which entity should have visibility, (ii) whether the visibility is considered as fully guaranteed from RAN2 perspective only if the data content is standardized, although it may not be decoded by the entities in the MNO domain, and (iii) whether “Partial visibility for partially standardized data content” means, e.g., whether the data content is partially standardized, or whether the standardized data content can be partially decoded in the MNO domain, etc.

SA2 kindly asks SA1 to provide answers to the above Q1 and Q2.

SA2 kindly asks SA3 to provide views on whether the data collection options are feasible to satisfy the requirement described in RP-242389/S2-241048, and any potential security issue that may require stage 2 level architectural impact.

SA2 kindly asks SA5 to provide views on Option 3 in RP-242389/S2-241048 regarding feasibility of UP tunnel establishment toward OAM entity, its usage on AIML data collection, and any potential impacts on SA2 stage 2 level architecture and procedures.

3	Dates of next SA WG2 meetings

SA2#166:	18 - 22 November, 2024	Orlando, USA

SA2#166AH-e:	20 - 24 January, 2025	Electronic meeting

SA2#167:	17 - 21 February, 2025	Athens, Greece