3GPP TSG-WG SA2 Meeting #165	S2-2410281

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5750 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of N3GPP device behind UE/5G-RG as concluded in TS 23.700-66 KI#4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the support of Non-3GPP device connecting behind a UE as concluded in the TR 23.700-32 KI#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support Non-3GPP device connecting behind the UE the following changes are needed: - Introduction of new definition (in clause 3.1) of Non-3GP device behind UE/5G-RG. - Session management - QoS differentiation - Restriction of the number of simultaneously activate Non-3GPP device identifiers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.x	Non-3GPP device connecting behind the UE

### 5.x.1	General

This clause specifies the scenario of a Non-3GPP device connecting through the UE that does not use NAS and that it is not authenticated by 5GC, but the 5GS is able to recognize and manage traffic that originates from or is directed to the non-3GPP device, ensuring proper traffic handling within the 5G network, even if the device does not connect directly to the 5G core. This clause does not apply to the NAUN3 device defined in TS 23.316.

The support of identification of traffic for Non-3GPP devices connecting behind a 5G-RG is specified in TS 23.316 [84]. The identification of traffic from Non-3GPP device connecting behind a FN-RG is not supported.

### 5.x.2 Identification of a Non-3GPP Device connected behind a UE.

To enable identification of traffic from a Non-3GPP device, the UE stores the mapping between Non-3GPP Device Identifiers and device related information, e.g., QoS policies, MAC address, or device types (XR glasses, laptops, etc). When a Non-3GPP device is connecting to the UE, the UE identifies the device related information via a procedure outside the 3GPP scope (for example based on locally configured policies, via the application layer, etc.)  and binds a suitable Non-3GPP Device Identifier to the non-3GPP device based on the stored mapping. This association enables the 5G System to distinguish between the traffic generated by non-3GPP devices connected through the same UE. How the UE binds the traffic from a non-3GPP device connecting behind the UE to the associated Non-3GPP device identifier is implementation specific.

The Non-3GPP Device Identifier is a permanent identifier in a generic string format.

The Non-3GPP Device Identifier is unique per UE. The UE may bind the Non-3GPP Device Identifier to different Non-3GPP Device connected behind the UE or always to the same Non-3GPP device based on UE implementation specific policy, but at any point in time the identifier can be bind to only one single Non-3GPP device.

NOTE 1: 	This is different from the Connecticity Group Identifierd used for the NAUN3 device which can be associated to all Non-3GPP devices which are connected via a port, e.g. via a WLAN SSID.

NOTE 2:	Considering the variety of possible Non-3GPP devices e.g. laptop, tablet, UE using WLAN, surveillance camera, Smart TV, etc, etc the user decision and the application used to manage all these varieties of devices can play an essential role in binding decisions, for example a surveillance camera the binding may be “permananet” in order to always use the same identifier for the same camera, while binding for an laptop may depend, e.g. to generic laptops the same identifier can be used at different times since it is not important which device is use for which laptop, but alternativly for a desktop acting as a server it may be of interest to use always the same identifiers.

How and when the Non-3GPP Device Identifier is allocated to the UE is out of scope of 3GPP.

After the allocation, the Non-3GPP Device Identifier is provisioned to UDR in the UE subscription information of the UE at which can be connected to with the procedure described in clause X.Y. The Non-3GPP Device Identifier does not change after being assigned to a UE until it is removed from the UDR.

### 5.x.3 Provisioning of Non-3GPP Device Identifier.

Editor’s note : FFS

### 5.x.2	Session management enhancement

For Ethernet PDU Session Type, the UE send the Non-3GPP Device Identifier and may also send MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier to the SMF in PDU session modification procedure.

For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and it may send the UE IP Address and/ Port ranges associated with the Non-3GPP Device Identifier  to the SMF in PDU Session Modification procedure.

For IPv6 PDU Session Type, the UE sends the the Non-3GPP Device Identifier and it may send the IPv6 Address that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

### 5.x.3	QoS differentiation

QoS differentiation and policy control is defined in TS 23.503 clause 6.1.3.xx

# * * * * End of changes * * * *