SA WG2 Meeting #165	S2-2410286

14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1160 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Training procedure for Vertical Federated Learning when AF is as Server. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.2 of TR 23.700-84, the proposal specifies the training procedure of Vertical Federated Learning when AF is as Server in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the general training procedure of VFL when AF is as Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL functionality when AF is VFL server is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2G.2.X General training procedure for Vertical Federated Learning when AF is VFL Server.

Figure 6.2G.2.X-1. General procedure for VFL model training when AF is VFL Server

- The AF acting as VFL Server discovered the respective NWDAFs as VFL Clients as clause 6.2H and completed the VFL Training Preparation procedure.

- To start VFL training, VFL Server AF sends requests to the VFL Client NWDAF(s). The request includes an initial ML model that will be involved in the VFL training, an Analytics ID, VFL model correlation ID, and sample information that identifies the data sample to be used in the VFL process (e.g., UE ID). 
If the AF is untrusted, the requests go to the NEF, the NEF forwards the requests to the NWDAFs based on the AF’s instruction. Before the NEF forwards, the NEF translates the internal and external IDs (e.g., GPSI and SUPI).

2. [Optional] Each VFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3. The VFL Client NWDAF computes the intermediate results for its local ML models involved in the VFL training and reports the intermediate results to the VFL Server AF. The report includes sample information.
If untrusted AF is VFL Server, the report goes to the NEF, and the NEF forward to the VFL Server AF. Before NEF forwards, NEF translate the internal and external IDs.

4. The VFL Server AF computes the backward local ML model training information (e.g. gradient information or loss information) based on the intermediate training result received in step 3 and label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

5.  The VFL Server AF determines another round of VFL training is required. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process. The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process.

6.	If the VFL procedure continues, VFL server sends the backward local ML model training information (e.g. gradient information, loss information) to the VFL clients for next round of VFL training.

7.	Each VFL client computes gradient of its local model and updates its local ML model based on backward local ML model training information distributed by the VFL server at step 6. The VFL Server AF and each VFL Client NWDAF, stores the latest information about their locally trained Models.

The steps 2-7 should be repeated until the training termination condition is reached.

# >>>>END OF CHANGES<<<<