3GPP TSG-WG SA2 Meeting #165	S2-2410315

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-2408074, 8851)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5518 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-satellite-UE communications when serving satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, [Thales, CATT, CSCN, Honor, Tencent, NTT DOCOMO] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When UE’s serving satellite changes, service continuity should be maintained considering the user’s QoE. The below principles are taken as the baseline for introducing the feature of mobility support: Principle#1: we assume UE’s serving satellite changes mean the serving satellite with at least gNB onboard. Other cases like serving satellite changes with only UL CL/L-PSA onboard, or with only AGW onboard are not in the scope of this scenario. In scope of 5GC, there are 2 options considering when to change UL CL/L-PSA - Option#1: UL CL/L-PSA changes simultaneously with handover, - Option#2: UL CL/L-PSA changes only after SMF receives a decision from IMS. In case of Xn-HO, as there is connectivity between source and target, i.e. any N3, N6 connectivity is ensured, both option#1 and option#2 ensure connectivity. In case of N2-HO, as there is no connectivity between source and target, i.e. no N3 between target gNB and source UL CL/L-PSA, no N6 connectivity between target UL CL/L-PSA and source AGW, option#1 results in a period of call interruption, as there is no N6. If it is Xn-HO, we have 2 methods: Method#1: - gNB changes, UL CL/L-PSA simultaneously change - P-CSCF decision on IMS satellite media plane - SMF coordinates If to continue: update UL CL rule If not to continue: ground PSA Method#2: - gNB changes - P-CSCF decision on IMS satellite media plane - SMF coordinates If to continue: change UL CL and update UL CL rule If not to continue: ground PSA (rare case) Principle#2: for Xn-HO, method#2 is better considering UL CL rule update can be together in change UL CL procedure. If it is N2-HO, - gNB changes and fall back to PSA - P-CSCF decision (only no) on IMS satellite media plane - SMF coordinates If not to continue: do nothing Principle#3: for N2-HO, it can be viewed as a ground fallback trigger decision that was triggered by 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the feature of when UE’s gNB onboarded serving satellite changes, basically: - For Xn-HO, we document Method#2 - For N2-HO, we assume there is no need to be informed with a notification from IMS. SMF needs to know the connectivity between gNB onboarded target satellite and UL CL/L-PSA onboarded source satellite, if there is, it means it is Xn, it there is no, it means it is N2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Voice/video call drop rate significantly increased when the serving satellite changes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.x.2.2	Mobility support for UE-satellite-UE communication when serving satellite changes

Editor’s Note:	the following mechanism assumes gNB, UL CL/L-PSA, IMS-AGW onboarding the same satellite, how to adjust it to apply to other deployments is FFS.

In the UE-SAT-UE communication, if one of the UE’s serving gNB-onboarded satellite changes (e.g. due to UE’s movement, or the satellite’s movement), the following mechanisms should be used to ensure service continuity:

-	The Xn/N2-based handover procedure will be performed to change the serving gNB from the source satellite to the target satellite. The AMF will include the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request. The SMF will decide whether to fall back the PDU session to the ground based on the connectivity between the gNB-onboarded target satellite and the UL CL/L-PSA-onboarded source satellite.

NOTE: 		How the SMF retrieves the connectivity status between source and target satellites is an implementation issue.

-	If the gNB-onboarded source satellite and the UL CL/L-PSA-onboarded source satellite have connectivity (i.e. Xn handover), the SMF will decide to retain the existing UL CL/L-PSA, reusing the Xn-based handover procedure without relocating the user plane as described in clause 4.9.1.2.2 in TS23.502 [3];

-	If the gNB-onboarded source satellite and the UL CL/L-PSA-onboarded source satellite do not have connectivity (i.e. N2 handover), the SMF will decide to revert the PDU session to the PSA on the ground, reusing the procedure for removing the UL CL/L-PSA as described in clause 4.3.5.5 in TS 23.502 [3].

-	Once the handover is completed, the SMF will generate a user plane fallback indication that shows the 5GC user plane has fallen back to the ground, and report the target satellite ID and this indication (if applicable) to the PCF via Npcf_SMPolicyControl, and the PCF will then notify them to the P-CSCF via Npcf_PolicyAuthorization_Notify. Based on the decision outlined in clause AY.6.2 of TS 23.228 [15], the PCF will receive a U-S-U indication regarding whether the IMS satellite media plane optimization should continue or not, and an IMS-derived transport address and forward them to the SMF:

- If the U-S-U indicates to continue IMS satellite media plane optimization, the SMF will change the UL CL/L-PSA to the target satellite by performing a simultaneous change of UL CL and additional PSA for the PDU Session, as described in clause 4.3.5.7 of TS 23.502 [3], where the UL CL rule will also be updated with the IMS-derived transport address.

- If the U-S-U indication is to terminate IMS satellite media plane optimization, the SMF will remove the UL CL/L-PSA on the source satellite by following the procedure described in clause 4.3.5.5 of TS 23.502 [3].

# * * * * End of change * * * *