3GPP TSG-WG SA2 Meeting #165	S2-2410321

Oct. 14 - 18, 2024; Hyderabad, India

Source:	China Mobile, CATT, China Telecom

Title:	Discussion on Applying Integrated MPQUIC Steering Functionality to Proxy UDP, IP and Ethernet

Document for:	Discussion

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract: this document discusses & analyses how to adopt an integrated MPQUIC steering functionality to proxy UDP, IP and Ethernet.

# Discussion

TR 23.700-54 KI#2.1 has so far concluded and defined two new MPQUIC steering functionalities, i.e., Multipath QUIC-IP (MPQUIC-IP) and Multipath QUIC-Ethernet (MPQUIC-E), in addition to the Multipath QUIC-UDP (MPQUIC-UDP) that has been standardized in Rel-18.

But, we argue in the discussion paper that the workflow, protocol stacks & encapsulations of both MPQUIC-IP and MPQUIC-E are closely sharing the similar fundamentals as those of MPQUIC-UDP. Therefore, we propose to categorize all the three into the same integrated MPQUIC steering functionality.

## Workflow of MPQUIC Steering Functionality

The Rel-18 MPQUIC steering functionality (so named as MPQUIC-UDP in the document) is standardized by applying the IETF MASQUE technology. It extends the HTTP fundamental method ‘CONNECT’ [RFC-9110] thru the protocol extension ‘connect-UDP’ [RFC-9298]. The UDP proxying payload is encapsulated via the HTTP/3 Datagram payload [RFC-9297]. While the multipath QUIC technology [IETF-multipath-quic] is required to achieve the multi-path traffic transport, the QUIC extension [RFC-9221] is adopted for datagram-based traffic transport.

The Rel-19 FS_MASSS has concluded to apply MPQUIC to proxy both IP packets and Ethernet frames. The HTTP upgrade tokens for IP ‘connect-ip’ and for Ethernet ‘connect-ethernet’ are sharing the same fundamental scheme as that for UDP ‘connect-udp’.  When integrated together, the MASQUE-based scheme for proxying UDP, IP, and Ethernet traffic presents a holistic workflow as shown below:

The above figure demonstrates that, while MPQUIC-UDP, MPQUIC-IP and MPQUIC-E might have the different 1st-level (or called internal) proxying payload (i.e., UDP, IP and Ethernet), all of them are sharing the same integrated workflow thereafter.

## Encapsulation & Protocol Stacks of Proxying UDP, IP & Ethernet

This subsection shows the protocol stacks and encapsulation upon proxying UDP, IP and Ethernet.

- MPQUIC for Proxying UDP: -- the existing work in Rel-18

As per the [RFC-9298], i.e., proxy UDP in HTTP, the format of UDP-proxying HTTP Datagram payload is defined as:

UDP Proxying HTTP Datagram Payload {

Context ID (i),

UDP Proxying Payload (..),

}

Here, the ‘UDP proxying payload’ is the original UDP traffic that is supposed to be transported via MPQUIC in later stages. The format indicates the first-level encapsulation structure of MPQUIC-UDP.

- MPQUIC for Proxying IP: -- for the support in Rel-19

As per the [RFC-9484], i.e., proxy IP in HTTP, the format of IP-proxying HTTP Datagram payload is defined as:

IP Proxying HTTP Datagram Payload {

Context ID (i),

Payload (..),

}

Here, the ‘payload’ is the original IP traffic that is supposed to be transported via MPQUIC in later stages. The format indicates the first-level encapsulation structure of MPQUIC-IP.

- MPQUIC for Proxying Ethernet: -- for the support in Rel-19

As per the [draft-ietf-masque-connect-ethernet], i.e., proxy Ethernet in HTTP, the format of Ethernet-proxying HTTP Datagram payload is defined as:

Ethernet Proxying HTTP Datagram Payload {

Context ID (i),

Payload (..),

}

Here, the ‘payload’ is the original Ethernet frame that is supposed to be transported via MPQUIC in later stages. The format indicates the first-level encapsulation structure of MPQUIC-E.

When we combine the previous three 1st-level encapsulations with successive workflow stages, the protocol stacks of the three MPQUIC-based Proxying are:

Obviously, the encapsulations and protocol stacks of all the three MPQUIC-based Proxying, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, exhibit fundamentally the same characteristics.

## Steering Functionality: High-layer vs. Low-layer

The clause 5.32.6.1 in the TS 23.501 has defined two kinds of steering functionalities, i.e., the high-layer which operates above the IP layer vs. the low-layer which operates below the IP layer.

It is fairly straightforward to understand the MPQUIC-UDP is in the kind of high-layer steering, with the proxying UDP payload. Actually, both the MPQUIC-IP and MPQUIC-E are also in the kind of high-layer steering. The section #1.2 has clearly illustrated the encapsulation & protocol stacks of proxying IP and Ethernet. With the (complex) protocol encapsulations in place, it is obviously MPQUIC-IP and MPQUIC-E also operate above the IP layer, and thus belong to the high-layer steering.

In conclusion, all the three MPQUIC-based proxying schemes, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, belong to the high-layer steering category.

# Proposal

The section#1 elucidates that the traffic workflow, protocol stacks & encapsulations of both MPQUIC-IP and MPQUIC-E are closely sharing the similar fundamentals as those of MPQUIC-UDP. Therefore, we propose:

Proposal 1: All the three different proxying schemes for MPQUIC steering, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, shall be categorized as one integrated MPQUIC steering functionality.

For the integrated MPQUIC steering functionality, we propose to use the ‘proxied payload’ to distinguish the different schemes:

Proposal 2: The MPQUIC steering functionality defined in Rel-18 TS 23.501 is extended to accommodate new proxying payload, i.e., IP and Ethernet.

-	MPQUIC steering functionality for proxying IP (i.e., so-named as Multipath QUIC-IP or MPQUIC-IP) is based on CONNECT-IP [RFC-9484] to tunnel IP packets between a client (UE) and a proxy (UPF) using HTTP/3 over MPQUIC;

-	MPQUIC steering functionality for proxying Ethernet (i.e., so-named as Multipath QUIC-Ethernet or MPQUIC-E) is based on CONNECT-Ethernet [draft-ietf-masque-connect-ethernet] to proxy Ethernet frames between a client (UE) and a proxy (UPF) using the HTTP/3 over MPQUIC.

NOTE: The MPQUIC steering functionality defined in Rel-18 TS 23.501, for proxying UDP, is referred as Multipath QUIC-UDP or MPQUIC-UDP, based on CONNECT-UDP [RFC-9298] to tunnel UDP packets between a client (UE) and a proxy (UPF) using HTTP/3 over MPQUIC.