SA WG2 Meeting #165	S2-2410345

Hyderabad, India, October 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	Interim Conclusion on Converged Deployment of Topology 1 and Topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: The paper is proposed to give interim conclusion on converged deployment of Topology 1 and Topology 2.

1. Discussion

Several solutions in TR 23.700-13 mention the scenario of converged deployment of Topology 1 and Topology 2; this converged deployment benefits the extending coverage of Readers serving the AIoT devices.

The paper is proposed to give interim conclusion on converged deployment of Topology 1 and Topology 2.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

8.2.X	Converged Deployment of Topology 1 and Topology 2

The following aspects and principles are considered and agreed for the interim conclusion on converged deployment of Topology 1 and Topology 2:

The illustration of converged deployment of Topology 1 and Topology 2 is shown in Figure 8.2.X-1..

Figure 8.2.X-1: Illustration of converged deployment of Topology 1 and Topology 2

For the scenario of converged deployment of two topologies, a new AIoTMF function is responsible for handling AIoT services for both RAN Reader and UE Reader.

AIoTMF function receives the AIoT service request for Devices from AF (through NEF) and distribute it to RAN Readers and UE Readers.

AIoTMF function aggregates the service operation results from RAN Readers and UE Readers and sends back to AF.

AF may not be aware of which kind of Readers (RAN or UE) serving the devices.

The binding information of Device and Reader can be used to locate the serving Reader.

* * * * End of Changes * * * *