3GPP SA WG2 Meeting #165	 S2-2410356

# Haderabad, India, 14-18 October 2024	        (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1490 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6: Support of Standalone IMS Data Channel feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The wording of the description of IMS session with standalone data channel is not aligned the text. This CR is mainly to correct the term used and make the text accurate and concise. Additional, the current text in AC.X.2.2 assume the terminating UE will only alert the user after the orginiating UE downloaded the application. This is not possible and not necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changing the term from “IMS standalone data channel session” to “IMS session with standalone data channel”. - Clear up the text in multiple clause. - Change the terminating alert timing in AC.X.2.2 to when the terminating UE receives the SDP offer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and inconsistent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, AC.X.1, AC.X.2.1, AC.X.2.2, AC.X.2.3, AC.X.2.4, AC.X.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change* * *

## 3.1	Definitions

Refer to TS 23.002 [1] for the definitions of some terms used in this document.

For the purposes of the present document the terms and definitions given in TR 21.905 [68] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [68].

For the purposes of the present document, the following terms and definitions given in TS 23.003 [24] apply:

-	Distinct Public Service Identity

-	Public User Identity

-	Wildcarded Public User Identity

-	Wildcarded Service User Identity

Alias Public User Identities: A set of Public User Identities that belong to the same alias group as specified in TS 29.228 [30].

ALG: Application Level Gateway (ALG) is an application specific functional entity that allows communication between disparate address realm or IP versions, e.g. an IPv6 node to communicate with an IPv4 node and vice versa, when certain applications carry network addresses in the payloads like SIP/SDP. NA(P)T-PT or NA(P)T is application unaware whereas ALGs are application specific translation entities that allow a host running an application to communicate transparently with another host running the same application but in a different IP version or IP address realm. See IETF RFC 2663 [34] for more details.

For IMS, an IMS ALG provides the necessary application function for SIP/SDP protocols in order to communicate between different address realms or IP versions, e.g. IPv6 and IPv4 SIP applications.

Application data channel: A data channel within an IMS session used to transfer data of data channel applications between UEs or between the UE and the network.

AR Application Server: An application server used to control the service logic related to AR communication via IMS data channel.

Bootstrap data channel: A data channel established within an IMS session between the UE and the network, to transfer a graphical user interface that can include a list of data channel applications.

Business trunking: as defined in TS 24.525 [81].

Data channel application: A HTML page including JavaScript(s) and optionally image(s) and style sheet(s). It is downloaded from the network to the UE through the bootstrap data channel.

DC Application Server: An application server that interacts with the DCSF and the DC media function for data channel traffic handling.

Distinct Public User Identity: used in relation to wildcarded Public User/Service Identities to denote an explicitly provisioned Public User/Service Identity. See more details in TS 23.003 [24].

Entry point: In the case that border control concepts are to be applied in an IM CN subsystem, then these are to be provided by capabilities within the IBCF, and the IBCF acts as an entry point for this network (instead of the I-CSCF). In this case the IBCF and the I-CSCF can be co-located as a single physical node. If border control concepts are not applied, then the I-CSCF is considered as an entry point of a network. If the P-CSCF is in the home network, then the I-CSCF is considered as an entry point for this document.

Exit point: If operator preference requires the application of border control concepts then these are to be provided by capabilities within the IBCF, and requests sent towards another network are routed via a local network exit point (IBCF), which will then forward the request to the other network (discovering the entry point if necessary).

Geographical Identifier: A Geographical Identifier identifies a geographical area within a country or territory. See more details in clause E.8.

Geo-local service number: A local service number that is used to access a service in the roamed network (a local service where the subscriber is located).

Home local service number: A local service number is used to access a service that is located in the home network of the user.

HSS Group ID: This refers to one or more SBI capable HSS instances managing a specific set of IMPIs/IMPUs.

IMC: IMS Credentials as defined in TR 21.905 [68].

IMS application reference: An IMS application reference is the means by which an IMS communication service identifies an IMS application.

IMS application: An IMS application is an application that uses an IMS communication service(s) in order to provide a specific service to the end-user. An IMS application utilises the IMS communication service(s) as they are specified without extending the definition of the IMS communication service(s).

IMS communication service identifier: An IMS communication service identifier uniquely identifies the IMS communication service associated with the particular IMS request.

IMS communication service: An IMS communication service is a type of communication defined by a service definition that specifies the rules and procedures and allowed medias for a specific type of communication and that utilises the IMS enablers.

IMS enabler: An IMS enabler is a set of IMS procedures that fulfils specific function. An IMS enabler may be used in conjunction with other IMS enablers in order to provide an IMS communication service.

Instance identifier: An identifier, that uniquely identifies a specific UE amongst all other UEs registered with the same Public User Identity.

Inter-IMS Network to Network Interface: The interface which is used to interconnect two IM CN subsystem networks. This interface is not constrained to a single protocol.

IP Flow: Unidirectional flow of IP packets with the following properties:

-	same destination IP address and port number;

-	same source IP address and port number;

-	same transport protocol (port numbers are only applicable if used by the transport protocol).

IP-Connectivity Access Network: refers to the collection of network entities and interfaces that provides the underlying IP transport connectivity between the UE and the IMS entities. An example of an "IP-Connectivity Access Network" is GPRS.

IP SM GW (IP short message gateway): An IP SM GW is an AS providing the support of Short Message Service of the IMS domain. See more details in TS 23.204 [56].

Local Service Number: A local service number is a telephone number in non-international format. A local service number is used to access a service that may be located in the home network of the user (home local service number) or the roamed network of the user (geo-local service number).

Media Flow: One or more IP flows carrying a single media instance, e.g. an audio stream or a video stream. In the context of this specification the term Media Flow is used instead of IP Flow regardless of whether the actual IP packet corresponds to media plane information (e.g. audio RTP flow) or control signalling (e.g. RTCP or SIP Signalling).

MPS: Based on TS 22.153 [77]. Multimedia Priority Service allows authorized users to obtain and maintain radio and network resources with priority, also during national security or emergency situations when PLMN congestion may occur.

MPS session: A session (e.g. voice, video, data session) for which priority treatment is applied for allocating and maintaining radio and network resources.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NAT-PT/NAPT-PT: NAT-PT uses a pool of globally unique IPv4 addresses for assignment to IPv6 nodes on a dynamic basis as sessions are initiated across the IP version boundaries. NAT-PT binds addresses in IPv6 network with addresses in IPv4 network and vice versa to provide transparent routing between the two IP domains without requiring any changes to end points, like the UE. NAT-PT needs to track the sessions it supports and mandates that inbound and outbound data for a specific session traverse the same NAT-PT router.

NAPT-PT provides additional translation of transport identifier (e.g. TCP and UDP port numbers, ICMP query identifiers). This allows the transport identifiers of a number of IPv6 hosts to be multiplexed into the transport identifiers of a single assigned IPv4 address. See IETF RFC 2766 [33] for more details.

Network Address Translation (NA(P)T): method by which IP addresses are mapped from one group to another, transparently to end users. Network Address Port Translation, or NA(P)T is a method by which many network addresses and their TCP/UDP (Transmission Control Protocol/User Datagram Protocol) ports are translated into a single network address and its TCP/UDP ports. See RFC 3022 [65] for further details.

Outbound: Managing Client Initiated Connections in the Session Initiation Protocol (Outbound) defines behaviours for User Agents, registrars and proxy servers that allow requests to be delivered on existing connections established by the User Agent. See RFC 5626 [48] for further details.

Preferred Circuit Carrier Access: An IMS service that allows a specific long distance circuit carrier to be selected for a long distance call.

Preferred Circuit Carrier Selection: An IMS service that allows the subscriber to select a long distance circuit carrier per call when dialling a call origination.

Service User: According to TS 22.153 [77].

Stand-alone Non-Public Network: A non-public network not relying on network functions provided by a PLMN.

STUN: Simple Traversal of UDP Through NAT (STUN), provides a toolkit of functions. These functions allow entities behind a NAT to learn the address bindings allocated by the NAT, to keep those bindings open, and communicate with other STUN-aware devices to validate connectivity. See RFC 5389 [47] for further details.

STUN Keep-alive: Is a usage of STUN, to keep NAT bindings open.

STUN Relay: Is a usage of STUN, that allows a client to request an address on the STUN server itself, so that the STUN server acts as a relay. See IETF RFC 5766 [46] for further details.

Subscriber: A Subscriber is an entity (comprising one or more users) that is engaged in a Subscription with a service provider. The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of user authorized to enjoy these services, and also to set the limits relative to the use that users make of these services.

Transport address: A unique identifier of transport-layer address, i.e. a combination of a network address, protocol identifier and port number. For example an IP address and a UDP port.

IMS session with standalone data channel: An IMS session with only data channel media component(s) defined in TS 26.114 [76] without accompanying audio/video/messaging media.

* * * Second Change* * *

# AC.X	IMS Session with Standalone Data Channel.

## AC.X.1	General

IMS session could be established with standalone data channel. There are two types of IMS session with standalone data channel specified:

- Originating standalone bootstrap data channel setup using PSI.

- IMS session with standalone data channel with peer UE.

An originating standalone bootstrap data channel setup using PSI is used for downloading application list and applications from DCSF. A session setup using PSI cannot add audio/video media.

An IMS session with standalone data channel with peer UE can be established by using SIP Invite message, or by removing the MMTel media from an existing IMS session using SIP re-Invite message.

NOTE 1:	The behavior of remote UE responding to standalone IMS data channel session is not mandated and is up to its own implementation, i.e., it can reject the session and does not download the application.

When IMS session with standalone data channel is allowed, based on operator policy, additional subscription information for standalone data channel service may be included in service subscription for IMS Data Channel to be used by the IMS AS to determine whether to accept an IMS session with standalone data channel.

When subscription to IMS session with standalone data channel is needed, and if a UE that has not subscribed to IMS session with standalone data channel establishes a such session, the IMS AS shall reject the session establishment.

IMS session with standalone data channel use SIP session timer as per RFC 4028 [x] to avoid hanging resources in the UE and the network. Usage of SIP session timer in IMS is specified in TS 24.229 [10a].

The IMS session with standalone data channel is terminated by the existing session release procedure specified in AC.7.6. The session release may be triggered by the expiry of the session timer or by sending a SIP BYE request.

## AC.X.2	Procedures

### AC.X.2.1	Originating Standalone Bootstrap DC Setup using PSI

The Originating standalone bootstrap data channel setup using PSI is triggered by a UE or the user of a UE for downloading application list and/or applications. After standalone bootstrap DC is established, the UE downloads the application list via the bootstrap DC, and may display the list of application information to the user for selection, downloading, and/or initiating standalone application DC by establishing a separate IMS session. The UE terminates the IMS session with standalone data channel when the session timer expired.

Figure AC.X.2.1-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel using PSI.

AC.X.2.1-1: Originating Standalone Boostrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific target URI, such as a specific PSI indicating standalone bootstrap DC. The initial SDP contains offers only for the bootstrap data channel establishment request with bootstrap DC stream ID.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.1 apply.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP, determines that UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which are used to receive UE request for application downloading from MF or MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF for originating side and to terminate the session. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	IMS AS terminates the IMS session based on the specific Request URI and sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The session with bootstrap data channel is established.

14.	The bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 sends application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), UE#1 sends SIP BYE to release the bootstrap DC to save power.

### AC.X.2.2	Adding Application DC to IMS Session with Standalone Bootstrap Data Channel

The IMS session with standalone data channel setup between two UEs is triggered by the user of an originating UE. In case the user selected DC application has not been downloaded yet, the originating UE initiates standalone bootstrap DC setup during the IMS session setup procedure. The originating UE downloadeds the DC application selected by the user via the bootstrap DC, updates the IMS session to add application DC with the terminating UE and indicates the selected DC application to the terminating UE. If the indicated DC application is not downloaded yet, the terminating UE downloads the indicated DC application via the bootstrap DC.

The procedure specified in clause AC.7.1 applies with the following dirrefences:

-	In step 1, UE#1 sends the initial INVITE request with an SDP offer only including media component for bootstrap data channel for originating side and optional for terminating side;

-	In step 2, the IMS AS determines whether the IMS session is an IMS session with standalone data channel, and determines if the session is allowed to be established based on local configuration or subscription data. If IMS session with standalone data channel is not allowed, the IMS AS shall reject the request;

-	In step 11, the media component for bootstrap data channel for terminating side may be included in an SDP early offer to enable the terminating side downloading the DC Application selected by the user of UE#1 before ringing;

-	In step 14, SIP precondition is not needed, and after the boostrap DC is established, the UE#1 may send SIP UPDATE message to add an application DC selected by the user of UE#1;

-	Steps 23-24 may be skipped if the SDP offer only including media component for bootstrap data channel for originating side;

-	In step 25, the UE#2 alerts the user of UE#2 after receiving the SDP offer from step 1.

### AC.X.2.3	Combined Standalone Bootstrap DC and Application DC Setup

The IMS session with standalone data channel setup between two UEs is triggered by the user of an originating UE. In case the the user selected DC application has been downloaded, the originating UE initiates combined standalone bootstrap DC and application DC setup during the IMS session setup procedure. The application DC is established between the originating UE and terminating UE. If the selected DC application is not downloaded yet, the terminating UE downloads the selected DC application via the bootstrap DC.

In this procedure, the assumption is that the terminating UE alerts the user and may display the application information to the user.

When the user of the terminating UE accepts to use the application for the communication, the IMS session is answered by the terminating UE.

The procedure specified in clause AC.7.2.1 applies with the following dirrefences:

-	In step 1, UE#1 sends initial INVITE request with an SDP offer including the combination of bootstrap DC and application DC components;

-	In steps 7 and 8, the media component for bootstrap data channel for terminating side may be included in an SDP early offer to enable the terminating side downloading application before ringing;

-	In step 10, if the UE#2 has not yet downloaded the application, the UE#2 returens a 183 with the SDP answer for bootstrap DC and set ports of application DC to zero, and downloads the corresponding DC Application signalled in the SDP offer;

-	In step 18, the UE#1 sends a SIP UPDATE with an updated SDP contains the application data channel information, as well as the associated DC application binding information, according to TS 26.114 [76]. When the corresponding DC Application is downloaded, the UE#2 sends 200 OK with SDP answer response for the SIP UPDATE, then the application DC is established. The UE#2 start to play ringtone for the user of UE#2 and sends 180 Ringing response to UE#1 after the DC application has been downloaded and the application DC is established. The UE#2 returns a 200 OK response for SIP INVITE if the user of UE#2 accepts to use the DC application requested by UE#1.

NOTE 1:	UE#2 may show the DC application information to the user based on the downloaded DC Application. Such as the human readable name, the detail introduction, the required permissions etc. The detail of what information should be show to the user is out of scope of this specification.

NOTE 2:	The behavior of remote UE responding to the session above is not mandated and is up to its own decision.

NOTE 3:	This procedure covers only the scenario of UE#1 selects a DC application to communicated with UE#2. The bootstrap DC is established only with DCSF of UE#1. If the UE#2 want to add another DC application of UE#2, another bootstrap DC with DCSF of UE#2 may also be established by UE#2 by sending SIP reINVITE message.

### AC.X.2.4 Adding MMTel Media to Standalone IMS Data Channel Session

When an IMS session with standalone data channel between two UEs is ongoing, any of the UE may be triggered by the corresponding user to add audio/vedio/messaging media into the IMS session.

Figure AC.X.2.4-1depicts a signalling flow diagram for adding audio/vedio/messaging media to IMS session with standalone data channel between two UEs.

Figure AC.X.2.4-1 Adding Audio/Video/Messaging Media to IMS Session with Standalone Data Channel

1.	A standalone application data channel has been established between UE-A and UE-B.

2.	UE-A sends SIP re-INVITE which include the media description of both standalone application data channel and audio/video/messaging media description.

3.	The IMS AS, DCSF and UE-B handles the SIP re-INVITE as specified in the TS 23.228 Annex AC 7.2.1.

4. 	UE-B sends 200 responses to UE-A through IMS AS, indicating the call has been established. IMS AS sends notify message to DCSF as specified in Rel-18.

### AC.X.2.5 Removing MMTel Media from IMS Data Channel Session

When an IMS session with data channel and audio/video established between two UEs is ongoing, any of the UE may be triggered by the corresponding user to remove audio/vedio media from the IMS session, which turns the session to an IMS session with standalone data channel.

Figure AC.X.2.5-1depicts a signalling flow diagram for removing audio/vedio media from IMS data channel session between two UEs.

Figure AC.X.2.5-1 Removing Audio/Video/Messaging Media from Exisiting IMS Session

1.	An IMS DC session has been established between UE-A and UE-B, which include audio/video and IMS DC media.

2.	UE-A sends SIP re-INVITE with an SDP containing media information for audio/video media and data channel, which sets the port number of audio/video media stream to zero, as specified in RFC 3264 [72].

3.	The IMS AS, DCSF and UE-B handles the SIP re-INVITE.

4. 	UE-B sends 200 responses to UE-A through IMS AS, indicating the audio/video media has been removed. IMS AS sends notify message to DCSF as specified in Rel-18.

* * * End of Changes * * *