3GPP TSG-WG SA2 Meeting #165	S2-2410364

# Hyderabad, India, October 14 – 18, 2024		(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5766 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration for Xn HO between MWABs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Mobility Support was added to the spec in the last meeting. When a UE in RRC_CONNECTED moves between MWABs, Xn handover can be supported via the path between MWAB-UEs. One possible path between MWAB-UEs is from an MWAB-UE to UPF to another MWAB-UE, which includes 2 NR Uus and 2 backhauls between BH gNB and BH UPF. Another possible path between MWAB-UEs is PC5 link between the 2 MWAB-UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Xn Handover between MWABs to support for Mobility of UEs served by MWABs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility regarding MWAB is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.2.1, 5.49.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR 23.501 CR 5596 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.49.2.1	Configuration of the MWAB-UE

The MWAB-UE manages the BH PDU Session(s) for the MWAB operation based on the configuration by the HPLMN or Subscribed SNPN of the MWAB-UE. The configuration of the MWAB-UE can be UE Local Configuration, or URSP rules as defined in TS 23.503 [45], for the determination of the proper PDU session parameters, e.g. the DNN, S-NSSAI, the SSC Mode, etc. The URSP rules can be preconfigured or provisioned to the MWAB-UE by its HPLMN using UE Policy delivery as described in clause 4.2.4.3 of TS 23.502 [3]. The configuration of MWAB-UE may also be 5G ProSe Policy/Parameters based on MWAB-UE’s authorized 5G ProSe capability.

5.49.2.2	Configuration of the MWAB-gNB

The MWAB-gNB's configuration is managed by the OAM of the MWAB Broadcasted PLMN/SNPN, and the address (or FQDN) of the OAM server per MWAB Broadcasted PLMN/SNPN may be configured at the MWAB-gNB. The OAM server of the MWAB Broadcasted PLMN/SNPN provides configuration parameters e.g. for the purpose including the access stratum operation of the MWAB-gNB, the N2, N3 and Xn interface management, activating/deactivating the MWAB-gNB operation, and to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing.

Xn information may include list of MWABs and gNBs for Xn over PC5, list of MWABs and gNBs for Xn over BH PDU Session, and ProSe information for Xn interface which may include:

-	5G DDNMF address of MWAB Broadcasted PLMN

-	ProSe Application Server for Xn service

-	Application Identifiers for Xn service

-	Location of MWAB and gNB

Editor's Note:	Details of the configuration information needs the coordination with SA5 and RAN3.

# * * * * End of changes * * * *