3GPP TSG-WG SA2 Meeting #165	S2-2410370

# Hyderabad, India, October 14 – 18, 2024		(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5769 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Xn Handover between MWABs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Mobility Support was added to the spec in SA2#164. When a UE in RRC_CONNECTED moves between MWABs, Xn handover can be supported via the path between the MWAB-UEs. One possible path between the MWAB-UEs is from an MWAB-UE via the UPF of the BH PLMN to another MWAB-UE, which includes 2 NR Uus and 2 backhauls between BH gNB and BH UPF. Another possible path between the MWAB-UEs is PC5 link between the MWAB-UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Xn Handover between MWABs to support for Mobility of UEs served by MWABs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility regarding MWAB is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR 23.501 CR 5596 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.49.5.1	UE mobility when a MWAB-gNB cell is involved

The existing UE mobility procedures without MWAB-specific UE enhancements can be reused to handle a UE mobility to and from a MWAB-gNB while the UE is in CM-IDLE or CM-CONNECTED modes, irrespective of whether the MWAB is static or the MWAB is moving, i.e.:

-	For UEs in RRC_IDLE and RRC_INACTIVE state procedure for cell (re ) selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE is used as normal irrespective of cells are belonging to a MWAB-gNB or a gNB.

-	For UEs in RRC_CONNECTED state, the MWAB-gNB or gNB triggers the handover procedures to the neighbouring cells (irrespective of whether they are belonging to a MWAB-gNB or a gNB) for UEs in RRC_CONNECTED state. When UEs in RRC_CONNECTED state move from an MWAB-gNB to another MWAB-gNB, Xn handover may be supported via a path from an MWAB-UE via the UPF of BH PLMN to another MWAB-UE or via PC5 path between the MWAB-UEs.

UEs and MWAB-gNB use existing procedures as specified in clause 8.23 of TS 38.401 [42], TS 23.502 [3], or TS 38.304 [50].

# * * * * End of changes * * * *