3GPP TSG-SA WG2 Meeting #165	S2-2410403

# Hyderabad, India, October 14-18, 2024									(revision of S2-2409421)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1428 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-Satellite-UE communication in IMS - Functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, CATT, Samsung, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Support of UE-Satellite-UE communication in IMS focusing on functionality. This CR is written based on the following principle as well as the above-mentioned conclusion. - At call setup, SDP offer in SIP INVITE arriving to the terminating network contains the IP address information of the ground IMS AGW only and does not contain the one of the satellite IMS AGW. Practically, SDP answer and the next SDP offer are used by P-CSCF for negotiation. No new SDP attribute is introduced. (A new SIP header is introduced for conveying satellite ID.) - At change of satellite, either P-CSCF or IMS AS (requested by P-CSCF via SIP MESSAGE) sends SIP re-INVITE during the procedure to complete IMS AGW relocation. I.e., two options are described. No new SDP attribute is introduced. - No impact related to Subscription data. - No UE impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Support of UE-Satellite-UE communication in IMS with regards to functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

Annex AX (normative):
Support of UE-Satellite-UE communication in IMS

AX.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites with the media not transiting through any network elements on the ground. The ground fallback routing refers to the case with the media transiting through the ground segment.

The P-CSCF is responsible for determining whether to activate UE-satellite-UE communication, that is, whether it is possible to apply media routing that only relies on IMS user plane on-board satellite. P-CSCFs serving the MO UE and the MT UE shall negotiate with each other to determine whether to activate UE-satellite-UE communication.

AX.2	Architecture and functional entities

AX.2.1	Architecture

Editor's note: 	Architecture is FFS.

AX.2.2	Functional entities

AX.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionalities:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in Annex AX.3.

-	the P-CSCF interacts with IMS AGW, as described in Annex AX.4.

AX.3	Optimized media routing activation

AX.3.1	At call setup

The P-CSCF receives from the PCF the identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the identifier of the satellite serving the remote UE as per clause AX.5.1. The P-CSCF shall use this information to determine whether or not activate optimized media routing.

NOTE:	How the P-CSCF uses the satellite identifier to derive whether the two satellites are connected and whether optimized media routing is possible is not specified.

If P-CSCF determines the activation of optimized media routing, the P-CSCF shall instruct PCF to authorize the necessary resources as per clause 5.6.2 and clause 5.7.2 and as defined in TS 23.502 [94], so that the PCF proceeds to establish a path for optimized media routing.

AX.3.2	At change of satellite

The P-CSCF shall determine whether optimized media routing continues to be possible between a satellite that is about to serve the local UE and the satellite serving the remote UE based on the identifiers of those satellites. The P-CSCF obtains the identifier of the new satellite from PCF as per clause AX.5.2.1 and as defined in TS 23.502 [94].

If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF shall send a message to the PCF as per clause AX.5.2.1 and as defined in TS 23.502 [94] to establish a path on the target satellite for optimized media routing.

If the P-CSCF determines that optimized media routing is no longer possible e.g. due to a change of satellite, the P-CSCF shall execute the ground fallback procedure as per clause AX.5.2.2 to reroute the media via the ground.

AX.4	Interaction with IMS AGW

AX.4.1	At call setup

P-CSCF shall select an IMS AGW on ground according to TS 23.334 [74] at first at call setup or while not activating optimized media routing. If the P-CSCF later determines the activation of optimized media routing, the P-CSCF shall release the IMS AGW on ground if any when needed and select instead an IMS AGW on satellite according to TS 23.334 [74].

P-CSCF selects an IMS AGW on satellite based on the satellite identifier. It is assumed that IMS AGW has provided its satellite identifier to P-CSCF during the IMS AGW registration procedure.

AX.4.2	At change of satellite

When P-CSCF is informed that the satellite serving UE changes as defined in TS 23.502 [94], the P-CSCF shall initiate to establish an IMS-AGW on the target satellite as per clause AX.5.2.1 or on ground as per clause AX.5.2.2 depending on whether optimized media routing continues to be possible.

After completion of establishment of the IMS AGW on the target satellite or on ground, the P-CSCF shall inform the local UE and the remote network to use the newly established IMS AGW.

## ---End of the Change---