3GPP TSG-WG SA2 Meeting #165	S2-2410411

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion and analysis on Device ID related issues

Document for:	Discussion

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: A discussion on AIoT Device IDs, types and their uses.

1. Introduction

Following Ambient IoT Device ID issues have been discussed during the study phase:

-	Ambient IoT Device Identifier components

-	Temporary ID

-	Permanent Equipment ID

This discussion paper analyses the above topics to assess whether the above IDs or ID components are necessary and how to use different IDs or ID components by considering the use cases and the Ambient IoT Device complexity.

2. Discussion

2.1	Ambient IoT Device Identifier Components

Based on the interim conclusion, in clause 8.2.1 of the TR, about Identifier and Identification Management, the Ambient IoT Device Identifier can be assigned by an operator or by a third party. The main difference for these two different assignment modes is that which entity determines the identifier of the component used to identify a specific Ambient IoT Device.

In order to make sure that all these Ambient IoT Device Identifiers are unique globally (as the aim of the Ambient IoT Device Identifier is to identity the AIoT Device and locate subscription-like information), another component is required to identify who holds the subscription-link information (i.e. the one who assigned the Ambient IoT Device Identifier and ensured uniqueness) so that the network can interact with the corresponding Credentials Holder to perform ID validation. Similar to the SNPN feature, the Credentials Holder can be either operator network or an entity external an operator network.

Based on the above consideration, the following components are assumed to be included in the Ambient IoT Device Identifier:

-	Home Network Identifier: either identify an operator network holding the subscription-like information. The subscription-like information can then be used to locate a Credential Holder.

-	Instance Identifier: identify a specific Ambient IoT Device within the Home Network Identifer.

Hence, the unified format for the Ambient IoT Device Identifier can be represented as:

Figure 2.1-1: Unified Structure of Ambient IoT Device Identifier

Particularly, if the Ambient IoT Device Identifier is assigned by an operator, the structure of the Ambient IoT Device Identifier can be:

Figure 2.1-2: Structure of Ambient IoT Device Identifier assigned by Operator

In this case:

- The network identifier is used to identify an network holding subscription-like information for ID validation.

- The Instance Identifier (i.e. Operator Allocated Identifier) component can be further subdivided to include Enterprise, group, category information etc., which is based on implementation.

If the Ambient IoT Device Identifier is assigned by a third party, the structure of the Ambient IoT Device Identifier can be:

Figure 2.1-3: Structure of Ambient IoT Device Identifier assigned by Third Party

In this case:

- The network identifier is used to identify either an operator network holding subscription-like information for ID validation or indicate that it is an external entity (i.e. no operator network) holding subscription-like information, for example, by setting the Network Identifier as a special value. The network can check the local configuration to determine the external entity which is responsible for ID validation.

- The Instance Identifier (i.e. Operator Allocated Identifier) component can be GS1 EPC [1] or customized identifier.

In this way, if a Network Identifier is set, then in combination with the instance identifier, it can be used to locate and access subscription-like information in the same way as the operator allocated identifier (e.g. the Network Identifier is used to locate the UDM holding the subscription-like data, and the instance identifier is used to locate the subscription record within that UDM).

2.2	Analysis on the Feasibility of Temporary ID

Temporary ID is proposed to achieve shorter ID report and address privacy issues and needs to be rewritten in the device while it is deployed. However, according RFID related protocol, papers and product sheet [2][3][4][5][6][7][8][9][10][11], the following complexity, performance and efficiency impacts can be foreseen (for detailed analysis, S2-2408556 can be referred):

Observation 1: The read sensitivity is up to -25.5 dBm with a dipole antenna, while the write sensitivity is up to -20 dBm with a dipole antenna. In other words, the power consumption of writing operation to non-volatile memory is three times as many as read operation. Therefore, for the non-volatile memory in Device 1, the writing operation consumes higher power than reading operation.

Observation 2: According observation 1, if all conditions except communication range remain same for read operation and write operation, shorter communication range is required to ensure reliable write operation compared with read operation. Hence, the communication range for reliable writing operation is expected to be shorter than reading operation.

Observation 3: The memory write speed is 3.2 ms per Write, BlockWrite, Lock or Kill operation, for writing up to 32 bits. In other words, the latency of writing operation can be 10 times as much as the latency of reading operation. Therefore, the writing operation introduces much larger latency compared with reading operation, leading to significantly lower service operation efficiency.

Observation 4: Based on the Impinj product data sheet [6][7][8] which compares the performance between M700 Series RAIN RFID Tag Chips and M700 Series RAIN RFID Tag Chips with Extended Memory, the performance table also shows that the Tag Chips with extended memory could result in lower read sensitivity. Hence, enlarging non-volatile memory will decrease read / write sensitivity, further leading to increasing power consumption, decreasing throughput and increasing latency.

Observation 5: Based on the Impinj RFID tag products feature introduction [7][8], the tag memory (non-volatile memory) is designed to endure 10,000 write cycles or retain data for 10 years. Hence, the writing operation should be avoided as much as possible to enable a longer life cycle of an Ambient IoT Device.

Observation 6: Using Temporary ID adds more complexity to an Ambient IoT Device, increases power consumption and decreases service operation efficiency significantly because additional procedures are required write the Temporary ID.

Observation 7: Applying a Temporary ID mechanism would shorten the life cycle of an Ambient IoT Device due to the limitation of maximum number of writing operation endured by an Ambient IoT Device.

Observation 8: Additional issues are introduced by a Temporary ID (e.g. synchronization between an Ambient IoT Device and network especially when the Ambient IoT Device moves to a new area) which have not been addressed.

Observation 9: Variable Device ID length and truncated Device ID report design can solve the requirement of shorter ID report. There are also alternatives to address privacy issues instead of using temporary ID.

Additionally, based on the LS from RAN 1 (R1-2407364), writing may not always be possible at all times and RAN1 suggests that frequent writing to NVM should be avoided.

Hence, based on the overall consideration, it is not recommended to be supported by SA2.

A harmonized design for both Device 1 and Device 2 is the design target pursued by RAN. From CN side point of view, this should also be the fundamental principle to be considered for solution design and make the solution applicable to the low-end capability. The mechanisms applied to high-end devices can be considered in the future release.

Proposal: Temporary ID is not supported by the Ambient IoT.

2.3	Permanent Equipment Identifier

In some solutions, Permanent Equipment Identifier is also mentioned. However, due to the ultra-low complexity for Ambient IoT Device, we need to think about following questions:

- Whether Ambient IoT Devices need to be configured with Permanent ID and Permanent Equipment ID simultaneously?

- Whether the ID reported by the Ambient IoT Devices and the ID used by Operator for Device management should be different?

Firstly, current UE is configured with PEI for following purpose, in addition to the SUPI for the subscription:

-	PEI or IMEI is used to identify the equipment of the UE

-	PEI or IMEI is used by Operator to track the stolen cellphone and prevent it from accessing to the network via access control.

However, if the Ambient IoT Device is additionally configured with a PEI, complexity will be increased further:

- The required memory is enlarged:

  - The size of chipset is increased, leading to higher power consumption

- Cost is increased due to larger memory bank requirement

- The size of message sent by AIoT Device is increased:

  - Increase transmission latency, lower inventory efficiency

  - More power consumption is needed

Therefore, considering the complexity it brings if PEI is additionally configured, we must think about the necessity carefully.

-	Firstly, from Ambient IoT service point of view, as long as Ambient IoT Device Identifier can be used to identify an Ambient IoT Device and subscription, there is no need to use PEI to identify a specific Ambient IoT Device.

-	Secondly, if an operator would like to prevent an AIoT Device from “accessing” the network, disable operation can be triggered to prevent the Ambient IoT Device from responding to the network by using the Ambient IoT Device Identifier to trigger such procedure.

-	Thirdly, from Device management point of view, there may be some concern that if the Ambient IoT Device Identifier is not assigned by the operator, the operator cannot use such Identifier to locate the subscription-like information. However, the Ambient IoT Device Identifier is just a string or number, no matter which entity assigns it, and it can be used to index the subscription-like information.

-	Last but not least, there are no assumptions that a single identifier can’t identify an AIoT Device an locate the subscription-like information or that an Ambient IoT Device Identifier onboarding procedure can’t be supported. The Ambient IoT Device Identifier configured in the Ambient IoT Device would not change during the devices lifecycle, which is the same as a PEI lifecycle. Therefore, using one permanent Identifier for Ambient IoT Device is enough and there is no need for a separate additional identifier.

Based on the above analysis, it is proposed that:

-	A PEI is not needed, therefore mandating one not necessary.

3. Conclusion and proposal(s)

It is suggested to update the interim conclusion for Identifier and Device Management as follow:

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management:

-	An Ambient IoT Device Identifier can be assigned by an operator or by a third party.

-	The Ambient IoT Device Identifier includes following components:

-	Home Network Identifier: either identify the operator holding the subscription-like information or indicate that no operator is holding the subscription-like information (e.g. by setting to a specific value).

-	Instance Identifier: identify a specific Ambient IoT Device.

NOTE 1:	The Instance Identifier can be GS1 EPC [10] or based on implementation.

-	The Home Network Identifier is used to locate the network storing subscription-like information. If the Home Network Identifier indicates that no operator holding the subscription-like information, local configuration can be checked to locate the subscription-like information.

4. References

- GS1 TDS Release 2.1: "EPC Tag Data Standard".

- EPC™ Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz

- A 2 Kbits Low Power EEPROM for Passive RFID Tag IC,  Chinese Journal of Electronics (Vol.31, No.1), Jan. 2022

- An ultra low power non-volatile memory in standard CMOS process for passive RFID tags, IEEE Custom Integrated Circuits Conference, Oct. 2009

- IMPINJ M830/M850 SERIES TAG CHIPS TAG CHIP DATASHEET (https://support.impinj.com/hc/article_attachments/30597440310803)

- About RFID and RAIN RFID (https://www.impinj.com/products/technology/about-rfid)

- Impinj M700 Series RAIN RFID Tag Chips with Extended Memory (https://support.impinj.com/hc/article_attachments/11095292602003)

- IMPINJ M700 SERIES TAG CHIP DATASHEET (https://support.impinj.com/hc/article_attachments/30596888296595)

- TAG CHIP DATASHEET IPJ-M780A-A01 IPJ-M781A-A01 (https://support.impinj.com/hc/article_attachments/30027979278611)

- UCODE® RAIN RFID (https://www.nxp.com/products/rfid-nfc/ucode-rain-rfid-uhf:MC_50483)

- https://en.wikipedia.org/wiki/Atomicity_(database_systems)