3GPP TSG-WG SA2 Meeting #165	S2-2410414

HYDERABAD, INDIA, OCTOBER 14-18, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue #2, Evaluations and Conclusions for Subscription management

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusion to support subscription management for AIoT device, AF request and UE reader in the 5G network, according to the evaluations and summary of relative solutions.

1. Introduction

The subscription management is one of the most important functionalities in 5G system. Key issue #2 includes the study point for subscription management of Ambient IoT as follows:

-	Study whether subscription management, registration management and/or connection management are necessary for an Ambient IoT Device or a group of Ambient IoT Devices, and if so identify the necessary state machine(s), procedures and functionality considering the Ambient IoT Devices capability and characteristics.

This contribution gives evaluations and summaries in terms of subscription management for the solutions in the present report, this includes the following aspects:

-	dedicated subscription data for AIoT device, see clause 2.1

-	dedicated subscription data for 3rd party AF, see clause 2.2

-	additional UE subscription data for UE acting as a Reader, see clause 2.3

Finally, this contribution concludes to support the dedicated subscription data for AIoT device, dedicated subscription data for 3rd party AF, and additional UE subscription data for UE acting as a Reader and makes a proposal for inclusion in the TR in clause 3.

2. Discussion

2.1 	Support of subscription management for Ambient IoT Device

2.1.1	Rational

The key issue #1 targets a system architecture to support "Validation of the Ambient IoT Device identifier" as one of the objectives. This is inline with the SA1 Requirements in TS 22.369: "The 5G network shall support suitable management mechanisms for an Ambient IoT device or a group of Ambient IoT devices."

The study assumes that communication spectrum is licensed, thus there is a need to authorize a AIoT device communicating with 5G network via its licensed spectrum. In order to achieve this, the 5G network needs to identify such AIoT device and check whether it is subscribed to the 5G network, i.e., the 5G network needs to perform “Validation of the Ambient IoT Device identifier”. This is the way the MNOs to ensure that their network resources are utilized by authorized users thus to secure their network infrastructures and protect their investments.

When doing an inventory, AIoT Devices will respond to the "network pull" of devices into a network. Resources will have already been used to obtain the identity of the AIoT Device. A main purpose of Inventory is to determine what AIoT Devices are present, and as AIoT Devices can be nomadic, devices will leave and new devices will arrive. The inventory procedure determines the identity of an AIoT Device, and therefore determine whether it is manageable and should be communicated with by the operator. We cannot and do not want to prevent devices from responding to inventory, but an operator can subsequently determine whether to allow further access/communication with that device, including for example, not reporting it in Inventory results to an AF, not passing on commands to the AIoT Device from an AF etc. Subscription-like information is the basic building block which is used by an operator to determine whether the device is known and therefore to be communicated with by an AF.

Observation 1: In a nutshell, the business requirements require the 5G network to provide validation of the Ambient IoT Device identifier as one of the system functionalities, and there's no doubt the subscription management of Ambient IoT device is used to achieve the validation of the Ambient IoT Device identifier.

2.1.2	Solution Summary

There are 42 solutions in total in the present report, the below table 2.1.2 shows the summary on the following aspects regarding subscription management for Ambient IoT Device:

-	Is “Validation of the Ambient IoT Device identifier“ within 5G network.

-	If yes, is "dedicated subscription data for AIoT device" used.

-	If yes, where to store the "dedicated subscription data for AIoT device".

-  What items are in "dedicated subscription data for AIoT device".

Table 2.1.2: Summary of the solutions regarding subscription management for Ambient IoT Device

| Is "Validation of the Ambient IoT Device identifier" within 5G network | Yes | Sol#5, 6, 8, 9, 10, 12, 14, 15, 17, 18, 19, 21, 30, 31, 32, 33 |
| --- | --- | --- |
|  | No | Sol#3, 22, 34 |
| If yes, is "dedicated subscription data for AIoT device" used | Yes | sol#5, 6, 8, 9, 10, 12, 14, 15, 17, 18, 19, 21, 30, 31, 33 |
|  | No | Sol#3, 22, 34 |
| If yes, where to store the "dedicated subscription data for AIoT device" | UDM | sol#5, 6, 7, 8, 9, 10, 12, 14, 15, 17, 18, 19, 21, 30, 31, 33 |
|  | AAA | Sol#14, 17 |
| What items are in "dedicated subscription data for AIoT device" | Device ID | Sol#5, 6, 8, 9, 10, 12, 14, 15, 17, 18, 19, 21, 30, 33 |
|  | Last known TMF | Sol#5, 8, 9 |
|  | Last known Reader | Sol#6, 17, 28 |
|  | Validation status | Sol#6， |
|  | Device Status | Sol#6, 8, 10, 15, 17, 33 |
|  | credential | Sol#6, 10, 12, 14, 17, 19 |
|  | a list of AIoT services (service ID, credentials) | Sol#6, 33 |
|  | permissible service area | Sol#33 |
|  | device capability information | Sol#8, 30 |
|  | AM and SM subscription data, online subscription | Sol#25 |
|  | frequency configuration | Sol#39 |
|  | TID | Sol#10 |
|  | No security and privacy operation | Sol#41 |
|  | device owner information | Sol#16 |

16 out of 42 solutions propose that "Validation of the Ambient IoT Device identifier" is performed by 5G core,

-	15 out of the 16 solutions explicitly mention to use subscription data for Ambient IoT device in the UDM to perform "Validation of the Ambient IoT Device identifier"

-	2 out of the 16 solutions explicitly mention to use AAA as an alternative to UDM to store subscription data for Ambient IoT device

-	14 out of the 16 solutions show that Device ID as part of the subscription data for Ambient IoT device

-	6 out of the 16 solutions show that Device Status, Device Credentials as part of the subscription data for Ambient IoT device

-	3 out of the 16 solutions show that last known CN NF, last known Reader as part of the subscription data for Ambient IoT device

There are 3 solutions that explicitly show that the 5G network doesn’t perform the "Validation of the Ambient IoT Device identifier", and it is not clear how to achieve such system functionality as business requirements mandate.

As the ambient IoT Device Identifier can be assigned by an operator or by a third party, if the ID is assigned by the operator, then probably the subscription-like data is stored at the UDM within the core network, and if the ID is assigned by the third party, in addition to the UDM, the subscription-like data has another alternative to be accessed, i.e., AAA external to the network.

Proposal 1: The subscription-like data for AIoT device is used to perform "validation of the Ambient IoT Device identifier", one of the system functionalities for Ambient IoT. Such subscription-like data is dedicated for an Ambient IoT Device and it is stored either in a UDM within the core network or via a AAA external to the AIoT network. The subscription-like data contains at least the AIoT Device ID, Device Status, Device Credentials, last known CN NF, last known Reader.

2.2 	Support of authorising a 3rd party AF

As stated in TS 22.369: "The 5G system shall provide suitable mechanisms to support communication between a trusted and authorized 3rd party and an Ambient IoT device or group of Ambient devices". This requirement assumes that the 3rd party is authorized to 5G network during communication with the Ambient IoT Device.

There are 42 solutions in total in the present report, the below table 2.2 shows the summary on the following aspects regarding subscription data for 3rd party AF:

-	Which NF to authorize the 3rd party to communicate with the Ambient IoT Device

-	What profile is used to authorize the 3rd party to communicate with the Ambient IoT Device

-	If subscription is used, what items are in "dedicated subscription data for 3rd party AF"

Table 2.2: Summary of the solutions regarding subscription data for 3rd party AF

| Which NF to authorize the 3rd party to communicate with the Ambient IoT Device | New AIoT CN NF | Sol#3, 6, 17, 33, 41 |
| --- | --- | --- |
|  | NEF | Sol#4, 5, 8, 10, 11, 15, 18, 19, 21, 27, 30, 31, 39 |
| What profile is used to authorize the 3rd party to communicate with the Ambient IoT Device | Subscription data in UDM | Sol#5, 6, 15, 19, 21 |
|  | SLA or Operator Policy | Sol#11, 27, 33 |
| If subscription is used, what items are in "dedicated subscription data for 3rd party AF" | AF ID | Sol#15, 17 |
|  | Type of Request/Allowed service operations | Sol#15, 17 |
|  | Allowed service area | Sol#17, 19 |
|  | Allowed AIoT Device ID scope | Sol#17 |
|  | No security and privacy Indication | Sol#41 |

18 out of 42 solutions propose that the 5GC authorizes the 3rd party to communicate with the Ambient IoT Device

-	13 out of the 18 solutions go with NEF, while 5 go with new Ambient IoT CN NF

-	5 out of the 18 solutions explicitly mention to use subscription from UDM, while 3 go with SLA and operator policy

-	4 out of the 18 solutions explicitly show the items in the subscription data

The NEF is typically used to perform API or AF level authorization, while the new Ambient IoT CN NF can perform finer authorization by looking into the AIoT service parameters of AF request, e.g., type of request, allowed service operations, allowed service area, allowed ID scope, security indication etc. So, the authorization by the NEF and the authorization by the new Ambient IoT CN NF are complementary.

Proposal 2: The new Ambient IoT CN NF or NEF are both used for authorization of the AF request targeting an Ambient IoT service/AIOT Devices. The subscription data for 3rd party AF from UDM is used by new Ambient IoT CN NF while the SLA and operator policy are used by NEF.

2.3 	Support of additional UE subscription data for UE Reader

As stated in TS 22.369: "Based on subscription and operator policies, the 5G system shall authorize an Ambient IoT capable UE to communicate with a specific Ambient IoT device or with a group of Ambient IoT devices". This requirement clearly indicates that the UE subscription is used to authorized UE to act as a Reader for communication with the Ambient IoT Device.

There are 42 solutions in total in the present report, the below table 2.3 shows the summary on the following aspects regarding subscription data for 3rd party AF:

-	Which NF to authorize the UE as Reader

-	What new items are in UE subscription data

Table 2.3: Summary of the solutions regarding UE subscription data for UE Reader

| Which NF to authorize the UE as Reader | New AIoT CN NF | Sol#11 |
| --- | --- | --- |
|  | AMF | Sol#4, 12, 21, 22, 30, 31, 32, 34 |
|  | UDM | Sol#4, 11, 12, 22, 30, 31, 32, 34 |
| What new items are in UE subscription data | Whether the UE is enabled/disabled to serve as AIoT intermediate node | Sol#32 |
|  | A list of AIoT service(s) for which is allowed to transmit AIoT data/signaling to the AIoT devices | Sol#32 |
|  | Selection information for new AIoT CN NF | Sol#32 |

9 out of 42 solutions propose to use AMF and UDM to authorize the UE as Reader during UE registration procedure, while 1 out of 42 solution proposes to use the new AIoT CN NF.

Only 1 out of 42 solutions explicitly shows the new items in the UE subscription.

The AMF is the anchor point for UE access, it is convenient and easy for AMF to handle UE subscription to authorize the UE to act as a Reader.

Proposal 3: The UE subscription is enhanced to indicate whether the UE is enabled/disabled to serve as AIoT intermediate node, this indication in UE subscription is used by AMF to authorize the UE as a AIOT reader during the UE registration procedure.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.2	Interim Conclusion on Key Issue #2

8.2.x	Subscription Management

Subscription data for an Ambient IoT Device is required:

-	It is used by the new Ambient IoT NF (AIoTF) to achieve validation of the Ambient IoT Device identifier

-	It is dedicated subscription data for Ambient IoT Devices, i.e., different with UE subscription data.

-	It is stored either in the UDM within a 5G network or accessed via AAA server and external to 5G network.

-	It contains at least an Ambient IoT Device ID, Device Status (Active, to be disabled, Disabled etc.), Device Credentials, last known CN NF, last known Reader.

NOTE:	Security related materials in subscription data for Ambient IoT device depends on the outcomes from SA3 study.

Subscription data for 3rd party AF is optional to support, if it is required:

-	It is used by the new AIoTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service.

-	it is stored at UDM within 5G network.

UE Subscription data is enhanced to include an indication that whether the UE is enabled/disabled to serve as AIoT intermediate node, this indication in the UE subscription is used by the AMF to authorize the UE as a AIOT reader during UE registration procedure.

# * * * * End of changes * * * *