3GPP TSG-WG SA2 Meeting #165	S2-2410415

HYDERABAD, INDIA, OCTOBER 14-18, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3 Conclusion: For information transfer

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusions to use AIoT NAS and NEF services to support information transfer according to the evaluations and summary of relative solutions

1. Introduction

Key Issues #3 includes the study on how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices and related services. The exposure of the services to an AF should follow the existing 5GS reference architecture to use an NEF for an AF to request service for AIoT.

According to the SA1 requirements in TS 22.369, the 5G network shall play a key role to transfer the AIoT data between 3rd party and Ambient IoT device, for example:

-	Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to obtain data from Ambient IoT devices (e.g., sensor data) and provide it to a trusted 3rd party via the 5G network.

-	The 5G system shall be able to support mechanisms to communicate: between an Ambient IoT device and the 5G network using Ambient IoT direct network communication or Ambient IoT indirect network communication, or

-	The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in a specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

-	The 5G system shall be able to collect charging information in a suitable way for using Ambient IoT services on per Ambient IoT device basis or a group of Ambient IoT devices (e.g., total number of communications per charging period)

-	The 5G system shall enable security protection suitable for Ambient IoT, without compromising overall 5G security protection.

-	The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network

Observation: The 5G network plays a key role to transfer the AIoT data between a 3rd party and an Ambient IoT Device, this includes 1) receiving 3rd party instruction to trigger a group of Ambient IoT Devices, 2) Trigger a group of Ambient IoT devices in a specific area and which action the Ambient IoT Devices need to perform when triggered, 3) obtaining data from Ambient IoT devices and provide the AIoT data to the 3rd party, 4) collect charging information per Ambient IoT device basis or a group of Ambient IoT devices, 5) security and privacy protection of information exchanged between an Ambient IoT device and the 5G network.

This contribution gives evaluations and summaries in terms of information transfer for Ambient IoT services in key issue #3, and concludes to support AIoT NAS and NEF services for information transfer of key issue #3.

2.	Summary and Conclusions

2.1 Information Exchanged Summary

2.1.1	Overview

There are 42 solutions in total in the present report and most of solutions in the present report support the following types of information to be exchanged:

-	Inventory Request / Inventory Response

-	Command Request / Command Response, Command including Read, Write and Disable

1 solution additionally supports state change or control. 3 or 4 solutions additionally support Enable or Execute or Temp ID. Because of the little support of those information exchanged, thus it is proposed to focus on the Inventory and Read, Write, Disable Commands.

The types of information exchanged is the same for Topology 1 and Topology 2. The below Table 2.1.1-1 shows the summary for information transfer Ambient IoT services.

Table 2.1.1-1: Summary of Type of Information Exchanged

| Type of Information Exchanged | Solutions |
| --- | --- |
| Inventory Request to trigger the AIoT devices | Sol#3, 4, 5, 6, 8, 10, 11, 14, 16, 17, 18, 19, 20, 21, 22, 23, 26, 27, 28, 29, 30, 31, 32, 33, 37, 38, 40, 41, 42 |
| Inventory Response with AIoT Device ID | Sol#3, 4, 5, 6, 8, 10, 11, 14, 16, 17, 18, 19, 20, 21, 22, 23, 26, 27, 28, 29, 30, 31, 32, 33, 37, 38, 40, 41, 42 |
| Read Command Request/Response | Sol#3, 4, 5, 6, 8, 9, 11, 14, 17, 18, 19, 20, 21, 22, 26, 27, 28, 30, 31, 32, 33, 42 |
| Write Command Request/Response | Sol#3, 4, 5, 6, 8, 9, 11, 14, 17, 18, 19, 20, 21, 22, 26, 27, 28, 30, 31, 32, 33, 42 |
| Disable Command Request/Response | Sol#3, 4, 5, 6, 8, 9, 14, 17, 18, 19, 20, 21, 22, 26, 27, 28, 30, 33, 39, 42 |
| Control | Sol#4 |
| Enable | Sol#4, 22, 30, 39 |
| State Change | Sol#15 |
| Execute | Sol#22, 30, 32 |
| Temp ID | Sol#1, 13, 36, 37 |

From this information the AIoT service operations an AF can request are Inventory, Read, Write and Disable.

The AF should not be involved in whether specific Reader is a Topology 1 reader or a Topology 2 reader. By keeping this exposure topology agnostic, 3GPP can provide a unified system to an external application, while allowing the 3GPP system to manage the readers and the communication with them. This is the key concept of a “unified architecture”.

The other key concept of a “unified architecture” is that the AIoT Device operates and behaves the same for a UE Reader or a RAN Reader, i.e., the AIoT Device is also topology agnostic, and the conclusions assumes this case.

With these 2 external interfaces (to the AIoT Device and to the AF) the 3GPP system is unified.

2.1.2 Common Parameters

The service operations have common parameters, including which Reader(s) an operation is targeting (represented by a set) and which AIoT Devices an operation is targeting.

The readers are identified by a set, and the construction of the set is up to operator policy. A set can be used to represent a single specific reader, a reader in a specific area, all readers in an area, mobile readers, static readers, etc, this is all entirely deployment dependent based on e.g., physical locations, operators and applications preferences and service agreements, and therefore can’t be standardised, so a flexible approach is needed.

An AIoT Device has an AIoT Identifier, which is assumed to include home network operator and individual AIoT Device identification, and this conclusion does not assume any assignment or ownership models. When performing an operation, the AIoT Device(s) are paged by a Reader, and a MASK is used in the paging allowing an AIoT Device to determine whether it is the target for the paging. The AIoT Device does this by comparing a variable length bitstring within the MASK against its AIoT Device Identifier at an offset included in the MASK, as illustrated in Figure 2.1.2-1 below.

Figure 2.1.2-1: MASK Comparison with AIoT Device Identifier

Following the existing technologies in the RFID space, for example GS1, an AIoT Device has memory which can be read and written to and the Read/Write service operations allow an AF to read/write data from/into that application memory in the AIoT Device. Therefore, the AF needs to provide an address to read from or write to, a length to read or write, and in the case of write the actual data to write. The AF is provided with the result of a Read or Write request, which in the case of Read includes the data read from the AIoT Device.

2.2	Summary of Topology 1 Solutions

There are 42 solutions in total in the present report, the below Table 2.2-1 shows the summary of the path for information transfer for Ambient IoT services in topology 1.

For Inventory: there is also a consideration of the RAN agreements for “paging” and responses. Examining (many of / all of) the solutions there seems to be a strong mapping of Inventory Request (i.e., a request for a specific AIoT Device / group of AIoT Devices to report their identity) to the RAN “paging” like step and the Inventory Response as a result of the “paging”.

-	Inventory Request:

-	20 of the 42 solutions propose to use hop-by-hop Path (AIoT Device <-> Reader <-> 5GC <-> AF).

-	Inventory Response:

-	5 out of the 42 solutions propose to use hop-by-hop Path (AIoT Device <-> Reader <-> 5GC <-> AF)

-	15 out of the 42 solutions propose to use AIoT-NAS Path (AIoT Device <-> 5GC <-> AF)

For Command Request/Command Response:

-	2 out of the 42 solutions propose to use hop-by-hop Path (AIoT Device <-> Reader <-> 5GC <-> AF)

-	16 out of the 42 solutions propose to use AIoT-NAS Path (AIoT Device <-> 5GC <-> AF)

-	1 out of the 42 solutions propose to use App Path (AIoT Device <-> AF)

Table 2.2-1: Summary of the solution paths for Inventory/Commands in Topology 1

| Path for Inventory Request/Response | Hop-by-Hop Path: AIoT Device <-> Reader <-> 5GC <-> AF | Inventory Request: Sol#3, 4, 5, 6, 8, 10, 14, 16, 17, 18, 19, 20, 26, 27, 28, 29, 33, 37, 40, 41 Inventory Response: Sol#3, 19, 20, 40, 41 |
| --- | --- | --- |
|  | AIoT NAS Path: AIoT Device <-> 5GC <-> AF | Inventory Request: None Inventory Response: Sol#4, 5, 6, 8, 10, 14, 16, 17, 18, 26, 27, 28, 29, 33, 37 |
| Path for Commands & Response related services | Hop-by-Hop Path: AIoT Device <-> Reader <-> 5GC <-> AF | Command/Response: Sol#19, 20 |
|  | AIoT NAS Path: AIoT Device <-> 5GC <-> AF | Command/Response: Sol#4, 5, 6, 8, 9, 10, 14, 16, 17, 18, 26, 27, 28, 29, 33, 37 |
|  | App Path: AIoT Device <-> AF | Command/Response: Sol#3 |

3GPP shall be responsible for the E2E call flow without relying on other protocols. If service operation is transferred at APP layer, it means that AIoT Device is produced with customization, which leading to no massive production and the cost would be high.

AIoT NAS provides a way for the 5GC to have communication with the AIoT Device without RAN having to be involved in all the exact details of every flow end to end. There always needs to be information about the requested operation and its associated parameters, the NAS header is a small delta on those parameters. There is a small NAS overhead with the introduction of the NAS Header in NAS path, compared with hop-by-hop path. This could be a single octet (or less if 8-bit alignment is not required). This small NAS overhead brings limited impacts for the most commonly used inventory and read scenarios, considering many benefits it brings.

AIoT NAS will be an important building block for future enhancements and evolution if the specification in the future, it is not just related to security, but overall functionality, for example the basic operations like Read/Write.

Therefore, AIoT-NAS to support information transfer for Ambient services between AIoT Device and 5GC is recommended.

2.3	Summary of Topology 2 Solutions

The path for information transfer for Ambient IoT services in topology 2 and comparisons of the impacts is shown in Table 2.3-1.

Table 2.3-1 Summary of the solution paths for Inventory/Commands in Topology 2

| Sol Categories | Solutions | Impacts summary | Pros | Cons |
| --- | --- | --- | --- | --- |
| Control Plane Based | Sol#4, 6, 10, 11, 12, 17, 21, 23, 27, 28, 30, 31, 33, 38, 39 | The UE reader transmits AIoT data over the control plane: AIoT Device <-> UE Reader <-> 5GC <-> AF. Enhanced UE NAS to support AIoT NAS transfer/control UE reader. Dedicated AIoT NAS between AIoT Device and 5GC. Enhanced AMF to support AIoT message routing. New NEF AIoT service. | No impacts on RAN Suitable for Rel-19 ultra-low complexity Ambient IoT devices, whose data is small and infrequent. | AMF stands in between the RAN and new NF for every interaction between the AIoT device and the 5GC. |
| User Plane Based | Sol#32 | The UE reader transmits AIoT data over the user plane terminated at 5GC: AIoT Device <-> UE Reader <-> RAN <-> UPF <-> 5GC <-> AF. Configuration of a PDU Session at UE and UPF (AIOTF Address, DL/UL Port ID) for AIoT data. Extra Subscription / policy for the PDU Session for AIoT data (UE reader subscription, AIoTF address, DNN/S-NSSAI for AIoT). Dedicated AIoT NAS between AIoT Device and 5GC. Dedicated UE AIoT layer/protocol between UE and 5GC to control UE reader and exchange the DL/UL Port ID. New NEF AIoT service. | No impacts on RAN and UE NAS from the path point of view. No Impacts on AMF from the path point of view. | Sensitive information for 5GC CP NF (AIoTF address, Port ID) is exposed to the external entity, this has security problems and how to ensure the connectivity between UE reader and 5GC is not clear. UE requires the cross Application - AS layer interaction in the UE so the message received from AIoT AF via UP will trigger the AS layer functionality towards AIoT devices. 5GC CN NF is bound to UE via subscription, this restricts the flexibility for AIoT service, e.g., AIoTF relocation during UE movement. |
| Application-Level Based | Sol#3, 4, 22, 34 | The UE reader transmits AIoT data over the user plane terminated at AS: AIoT Device <-> UE Reader <-> RAN <-> UPF <-> AF. Pre-configuration of a PDU Session at the UE (DNN, S-NSSAI, AIoTF FQDN) for AIoT access. Dedicated UE AIoT layer between UE and 5GC to manage UE reader. | No Impacts to 5GC except the configuration of dedicated PDU Session and the new UE AIoT layer from the path point of view. | No control point in 5GC to allow validation, charging, security etc. |

There are 19 solutions in total to address information transfer in Topology 2:

-	15 out of the 19 solutions support to use Control Plane Based method,

-	1 out of the 19 solutions support to use User Plane Based (termination at 5GC), and

-	4 out of the 18 solutions support to use Application-Level Based (termination at AS).

To allow the 5GC to have control for Ambient IoT services as captured in the observation, then it is proposed to exclude the Application-Level Based mechanism to support information transfer for topology 2.

It is further proposed to exclude the UP based solutions for information transfer for topology 2 considering lowest levels of support for UP based solution, the extra complexity to use a dedicated AIoT PDU Session and the big drawbacks it brings, for example:

-	Extra Complexity: 1) DNN and S-NSSAI for AIoT service in SM subscription, 2) AIoTF address provisioning at UE reader during PDU session procedure, 3) Exchange of AIoT session information (AIoT Service ID, UL port ID and DL port ID etc) between UE Reader and AIoTF, 4) Cross Application - AS layer UE implementation, because when the UE readers receive the command messages from the application in application layer from the PDU session, then the UE needs to trigger the AS layer functionality to inventory or send/receive data from AIoT devices.

-	Security Concerns: Sensitive AIoTF (as 5GC CP NF) information (AIoTF address, Port ID) is exposed to the UE and use plane, this bring additional security concerns and it is not clear how to ensure the security for connectivity between UE reader and AIoTF.

-	Restricted Deployment: AIoTF is bound to a UE Reader via subscription, this restricts the flexibility for AIoT service, e.g., how to support AIoTF relocation during UE Reader movement.

For Control Plane Based method, the AIoT NAS between the AIoT Device and network is exactly the same for topology 1. No extra efforts are required except for e.g., a new 5GC “SM-like” NAS containers to carry the AIoT NAS messages and AIoT service control (e.g., request an inventory) to/from the UE Reader. The impacts are small and the method is suitable for Rel-19 ultra-low complexity Ambient IoT devices, whose data are small and infrequent. Since QoS is not required for AIoT data transmission, any extra relay of AMF in between the AIoT Device and the AIoTF is acceptable.

2.4	Conclusions

Based on the above analysis, observation, summaries and overviews of solutions in the TR, conclusions in the text proposal are made.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change (all texts are new) * * * *

8.3	Conclusion for KI#3: Support of Ambient IoT Services

The key studies the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE 2:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g., for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

The following aspects and principles are agreed for conclusion on support of AIoT Services between the network and the AF:

1.	The NEF provides an AIoT Service to an AF with the following service operations:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device’s application memory.

-	Write: Request to write information to an AIoT Device’s application memory.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

-	Notify: Used to inform the AF consumer of the status and results of the above service operations.

2.	The AIoTF provides services to:

-	perform inventory operations,

-	to send read/write command to an AIoT Device to read/write data in the AIoT Device memory, or to send disable command to an AIoT Device permanently.

3.	The services, services operations and related parameters are common for topology 1 and topology 2 and provide the AF with a unified interface, so an AF does not need to differentate between topology 1 and topology 2.

4.	The AF, common to all operation requests, provides information which identifies the target AIoT Device(s) for the operation. The AIoTF determines a MASK based on the information from the AF. The MASK is a bit string that is provided to a Reader by the AIOTF to be included in the paging message. AIoT devices compare the MASK with (part of) their own AIoT Device Identifier (including Home Network Identifier) to determine whether respond to the paging message.

5.	The AF, common to all operation requests, provides a Reader Set ID which is used to identify a set of readers for the operation. The set of readers can contain a single Reader, or multiple Readers and the Readers can be either RAN Readers for topology 1, UE Readers for topology 2, or a mixture with the set.

NOTE X:	Management of which readers are in each set is up to the network implementation.

6.	The AF, for a Read service operation, provides the address in an AIoT Devices application memory to read from and the length of the data to read.

7.	The AF, for a Write service operation, provides the address in an AIoT Devices application memory to write to, the data to write and the length of the data to write.

The following aspects and principles are agreed for conclusion on support of information exchanged between the AIoT Device and network:

8.	A dedicated lightweight Ambient IoT NAS is used to support the information transfer between Ambient IoT Devices and an AIoTF, and supports the following types of information exchanged:

-	Device ID Report from AIoT Device to 5GC.

-	Read/Write/Disable Command Request from 5GC to AIoT Device.

-	Read/Write/Disable Command Response from AIoT Device to 5GC.

9.	The dedicated lightweigh Ambient IoT NAS is identical for both topologies and the AIoT Device is agnostic to whether it is transferred by a RAN Reader in Topology 1 or a UE Reader in Topology 2.

The following aspects and principles are agreed for conclusion on the support of Topology 2 and UE Readers:

10.	To control the UE Reader functionality the UE NAS is enhanced to carry the AIoT service control (e.g., request an inventory) to/from the UE Reader and transport the dedicated lightweight Ambient IoT NAS between an AIoT Device and an AIoTF via the UEs and serving AMF.

# * * * * End of changes * * * *