3GPP TSG-WG SA2 Meeting #165	S2-2410416

HYDERABAD, INDIA, OCTOBER 14-18, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluations and Conclusions for topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusion proposals to use new Ambient IoT NF to support topology 2 according to the evaluations on respective solutions.

1. Discussion

TR 23.700-13 includes the study for Topology 2 as described in the architecture assumptions:

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

This contribution focuses on the evaluations on solutions for topology 2, summarizing the architecture enhancements for them, making comparisons on the impact, and suggests a way forward as conclusion.

The table below summarizes the architecture enhancements for topology 2 and make comparisons on the impacts:

| Sol Categories | Solutions | Impacts summary | Pros | Cons |
| --- | --- | --- | --- | --- |
| New NF | Sol#3, 6, 10, 17, 21, 23, 28, 30, 31, 32, 33, 38 | A new 5GC NF to support AIoT, this new NF is separate from the AMF. New Interfaces/Services relative to this new NF Enhanced AMF to support AIoT message routing. New Services for the NEF. New UDM subscription to authorize the UE to act as a reader. | The new NF is dedicated for Ambient IoT, this allows flexible deployment options and keeps consistent with topology 1 architecture. | AMF stands in between the RAN and new NF for every interaction between the AIoT device and the 5GC. |
| Enhanced AMF | Sol#4, 10, 28, 31, 33, 34, 38 | AMF enhancements to support AIoT. New Services for the AMF. New Services for the NEF. New UDM subscription to authorize the UE to act as a reader. | NFs and Interfaces in the 5GC can be reused as much as possible. | Separate architecture option from Topology 1. Not able to enable flexible deployment. |
| Enhanced NEF | Sol#11, 42 | NEF enhanced to support AIoT. AMF enhanced to support AIoT message routing. New Services for the AMF. New Services for NEF. New UDM subscription to authorize the UE to act as a reader | NFs and Interfaces in the 5GC can be reused as much as possible. | Separate architecture option from Topology 1. Not able to enable flexible deployment. |
| Application-Level Solutions | Sol#3, 22 | Dedicated AF/AS to support AIoT, 5G network provides data pipe to transparently transfer Ambient data. | No Impacts to 5GC except a dedicated PDU Session. | No control point in 5GC to allow validation, charging, security etc. |

Summary: there are 18 solutions in total to address architecture issues for topology 2

-	12 out of the 18 solutions support to use the new NF,

-	7 out of the 18 solutions support to use enhanced AMF

-	2 out of the 18 solutions support to use enhanced NEF

-	2 out of the 18 solutions support to use Application-Level mechanism

Proposal 1: To keep architecture consistent with topology 1 and allow the 5GC to have control points for Ambient IoT services and AIoT Devices, it is proposed to have the new Ambient IoT NF in 5GC to support the topology 2. To enable flexible deployment, it is an implementation choice of operator whether to collocate the new NF with AMF or NEF, or whether to keep it as a standalone NF.

Proposal 2: To authorize/revoke the UE to act as a Reader, UE subscription in the UDM is enhanced with this authorization, as the solutions in the TR support.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

## 8.1	Conclusion on Key Issue #1

### 8.1.2	Architecture to Support Topology 2

At least the following principles are agreed for the architecture to support topology 2:

-	A new core network function (AIoTF) is introduced to support Ambient IoT.

-	The AMF and UE subscription data in the UDM is enhanced to support authorizing/revoking the UE to act as a reader during UE registration procedure.

# * * * * End of changes * * * *