3GPP TSG-SA WG2 Meeting #165	S2-2410420

Hyderabad, India, 14-18 October 2024

Agenda item:	9.11.2

Source:	Nokia

Title:	Issue with AoI for Slice subject to Area Restriction

WI/Release:	eNS_Ph3/ Rel-18

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 a paper was submitted (R3-244413) observing that in TS 23.501, SA2 has designed a new mechanism in release 18 related to enforcing Network Slice Area of Service not matching deployed tracking areas. In this mechanism, the SMF subscribes to “UE mobility event notification” to be informed by AMF when the UE is “IN” or “OUT” of the NS-AoS and the AMF configures for this AOI reporting towards the gNB where the AoI corresponds to the NS-AoS of the slice.  This is described in TS 23.501 section 5.15.18.3 as follows:

5.	When the AMF determines that the S-NSSAI of a PDU Session is restricted to an NS-AoS in the PDU session, the AMF indicates to the SMF that the PDU Session is subject to area restriction for the S-NSSAI. As a result, the SMF subscribes to "UE mobility event notification" event for reporting UE presence in Area of Interest by providing S-NSSAI to the AMF as described in clauses 5.6.11 and 5.3.4.4 if this event has not been subscribed before. The AMF determines the UE presence in Area of Interest as described in Annex D, clauses D.1 and D.2 of TS 23.502 [3] and notifies the result to the SMF. When SMF is notified that the UE location is outside of Area of Interest, SMF shall not send user data as payload of NAS message (see clause 5.31.4.1) in downlink directions and disable data notification.

Based on the information, when incoming packet arrives, the SMF refrains from triggering Data Notification, Network-triggered PDU session setup and Paging towards the UE for the slice.

However, it was observed in the paper that this mechanism has an issue, especially in the typical case where the UE has several PDU sessions which can be of different S-NSSAIs. The Discussion clause below describes this issue and investigates possible solutions.

# 2	Discussion

Suppose that a UE is operating on 3 slices with respective IDs 1, 2, 3. Each of those slices is associated with a respective area of interest AoI1, AoI2 and AoI3 which correspond to the slice Area of Service (AoS). For example, the AoI1 associated with slice 1 is defined as matching the slice AoS of slice 1. In this example, each of the AoIs is associated with the below cell list:

Slice 1: AoI 1 = {cell 1, cell 2, cell 3}

Slice 2: AoI 2 = {cell 2, cell 3, cell 4}

Slice 3: AoI 3 = {cell 3, cell 4, cell 5, cell 6}

The cells are deployed in an area as shown in the figure below, where each color represents the cells controlled by an NG-RAN node (gNB). For instance, green color (gNB1), orange color (gNB2) and pink color (gNB3). All the cells shown are within the registration area (RA) of the UE.

UE

UE

Figure 1: RA of the UE and cell distribution in TAs of the RA and UE positioning in a cell within the RA.

While the UE moves in the RA, UE is currently in cell 3 that is controlled by gNB1. gNB1 decides to bring the UE to RRC_INACTIVE state. In that case gNB1 has to build the RNA for the UE. Based on gNB1’s decision there are several ways for RNA construction as listed below:

Options: a) RNA = {cell 1, cell 2, cell 3}, b) RNA = {cell 1, cell 2, cell 3, cell 4, cell 5, cell 6}, c) RNA = {cell 3}

If NG-RAN AOI-based location reporting is enabled by SMF (via AMF) then based on the chosen RNA from the gNB we would have the following possible reporting options from NG-RAN reports towards SMF:

- Case 1: big-size RNA: If gNB1 builds RNA = {cell 1, cell 2, cell 3, cell 4, cell 5, cell 6}

  - AoI reporting to SMF looks as: AoI 1 -> UNKNOWN, AoI 2 -> UNKNOWN, AoI 3 -> UNKNOWN

  - SMF has no idea if UE is in the Area of Service of slice 1, slice 2 or slice 3. Therefore, SMF will enable data notification and reactivate PDU sessions all the time i.e. for any incoming packet of any slice.

  - This also leads to UE has to be paged in any of the cells of RNA with most of the time a wasted paging.

  - For example, paging is wasted for incoming packet of slice 1 if UE is in cells 4,5 or 6.

  - Paging is wasted for incoming packet of slice 2 if UE is in cells 1,5 or 6 and for an incoming packet of slice 3 if UE is in cells 1 or 2.

  - In a nutshell, the saving of avoiding data notification/network-triggered PDU session setup/Paging is broken most of the time.

- Case 2: middle-size RNA: If gNB1 builds RNA = {cell 1, cell 2, cell 3}

  - AoI reporting to SMF looks as: AoI 1 -> IN, AoI 2 -> UNKNOWN, AoI 3 -> UNKNOWN

  - SMF knows where the UE is regarding AoI1 related to NS-AoS of slice 1 (i.e. UE is “IN”), so it can enable data notification and reactivate PDU session for an incoming packet of slice 1 with successful paging and reconnection.

  - But if instead an incoming packet comes for slice 2 or slice 3 then the data notification/paging is wasted if e.g. UE is located in cell 1 for an incoming packet of slice 2, or e.g. if UE located in cells 1,2 for an incoming packet of slice 3.

  - Here the saving of avoiding data notification/network-triggered PDU session setup/Paging works partially but not fully.

- Case 3: small size RNA: If gNB1 builds RNA = {cell 3}

  - AoI reporting to SMF looks as:

  - AoI 1 -> IN, AoI 2 -> IN, AoI 3 -> IN

  - SMF enables trustworthy data notifications. Network-triggered PDU session setup and paging requests are not wasted but successful because the SMF knows where the UE is, and successful connection.

  - However, the drawback with such small RNA is high number of RNA updates as RNA contains only one cell.

The above examples show that the building of the RNA by a gNB will strongly influence how SA2 desired feature of avoiding data notification/paging when UE in outside slice AoS using AOI reproting effectively works.

The building of the RNA is currently fully under the responsability of the gNB but it can receive Core Network Assistance Information to help finding the best trade-off in size of the RNA.

In the above examples, if the gNB is not made aware by the AMF, when configured by AMF for AOI reporting, that this AOI reporting is for the purpose of slice AoS monitoring to prevent useless data notifications/paging then the gNB will not take this into account in the RNA building e.g. could typically decide big RNA to avoid frequent RNA update but then always report UE location as “unknown” which fails SA2 feature.

When AOI reporting is configured for slice AoS monitoring then gNB trade-off is:

1)  high number of RNA updates if RNA is restricted to a low number of cells.

2)  high number of useless data notifications/paging requests if RNA contains a large number of cells.

In order to have gNB make the right decision on RNA size, we think that gNB should be made aware when the AOI reporting is intended for the purpose of slice AoS monitoring.

Proposal: It is proposed that SA2 agrees it is deemed beneficial to make the NG-RAN aware that the he AOI reporting is intended for the purpose of slice AoS monitoring so that the DL paging vs RAN update balance can be implemented accordingly.

# 3	Conclusion

The paper has shown why it is important when AMF configures the AOI reporting to gNB to tell the gNB when it is due to this slice AoS monitoring. This will help the gNB to make the best trade-off for the RNA size to avoid useless data notifications/network-triggered PDU session setup/Paging requests as much as possible.  It is proposed to send the LS in S2-2410419 to RAN3 indicating that from SA2 perspective it can be beneficial to provide this assistance to the gNB and then it is up to RAN3 to decide the right approach and if one is selected we can align in our specifications.