3GPP TSG-WG SA2 Meeting #165	S2-2410446

# Hyderabad, India, October 14 - 18, 2024	   									   (revision of S2-2409105)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1429 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS AS Service Enhancement to Support IMS Event Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-77, for the conclusion of KI#1, it is agreed to re-use the existing Nimsas_SessionEventControl Notify service operation to notify the NEF/AF who subscribed for the IMS session events: 7. For the purpose of optimization and following 5GC exposure principles, any Notification related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request using Nimsas_SessionEventControl Notify service operation. The related CRs were postponed at SA2#164. It should be noted that there is dependency between KI#1 and KI#2; the AF using the IMS AS service as described for KI#2 (Nimsas_ImsSessionManagement) must be able to get notified for the details of the IMS session, e.g. the Media Type and Media Parameters, see clause AA.2.4.4 in TS 23.228. This is already supported by the Nimsas_SessionEventControl Notify service operation, thus defining a new notification for similar information should be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to re-use the existing Nimsas_SessionEventControl Notify service operation as concluded in the TR. The Nimsas_SessionEventControl service is enhanced to support subscription of IMS session events: 1) The subscribe and unsubscribe service operations are added. 2) The subscribe request may include the "acknowledgement to be expected" parameter. The value "True" is used by the NF service consumer to indicate that the NF service consumer may provide media instructions to the IMS AS via Nimsas_MediaControl_MediaInstruction service. The value "False" means the IMS AS can proceed with the session event without waiting for the media instructions. Omitting the parameter equals to "True" value, this ensures backwards compatibility with Rel-18 DCSF. 3) The subscribe request may include one or more Event filters to limit the notifications only to the session events that match with the filters: - The Event filter "IMS public user identity" is used to limit the notifications to a particular served subscriber. - The Event filters "Calling ID" and "Called ID" can be used to limit the notifications to a particular calling or called party. - The Event filter "Media Type" can be used to limit the notifications to a particular media type (DC, Audio, Video). - The Event filter "Data Channel Mapping and Configuration Information" can be used to limit the Data Channel related notifications to a particular Stream ID and/or DC application ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscription of the IMS events to IMS AS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision of S2-2409105 (postponed) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## AA.2.4	IMS AS Services

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF, HSS, NEF, AF |
|  | Notify | Subscribe/Notify | DCSF, NEF, AF |
|  | Unsubscribe | Subscribe/Notify | DCSF, HSS, NEF, AF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

### AA.2.4.2	Nimsas_SessionEventControl

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

Depending on the local configuration in the IMS AS (for implicit subscriptions), or if the "acknowledgement to be expected" indicator is set to "true" in the subscribe request, the IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |

#### AA.2.4.2.x	Nimsas_SessionEventControl_Subscribe service operation

Service operation name: Nimsas_SessionEventControl_Subscribe

Description: This service operation enables NF consumers (e.g. DCSF, HSS) subscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: NF type, Event ID, Notification Target Address.

Event ID represents one or multiple events as defined in Table AA.2.4.2.2-1.

Inputs, Optional: Event filter(s), Acknoglegement to be expected.

One or more Event filters can be used to notify the NF service consumer only for the events that match with the given Event filters:

- 	IMS public user identity: the identity of the served subscriber.

-	Calling ID: identity of the calling party.

- 	Called ID: identity of the called party.

-	Media Type: DC, Audio, or Video.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

Acknowlegement to be expected defines how the IMS AS proceeds with the session after the notification is sent:

- 	True (default): the IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification

-	False:  the IMS AS does not wait for acknowledgement of the session even notification. The IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

Outputs, Required: Result indication, when the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

#### AA.2.4.2.y	Nimsas_SessionEventControl_Unsubscribe service operation

Service operation name: Nimsas_SessionEventControl_Unsubscribe

Description: This service operation enables NF consumers (e.g. DCSF, HSS) unsubscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: Subscription Correlation ID

Inputs, Optional: None

Outputs, Required: Result indication

Outputs, Optional: None.

END OF CHANGES