3GPP TSG SA WG2 Meeting #165                                                        	S2-2410457

Hyderabad, India, 14th Oct – 18th Oct, 2024

Source:	Samsung

Title:	Additional aspects for S&F feature.

Document for:	Discussion

Agenda item:	5GSAT_Ph3

# 1. Introduction

SA2 has agreed conclusions for KI#2 i.e. for Store and Forward Satellite operation in the TR 23.700-29 and currently SA2 is in a process to agree normative text based on the agreed conclusions. This paper highlights additional aspects which needs to be considered to complete this feature.

# 2. Discussion

## Aspect-1: UE in NO service due to list of satellite IDs.

Scenario:

a) UE receives SAT-1, SAT-2 as list of satellite IDs in a given area-x.

b) UE moves to the new area, where SAT-1, SAT-2 will not serve the UE.

c) The UE can see the signal from SAT-3, but UE will not attempt to connect on it assuming it’s a forbidden cell for the UE to initiate NAS signaling. This is not intended based on deployment.

Proposal-1:

Network can provide max timer to the UE, which starts at the UE the moment it see a cell which is not broadcasting allowed satellite ID, at expiry of which the UE attempt TAU even though the current cell is not broadcasting satellite ID part of list of Satellite IDs.

Proposal 2:

The list of satellite IDs is valid only within the TAI list provided to the UE. This implies TAI list includes both the TAIs where list of satellite IDs are allowed for the UE and TAIs where list of satellite IDs are not allowed for the UE.

If UE moves out of TAI list, the UE will trigger TAU procedure (following current procedure) and receive the fresh list of satellite IDs.

Samsung position: We think proposal-2 is required for accept case and proposal-1 for reject case (because in reject message TAI list is not provided to the UE).

## Aspect-2: Cell does not broadcast satellite ID

When list of satellite IDs are available in the UE, the UE should consider the cell which is not broadcasting any satellite ID as an allowed cell to send signalling or it should consider forbidden cell to send any signalling is not clear based on agreed conclusions.

Proposal:

When UE is having list of satellite IDs, and if the cell UE selects does not broadcast any satellite ID, the UE:

a) considers the cell as forbidden to initiate signalling or

b) considers the cell as allowed to initiate signalling.

Samsung view: One of the above options SA2 has to decide as a way forward. We think both options can work.

## Aspect-3: Ping pong between different PLMNs for S&F service.

When UE has received reject message from PLMN-1 with S&F information (i.e. list of satellite ID and S&F wait timer). From past discussions, it is clear that UE is allowed to select a TN network while it is waiting to re-attempt attach on PLMN-1 for S&F service. But if UE has already attempted on PLMN-1 for S&F service is it allowed to select PLMN-2 for S&F service again is not clear from the conclusions.

Scenario:

a)	UE attempts attach procedure on PLMN-1.

b)	PLMN-1 rejects and provides a list of satellite ID and wait timer to the UE due to S&F satellite operations.

c)	PLMN-1 satellite passes away and lets assume UE is in discontinuous coverage for PLMN-1. Till MME fetches the UE context from ground.

d)	Meanwhile UE sees the PLMN-2 network(again support S&F service) and attempts attach procedure.

e)	PLMN-2 rejects and provides list of satellite ID and wait timer to the UE due to S&F satellite operations.

f)	At this point, we see below aspects needs a discussion:

1)	What UE should do with the list of satellite IDs and wait timer received by the UE in step 2 from PLMN-1 when it has received same parameters from PLMN-2.

2)	PLMN-1 network is also in the process of getting UE context and PLMN-2 network is also in the process of getting the UE context, which may take hours. Is this justified ?

3)	Should UE run the S&F wait timer for both PLMNs i.e. per PLMN ID.

Proposal-1:

The UE avoids this scenarios, by barring itself to select PLMN-2(for S&F service) when it is waiting to re-attempt on PLMN-1 for S&F service. Please note UE is allowed to select PLMN-2 for normal services (it can be either from TN network or NTN network)

Proposal-2:

The UE maintains list of satellite IDs and S&F wait timer per PLMN ID and manages them separately. The UE is allowed to attempt attach procedure on different PLMNs for S&F service.

Samsung view: We have slight preference for Proposal-1 because it simplifies UE implementations without any major impacts to the UE service.

# 3. Conclusion & proposal

We propose to discuss above additional aspects to complete S&F feature and incorporate the agreeable principles as part of Rel-19 work. The potential wording to capture above principles is proposed in S2-2410460 with green highlighted text.

# 4. Reference

NA.