3GPP TSG-WG SA2 Meeting #165	S2-2410474

# Hyderabad, India, October 14 - 18, 2024	 	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0152 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE privacy checking for SA3 alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 has developed Procedure of UE privacy verification for UE-only operation in clause 6.3.7 of TS 33.533, in which UE2/…/UEn needs to check whether exposure to SL Positioning Client UE and exposure to SL Positioning Server UE are allowed. This step happens after SL Positioning Server UE is selected by the target UE. Furthermore, it is also defined in clause 6.3.7 of TS 33.533: “The SL Positioning Server UE may be reselected based on the privacy check results of the Target/Reference UEs.” This should be reflected in UE only procedures, and impact SL Positioning Server UE selection. Moreover, SA3 has agreed to not define Located UE related info in the privacy profile; hence, it is not necessary to perform privacy checking for any Located UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a creteria for SL Positioning Server UE re-discovery & re-selection Update description on privacy checking in Ranging/Sidelink Positioning control procedure and Procedures for Ranging/SL Positioning service exposure through PC5 Remove the condition of Located UE privacy checking in the steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear on privacy checking in Ranging/Sidelink Positioning control procedure and SL Positioning Server UE re-discovery & re-selection |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3, 5.6.2.2, 5.11, 6.7.1.1, 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.3	SL Positioning Server UE Discovery & Selection

SL Positioning Server UE (re-)Discovery & (re-)Selection is performed by the Target UE, when it meets one or more of the following criteria:

-	The Target UE and the discovered SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning (e.g. because they are out-of-coverage or the serving network does not support Ranging/SL Positioning). SL Reference UE and Target UE may indicate whether its serving network supports Ranging/SL Positioning or not to each other. The indication of serving network not supporting Ranging/SL Positioning is also used to notify each other when UE has no NAS connection available.

NOTE:	If the serving network supports Ranging/SL Positioning, it can provide Ranging/SL Positioning Support indicator to UE as described in clause 6.3.2.

-	The Target UE is not capable of performing SL Positioning Server UE functionalities or related positioning methods.

-	The SL Reference UEs are not capable of performing SL Positioning Server UE functionalities or related positioning methods.

-	UE privacy check against the SL Positioning Server UE selected by the Target UE fails as defined in clause 6.3.7 of TS 33.533 [13].

The discovery of SL Positioning Server UE follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

The Target UE shall discover and select a SL Positioning Server UEs that are in the same or different serving PLMN of the Target UE and the SL Reference UE(s).

# * * * * Next change * * * *

5.6.2.2	Ranging/SL Positioning service exposure through PC5

For Ranging/SL Positioning service exposure to a UE through PC5 in the following way:

-	SL Positioning Client UE discovers one of the two or more UEs described in clause 5.6.1, e.g. the SL Reference UE(s) or Target UE(s), using the Ranging/SL Positioning UE discovery procedure (refer to clause 6.4);

-	SL Positioning Client UE invokes the Ranging/SL Positioning service request to one of the discovered UE which will work as the Target UE, with the attributes described in clause 5.6.1. For absolute location, this service request includes the SL Positioning Client UE's user info and Target UE's user info, and may also include the user info for a list of candidate Located UE(s). For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, and SL Reference UE's user info. The Ranging/SL Positioning service request/response is conveyed by PC5-U and handled in the Ranging/SL Positioning layer. The Ranging/SL positioning service request/response is handled as application layer traffic by V2X layer or 5G ProSe layer.

-	The UE which received a SL positioning service request from SL Positioning Client UE triggers the authorization of SL Positioning Client UE for the Ranging/SL Positioning exposure service invocation during PC5 link establishment as specified in TS 33.533 [13], performs the Ranging/SL positioning control between SL Reference UE and Target UE (as defined in clause 5.3), and returns the Ranging/SL positioning result to SL Positioning Client UE.

-	UE privacy is based on the local configured privacy verification information to determine whether or not its location related information can be exposed to the peer UE or the SL Positioning Client UE or the SL Positioning Server UE. If the privacy profile allows location exposure, the UE accepts the request to expose its location related information and proceeds.

5.11	UE-only Operation for SL Positioning using Located UE

When LMF is not involved for SL Positioning/Ranging, e.g. the LMF in the serving network does not support SL Positioning, UE-only Operation SL Positioning is used, including Target UE as SL Positioning Server UE and Target UE not as SL Positioning Server UE.

When the Target UE acts as a SL Positioning Server UE, the following principles applies:

-	The Target UE performs the Located UE's discovery and selection when absolute location is requested.

-	For absolute location, the Target UE obtains the location of the Located UE(s), and the Located UE(s) may trigger a 5GC-MO-LR to retrieve its location. The Target UE uses the location of Located UE(s) together with the Ranging/SL positioning measurement data or result to estimate its own location.

-	The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

When the Target UE does not act as SL Positioning Server UE, the following principles applies:

-	The Target UE performs the SL Positioning Server UE's discovery and selection.

-	The Target UE requests its absolute location from the SL Positioning Server UE.

-	The SL Positioning Server UE interacts with the Target UE to exchange UE capabilities, assistance data, Ranging/SL positioning measurement data, location results and location of Located UE(s).

-	The SL Positioning Server UE can optionally determine to use the location of Located UE(s) together with the Ranging/SL positioning measurement data or result to estimate the absolute location of Target UE.

-	The Target UE performs the Located UE's discovery and selection, and obtains the location of the Located UE(s).

-	The SL Positioning Server UE returns the Ranging/SL positioning result to the Target UE.

-	The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

When UE-only operation is triggered for the service request from a SL Positioning Client UE, the SL positioning client UE should send the service request to a discovered Target UE or SL Reference UE, the UE receiving the service request will work as the Target UE.

# * * * * Next change * * * *

6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1: Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.4 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or one or more SL Reference UE(s)/Located UE(s). When Model B discovery used, in the Solicitation message, target UE's user info is included and role of target UE and role of SL Reference UE/Located UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1's user info and UE2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE(s)/Located UE(s) is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

The SL Positioning Client UE may receive a supplementary RSPP signalling message including list of SL Reference UE(s)/Located UE(s) from target UE over the PC5 connection.

3.	The SL Positioning Client UE may select UE(s) to perform SL Positioning operation and select a UE (here UE1) to receive SL Positioning Service Request. The SL Positioning Client UE sends a SL Positioning Service Request using supplementary RSPP signalling message.

For absolute location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, and required positioning QoS. The Service Request may include info of list of candidate Located UE(s).

For relative location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, info of SL Reference UE(s), and required positioning QoS.

For ranging, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of UE1, info of UE2, and required positioning QoS.

For ranging, if UE1 and UE2 are not in proximity, the SL Positioning Service Request is responded with reject by the receiving UE (either UE1 or UE2) and remaining steps are not performed.

4.	The UE which received SL Positioning Service Request (here UE1) selects LMF or SL Positioning Server UE based on its status whether UE has NAS connection:

-	If SL Positioning Server UE is selected (the Server UE can be the Target UE itself or a UE different from the Target UE), Ranging/SL Positioning operation is performed as described in clause 6.8;

-	If LMF is selected, UE1 first checks with UE2/…/UEn about whether exposure to SL Positioning Client UE is allowed based on the procedure defined in clause 6.3.6.3 of TS 33.533 [13]. If the privacy check is successful, the Ranging/SL Positioning operation is then performed as described in clause 6.20.1 of TS 23.273 [8].

5.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * Next change * * * *

6.8	Procedures of Ranging/Sidelink Positioning control

Either UE-only Operation or Network-based Operation is applied in the Ranging/Sidelink Positioning control procedures.

UE-only Operation as specified in this clause is applied for the following cases:

-	Neither Target UE nor SL Reference UE is served by NG-RAN.

-	Network-based Operation is not supported by the 5GC network as described in clause 5.2.3.

-	Response to SL-MO-LR request allows the UE only operation for a period of time by the network due to congestion.

For any other cases, Network-based Operation as specified in clauses 6.20 of TS 23.273 [8] is applied.

Figure 6.8.1-1: Procedures for Ranging/Sidelink Positioning control (UE-only operation)

1.	UE1 (i.e. Target UE) may receive a Ranging/SL Positioning Service request from:

1a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1.

For absolute location, the service request includes the SL Positioning Client UE's user info and Target UE's user info, and required positioning QoS and may also include the user info for a list of candidate Located UE(s).

For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, SL Reference UE's user info (UE2/.../UEn), and Ranging/SL Positioning QoS information.

1b.	RSPP application layer.

The service request includes type of the result (i.e. absolute location, relative location or ranging information) and the required QoS.

2.	If exposure to SL Positioning Client UE is allowed by UE1, it discovers UE2/.../UEn (i.e. SL Reference UEs/Located UEs and optionally whether they support SL Positioning Server UE role) as defined in clause 6.4, if needed.

NOTE 1:	Details of security related procedures during UE discovery are defined in clause 6.2 of TS 33.533 [13].

3.	If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, UE-only Operation is applied.

4.	UE1 and UE2/.../UEn perform capability exchange using SLPP messages. Step 4 may be performed during step 7 with coordination of SL Positioning Server UE.

Before the step4, UE1 establishes the secure PC5 link with UE2/.../UEn.

In addition, if UE1 acts as SL Positioning Server, UE1 may also request for the absolute location of UE2/.../UEn from UE2/.../UEn using the supplementary RSPP signalling message, and if the absolute location is available and allowed to be shared (i.e. after privacy verification according to clause 6.3.7 of TS 33.533 [13]), the absolute location of UE2/.../UEn is transferred by UE2/.../UEn to UE1 by the supplementary RSPP signalling message; and

UE1 selects the Located UEs (e.g. UEx/…/UEy) from UE2/.../UEn for the subsequent SL positioning/ranging operation (e.g., based on information received during discovery, Ranging/SL Positioning capability of UE1/UE2/.../UEn, the absolute location of UE2/.../UEn).

5.	If UE1 does not support SL Positioning Server functionalities or UE1 opts to select a SL Positioning Server UE different from UE1, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step 2) and selected. If a SL Positioning Server UE is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers and selects the SL Positioning Server UE as described in clauses 5.2.3 and 6.4. UE1 establishes the secure PC5 link with the selected SL Positioning Server UE.

NOTE 2:	UE1 may be able to perform the selection of the SL Positioning Server UE based on information obtained in step 2.

If the Located UE is served by NG-RAN, it may use 5GC-MO-LR procedure to retrieve its absolute location.

NOTE 3:	After step 5a and before step 5b, UE1 checks with UE2/…/UEn about whether exposure to SL Positioning Client UE and exposure to SL Positioning Server UE, if selected, are allowed based on the procedure defined in clause 6.3.7 of TS 33.533 [13]. The UEs from UE2/.../UEn may be down-selected based on the privacy check result. If UE privacy check against the SL Positioning Server UE fails, step 5a may be repeated to reselect a SL Positioning Server UE.

NOTE 4:	Steps 5a, 5b, 6, 7, 8a, 9, 11, 13 are performed only when SL Positioning Server UE is different from UE1.

6.	If a SL Positioning server UE is selected, UE 1 sends a Ranging/SL positioning request using supplementary RSPP signalling message to the selected SL Positioning Server UE. This request indicates the other UEs 2 to n using the Application layer ID and indicates the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated.

7.	The SL Positioning Server UE sends requests to UE1 for capability of UE1 using the SLPP message and for the capabilities of UE2/.../UEn using the supplementary RSPP signalling (e.g. including SLPP containers that may contain Sidelink Positioning capability request for UE2/…/UEn) message with the corresponding Application Layer ID of UE2/.../UEn. UE1 responds to the SL Positioning Server UE with its own capability using SLPP message and the capabilities of UE2 to n using the supplementary RSPP signalling message (e.g. including SLPP containers that may contain Sidelink Positioning capability of UE2/…/UEn) with the corresponding Application Layer ID of UE2/.../UEn. If step 4 did not occur, UE1 retrieves capabilities from UE2/…/UEn using SLPP messages during this step.

The SL Positioning Server UE may downselect the UEs (e.g. UEx/…/UEy) from UE2/.../UEn for the subsequent SL positioning/ranging operation (e.g., based on UE capability).

8.	The SL Positioning Server UE provides the Sidelink Positioning assistance data to UE1.

8a.	For the Sidelink Positioning assistance data used by UE1, it is transmitted by SLPP message.

8b.	For the Sidelink Positioning assistance data used by UEx/…/UEy, it is transmitted using the supplementary RSPP signalling messages (e.g. including SLPP containers that may contain Sidelink Positioning assistance data for UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy and then UE1 sends the assistance data to each UE (UEx/…/UEy) by SLPP messages.

9.	The SL Positioning Server UE sends requests to UE1 for SL measurement information of UE 1 and UEx/…/UEy, if the SL Positioning Server UE performs the result calculation. For the SL measurement information of UE1, the request uses the SLPP message. For the SL measurement information of UEx/…/UEy, the request uses the supplementary RSPP signalling message (e.g. including SLPP container that may contain Sidelink Positioning location measurements request for UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy. In addition, the SL Positioning Server UE may also request for the absolute location of UEx/…/UEy via UE1 using the supplementary RSPP signalling message with the corresponding Application Layer ID of UEx/…/UEy. If the absolute location of UEx/…/UEy is available, then the absolute location of UEx/…/UEy is returned using a supplementary RSPP signalling message.

10.	If a SL Positioning Server UE different from UE1 is selected in step 5, SL-PRS measurement is performed between UE1 and UEx/…/UEy and possibly also amongst UEx/…/UEy. The UE1 requests for the SL measurement information from UEx/…/UEy by the SLPP messages and/or the absolute locations of UEx/…/UEy from UEx/…/UEy by supplementary RSPP signalling messages if requested in step9 by SL Positioning Server UE. The SL-PRS measurement data is transferred to UE1 using SLPP messages.

11.	SL-PRS measurement data of UE1 is transferred by SLPP message to the SL Positioning Server UE and SL-PRS measurement data of UEx/…/UEy is transferred by UE1 using the supplementary RSPP signalling message (e.g. including SLPP container that may contain SL-PRS measurement data of UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy to the SL Positioning Server UE if requested in step 9 in order to perform result calculation. The absolute location of UEx/…/UEy as received in step 9 is transferred by UE1 to the SL Positioning Server UE by the supplementary RSPP signalling message with the corresponding Application Layer ID of UEx/…/UEy if requested in step9.

12.	Based on the result types requested in step 6, absolute location, relative location or ranging information is calculated at the SL Positioning Server UE.

NOTE 5:	Details of step 4-12 are developed by RAN WGs.

NOTE 6:	The supplementary RSPP signalling message mentioned above is conveyed by PC5-U and handled in the Ranging/SL Positioning layer, whose detailed design is left to stage 3.

13.	The SL Positioning Server UE selected in step 5 sends a Ranging/SL positioning response using supplementary RSPP signalling message to UE1 including the result that is required in step 6.

14.	Ranging/SL Positioning result is transferred to:

14a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1;

14b.	Application layer.

# * * * * End of changes * * * *