3GPP TSG-WG SA2 Meeting #165	S2-2410489

14 - 18 October, 2024, Hyderabad, India

# Title:	[DRAFT] Reply LS on Configuration of Slice Usage Control Information

# Response to:	LS on Configuration of Slice Usage Control Information (S2-2409589/C4-243492)

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Ashok Kumar Nayak

E-mail Address:	ashok.nayak@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT4 regarding the LS on Configuration of Slice Usage Control Information. SA2 provides following answers to CT4 questions.

To support network slice usage control, CT4 defined the configuration of slice deregistration inactivity timer and PDU Session inactivity timer in UE subscription data in UDM. Such slice related timer is access type independent, see clauses 6.1.6.2.8 and 6.1.6.2.38 of 3GPP TS 29.503.

CT4 also observed that clause 5.15.15.2 and 5.15.15.3 of 3GPP TS 23.501 specifies that the AMF starts the slice deregistration inactivity timer per S-NSSAI per access type. Furthermore, in 3GPP TS 23.501 clause 5.15.15.3 there is an NOTE saying that for MA-PDU Session the PDU Session inactivity timer is access type independent, while it is not clear for SA-PDU Session whether the PDU Session inactivity timer is access type independent or not.

CT4 discussed how slice usage control information configured in the UE subscription is used by the AMF to start per S-NSSAI per access type slice deregistration inactivity timer, and by SMF to start PDU Session inactivity timer and would like to ask SA2 to answer the following questions:

Q1: For SA-PDU Session, is the PDU Session inactivity timer independent of access type?

Answer: PDU Session inactivity timer is access type independent, be it SA-PDU Session or MA-PDU Session.

Q2: To support per S-NSSAI per access type slice usage control defined in 3GPP TS 23.501, does it need to configure different timer values to different access type in UDM?

Answer: No. The timer value is common for both the access type.

Q3: If the answer of Q2 is no, what is the exact AMF/SMF behaviour of performing per S-NSSAI per access type network slice usage control (i.e. starting slice deregistration inactivity timer or PDU Session inactivity timer)? Does the AMF or SMF assign same timer value to 3GPP access and non-3GPP access?

Answer: The timer value is same for both the access type.

In case of slice deregistration inactivity timer, AMF applies the value and starts the timer in one access type when that particular S-NSSAI is added to the Allowed NSSAI of that corresponding access type. During that time there is no impact to the other access type which means until unless the UE request same S-NSSAI and AMF adds the S-NSSAI to the Allowed NSSAI of the other access type, the timer will not be started. Hence slice deregistration inactivity timer is per access type.

In case of PDU Session inactivity timer, SMF provides the timer value to UPF whenever the first access type of MA-PDU session is established. SMF does not give the timer value again to UPF when the second access type is added to the MA-PDU session. UPF will trigger inactivity only when it detects inactivity for both the access type. Hence the NOTE in SA2 specification clarifies that PDU Session inactivity timer is access type independent.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#166	October 18 – 22, 2024	Orlando, USA

TSG-SA2 Meeting#166 Ad Hoc-e      January 20 – 24, 2025	Online