3GPP SA WG2 Meeting #165	S2-2410506

# Hyderabad, India, October 14-18 2024	        										    (was S2-2408532)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5602 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of emergency service over Mobile Wireless Access Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This contribution added the support for emergency services over the MWAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add emergency service support over MWAB in new clause 5.49.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of VMR_Ph2 feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 5.49.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.x.8	Support of Emergency Services

MWAB may support regulatory requirements (e.g. emergency services, priority services) for UEs access 5GS via a MWAB-gNB, based on OAM configuration from the MWAB Broadcasted PLMN/SNPN.  LCS framework as defined in TS 23.273 [87] is used for providing the location service to the served UEs, with additional enhancements described in clause 5.49.7.

The MWAB-UE may be configured by the HPLMN with a list of dedicated Backhaul PDU session parameters (with specific S-NSSAI and DNN), for the support of emergency sessions of the UEs.  The HPLMN shall include the specific S-NSSAI in the MWAB-UE Subscribed S-NSSAIs.

NOTE 1:	The "specific S-NSSAI and DNN" is operator configured and set as part of the roaming agreement (in case of MWAB-UE roaming). The DNN can be assigned by the serving AMF from a default DNN in subscription for the S-NSSAI.

The AMFs in the Backhaul PLMN/SNPN shall not deregister a MWAB-UE that is registered with the specific S-NSSAI for emergency session backhauling. The SMFs in the Backhaul PLMN/SNPN shall not initiate the release of a PDU session associated with the specific S-NSSAI, unless it remains inactive for an operator configured extended period.

If the UE establishes the connection with MWAB-gNB for regulatory services, e.g. using regulatory service related Establishment Cause as defined in TS 38.331 [51], the MWAB-gNB shall inform the MWAB-UE, so that the UE's N2 and N3 connections are associated with the suitable Backhaul PDU sessions. If the MWAB-UE is configured with dedicated Backhaul PDU session parameters, its Local Configuration or URSP shall be configured to use the dedicated Backhaul PDU session parameters for such N2/N3 connections.

When the MWAB-gNB determines that a normal registered UE has established an emergency session (based on the dedicated ARP value), the MWAB-gNB may request the MWAB-UE to adjust the Backhaul PDU session association. The MWAB-gNB shall only trigger Backhaul PDU Session association change if the emergency session would not be interrupted, e.g. due to the N2 and N3 connection re-establishment via the new Backhaul PDU Session(s).

MWAB-gNB will handover the UEs to other cells before stopping operating as MWAB, e.g. instructed by OAM or with the Backhaul PDU Session deauthorized, as described in clause 5.49.3.

If allowed by the Backhaul PLMN, the MWAB-UE may maintain the Backhaul PDU Session(s) when itself initiates an emergency service session. In that case, the MWAB-gNB can continue to operate.

NOTE 2:	In case of MWAB-gNB roaming, the routing of the emergency service to a local PSAP will be ensured by the OAM system of the MWAB Broadcasted PLMN, e.g. selecting a local UPF.

The MWAB-UE only relases the Backhaul PDU session with the specific S-NSSAI when triggered by the MWAB-gNB or a configured inactive period (as specified in 5.49.2).

End of CHANGEs