3GPP TSG-WG SA2 Meeting #165	S2-2410509

# Hyderabad, India, 14 – 18 October, 2024										 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5799 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Subscription and policy control for network energy saving as service criteria |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusions for KI#2 in TR 23.700-66, the feature for subscription and policy control to enable network energy savings needs to be supported. It is concluded tha the 5CG NF can request energy consumption calculation per following granularities: per application, per UE, per-UE-per-QoS flow, per PDU session. The following proposals are derived: - The PCF is in charge of the policy control based on subscription information. - The PCF can determine AM policies which apply for the whole UE (e.g. based on the “per UE” granularity of EC measurements) - The PCF can determine SM policies which apply for a PDU Session (e.g. based on the “per application”, “per-UE-per-QoS flow”, “per PDU session” granularity of EC measurements). - The subscription information related to energy saving is stored in: - Access and Mobility ManagemetnSubscription data - Session Management Subscription data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add description for the subscription and policy control to enable network energy savings as per KI#2. It is proposed to introduce 2 new clauses: - One clause describes the subscription and policy control to support network energy saving for Access and Mobility management procedures; and - Another clause describes the subscription and policy control to support network energy saving for Session management procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KI#2 feature on the energy saving and energy efficiency WI is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses 5.X.Y and 5.X.Z. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.X	Support for Energy Saving

### 5.X.0	General

### 5.X.X	KI#1: Energy Consumption information calculation and exposure

### 5.X.Y	Subscription and policy control to support network energy saving for Access and Mobility management procedures

The 5GS supports an Access and Mobility management policy-controlled network energy efficiency and energy saving as service criteria based on a subscription information. An AM Energy Saving Information (AM-ESI) is introduced as part of the Access and Mobility Subscription data of the UE. The AM-ESI includes the following information:

-	indication that the UE is subject to energy saving when a certain criterion is met, e.g. due to energy consumption for a particular granularity exceeds a maximum energy limit. The enabling of the energy saving for the UE may results to service restrictions.

-	a maximum energy consumption limit which applies for the UE.

Upon retrieving the UE Subscription data, if the ESI indicates that the energy saving is enabled for the UE, the AMF provides AM-ESI in the AM policy association establishment or modification request to the PCF. The PCF subscribes with the Energy Collection NF for notifications whether the maximum energy limit is reached for the UE.

When the PCF is notified that the maximum energy limit is reached for the UE, the PCF determines to apply the energy saving for the UE by updating the AM policy control information which restricts the UE AM service parameters, e.g. including an energy saving indication, a new RFSP information or reduced QoS parameters (e.g. UE AMBR) to be applied to the UE. Optionally, applicability conditions may be included which indicate the conditions under which reduced QoS paramters applies. For example, the restrictions may apply for 1) non-GBR services; 2) to any type of non-mission critical services; 3) to all services; 4) for a time period or 5) the service area (e.g. a list of cells or TAIs).

NOTE 1:	The reduced QoS level may include reduced UE-AMBR or Slice-MBR which would contribute to the energy saving because less transmitted data means less energy consumption. According to the stage 1 service requirements, restricting the data rate due to energy restrictions can apply to background data or other services, i.e. any non-GBR data, if covered in the SLA between the customer and the network operator.

The AMF stores in the UE context the AM policy control information including the restricted UE AM service parameters, and if applicable, the applicability conditions. The AMF provides, as part on the UE context sent via N2, to the NG-RAN the AM policy control information. The NG-RAN applies the restricted UE AM service parameters.

### 5.X.Z	Subscription and policy control to support network energy saving for Session management procedures

The 5GS supports a Session Management policy-controlled network energy efficiency and energy saving as service criteria based on a subscription information. A SM Energy Saving Information (SM-ESI) is introduced as part of the Session Management Subscription data of the UE. The SM-ESI includes the following information:

-	a maximum energy consumption limit which applies for a DNN/S-NSSAI combination (i.e. for a PDU Session).

-	a maximum energy consumption limit per application which result to limitation for the granularity per-UE-per-QoS flow.

NOTE: A saparate QoS flow for the application traffic needs to be stablished, if the application is subject to energy consumption monitoring and policy control.

During the establishment of PDU Session, the SMF retrieves the Session Management Subscription data from the UDM, which may include SM-ESI. The SMF provides SM-ESI in the SM policy association establishment or modification request to the PCF.

When the SM-ESI is included, the PCF subscribes with the Energy Collection NF to obtain notifications when the energy consumption for a certain granularity (per application, per PDU Sesssion, per QoS flow) has reached the maximum energy consumption limit for the certain granularity.

When the PCF is notified that the maximum energy limit is reached for the UE, the PCF determines new PCC rule(s) for the PDU Session (e.g. that the service restriction due to maximum energy consumption for a certain granularity is reached). The PCF sends the new PCC rule(s) to the SMF.

The SMF determines QoS parameters for the PDU Session (e.g. reduced Session AMBR) and provides the QoS parameters to the NG-RAN and/ot to the UPF.

# * * * * End of changes * * * *