3GPP TSG|WG-SA2 Meeting #165	S2-2410513

Hyderabad, India, October 14 – 18, 2024	(revision of S2-20xxxx)

Source:	Samsung, Huawei, NEC

Title:	Discussion on solution for enforcing maximum number of device identifier

Agenda Item:	19.8.2

Document for:	Information

# 1.	Background

In the KI#4, there was a requirement how an operator can enforce the maximum number of simultaneous user identifiers per UE/5G-RG. SA2 studied and documented two solutions (Solution 32 and Solution 33).

Solution 32 (UE/5G-RG does enforcement): This solution was allowing operator to configure the quota for maximum number of simultaneous user identifiers to the subscription. During registration procedure, AMF after receiving this quota from UDM, provides this maximum number of simultaneous user identifier to UE/5G-RG. Whenever UE/5G-RG identifies the quota is reached, UE/5G-RG reject further connection with device identifier or proceeds with default URSP rule.

Solution 33 (Network/AMF does enforcement): This solution was allowing operator to configure the quota for maximum number of simultaneous user identifiers to the subscription in UDM. During PDU Session establishment or modification procedure, AMF after receiving the device identifier will increase the count and after identifying until the quota is reached, AMF rejects further connection with device identifier.

But SA2 could  not conclude the exact solution to be followed and hence it was agreed that the solution will be decided during normative phase.

Observation: When there is capability at UE/5G-RG available to enforce the maximum number of simultaneous user identifier, then solution 32 can work independently. However, it is not possible to expect the devices to have this capability. Hence, some sort of network enforcement mechanism is needed. Therefore, both solution 32 and solution 33 can co-exist together. In SA2#164 meeting, S2-2408060 was submitted along in the line of observation where both UE and AMF was enforcing the quota.

Another solution S2-2408012 was proposed where the provisioning of quota for maximum number of simultaneous user identifier is done to UDR and then PCF enforce it by retrieving the quota from UDR.

# 2.	Discussion

Based on the description in background section, it is required to address whether AMF or PCF based approach need to be adopted.

As a single device may be getting multiple services from network, (e.g. IMS service through PDU1 and Internet service through PDU2), it needs to be counted by network as 1. Hence irrespective of approach, the NF which does enforcement need to have the quota and knowledge about the current simultaneous number of device identifiers in the network.

| Option 1: (UDM and AMF based approach) (S2-2408060) | Option 2: (UDR and PCF based approach) (S-2408012) |
| --- | --- |
| 1) AF provisioning the max number to UDM 2) UDM provides this max number to AMF 3) AMF provides to UE 4) Both UE and AMF does the enforcement | 1) AF provisioning the max number to UDR 2) UDR provides this max number to SM-PCF 3) SM-PCF does the enforcement |

AMF (Option 1):

- Advantage: 1) AMF being the entry point for the UE/5G-RG to process the PDU session procedure, it will be aware and can count as 1 for a single device identifier irrespective of the number of PDU session, the same device identifier is part of. Once quota is exceeded, then AMF can easily reject it without affecting any other NFs of 5GC. 2) Also AMF provides this quota both during registration procedure and UCU procedure and based on this, UE/5G-RG itself can do the enforcement

- Disadvantage: 1) AF needs to provision the QoS differentiation policy to UDR and again provision the max number of simultaneous device identifier to UDM, the number of attempts and service operation will be twice.2) UE/5G-RG needs to provide the device identifier in NAS-MM message to AMF.

PCF (Option 2):

- Advantage: 1) AF can provision both the QoS differentiation policy and quota for max number of simultaneous identifiers at the same time to UDR. 2) SMF is already receiving the device identifier from UE/5G-RG in the NAS SM to provide the traffic differentiation, so not additional information is needed.

- Disadvantage: 1) As SM-PCF need to have the knowledge of current number of simultaneous device identifiers to do the enforcement, same SM-PCF needs to be invoked for all the PDU session triggered by UE/5G-RG for the connected non-3GPP devices. As the devices can get all kind of services like IMS, Internet, Gaming, Streaming etc, it is not practically possible for operator to deploy a PCF, which supports all kind of DNNs and S-NSSAI. Hence, this solution is practically not possible. 2) Also in this solution if PCF has to reject it, then it is not efficient in comparison to AMF as additional signaling from AMF to SMF and SMF to PCF is involved. 3) Also if any updates happens to the max number of simultaneous device identifier, network triggered PDU session modification flow is needed to inform the UE/5G-RG which again is not efficient in comparision to UCU flow.

Proposal: By looking at the advantages and disadvantages of both the solution, it is proposed to consider AMF based approach as it adds less complexity to the system in comparison to PCF based approach.

# 3.	Proposal

To ensure a simple solution, which addresses this optional requirement without adding any complexity to the system, it proposes to select one option and its corresponding action as a way forward from above two options in SA2 and Option 1 is preferred.