3GPP SA WG2 Meeting #165	S2-2410522

# Hyderabad, India, 14-18 October 2024									                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5800 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Delivery of non-3GPP Device ID information in NAS SM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions on KI#4 agrees following principles: -For non-3GPP devices requiring QoS differentiation, UE/5G-RG may send the Device Identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message, and the SMF will forward the Device Identifier and user plane address to the PCF. The PCF will take it into account for policy decisions based on the Device Identifier information retrieved from the UDR. Device Identifier is used as a data key to retrieve the information from the UDR. - When using an IP PDU Session Type, the user plane address is the UE IP Address(es) / Port Number(s) that carry the non-3GPP device's traffic. When using an Ethernet PDU Session Type, the user plane address is the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device's traffic. The CR implements the above principles via specification of non-3GPP Device Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification of Device Information signaled by UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS differentiation for non-3GPP devices is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x, 5.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES (All new)<<<<

## 5.x	QoS differentiation for non-3GPP devices

### 5.x.y	Non-3GPP Device Information

The Non-3GPP Device Information is signaled by UE when it wants to request QoS differentiation for the traffic of a non-3GPP device connected to it. When a non-3GPP device is connected to UE, the UE may contain the non-3GPP Device Information in PDU Session Establishement/Modification Request to SMF. The SMF forwards the Non-3GPP Device Information to PCF for policy control.

For IP PDU Session the Non-3GPP Device Information provided by UE consists of:

-	Non-3GPP Device Identifier is a generic string, and its allocation is out of scope of 3GPP.

-	IP address and port number associated with the device traffic.

NOTE:	This is the IP address and port number visible to the UPF, i.e., in case the UE acts as a NAT function for traffic from non-3GPP devices behind the UE, this refers to the IP address and port number assigned by the NAT function.

For Ethernet PDU Session the Non-3GPP Device Information provided by UE consists of:

-	Non-3GPP Device Identifier is a generic string, and its allocation is out of scope of 3GPP.

-	MAC address of the device.

- 	VLAN tag ID that is associated with the non-3GPP device's traffic.

# >>>>END OF CHANGES<<<<