3GPP TSG-SA WG2#165	S2-2410532

Hyderabad, India, 14-18 October 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Updating architectural assumptions based on RAN1 feedback on non-volatile memory of Ambient IoT devices

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to update the architectural assumptions based on RAN1 feedback on non-volatile memory of Ambient IoT devices.

# 1.	Discussion

In the Reply LS on Clarification of requirements for Ambient IoT [1], RAN1 provided the following feedback on the ability of an Ambient IoT device to update its non-volatile memory after receiving a trigger from the Reader:

- An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times. - The power consumption during writing to the NVM is higher than during reading. - RAN1 thinks that frequent or recurring writing to NVM should be avoided. - RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

As can be seen from the input from RAN1, it may not always be possible for an Ambient IoT device to write (update) its non-volatile memory (NMV). RAN1 also recommends avoiding frequent or recurring writing to the Ambient IoT device's NVM.

This is important since it implies that we cannot always rely on the Ambient IoT device to be able to store protocol-related state information (e.g., counters, identifiers) when the AIoT IoT device is not being energized.

Observation: An Ambient IoT device cannot always be assumed to be able to store dynamic protocol-related state information (e.g., counters, identifiers).

Proposal: Document this observation as an architectural assumption in TR 23.700-13.

# 2.	References

[1]	RAN1: Reply LS on Clarification of requirements for Ambient IoT. R1-2407364.

# 3.	Text proposal

It is proposed to agree the following changes to TR 23.700-13.

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP RP-234058: "New SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[7]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[8]	3GPP TR 38.769: "Study on solutions for Ambient IoT (Internet of Things) in NR".

[9]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[10]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[11]	GS1 Organisation: "The Global Language of Business". Available at: https://www.gs1.org/.

[12]	GS1: "Standards in the Healthcare Supply Chain". Available at: https://www.gs1.ch/de/media/1117.

[13]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[14]	3GPP TS 38.401: "NG-RAN; Architecture description".

[15]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[16]	BS ISO-IEC 15963-1-2020: Information technology - Radio frequency identification for item management - Part 1: Unique identification for RF tags numbering systems.

[X]	3GPP RAN WG1: Reply LS on Clarification of requirements for Ambient IoT. R1-2407364.

# >>>>END OF CHANGES<<<<

## 4.1	Architectural Assumptions

-	The following traffic types for Ambient IoT Device are to be studied:

-	DT: Device-terminated; and

-	DO-DTT: Device-originated - device-terminated triggered.

NOTE 1:	The DO-DTT additionally includes traffic from AIoT Devices, which is triggered by RAN/UE as reader, without CN sending traffic towards the AIoT Devices.

NOTE 2:	The final decision for including DO-A (Device-originated - autonomous) in the study depends on RAN decision.

-	The following two connectivity topologies as defined in TR 38.848 [7] are to be studied:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

-	The communication spectrum is assumed to be licensed.

-	Handover is not supported.

-	RRC states are not supported by AIoT Devices (see TR 38.769 [8])

-	No mobility (i.e. at least no cell selection/re-selection-like function) supported by AIoT Devices (see TR 38.769 [8])

Editor's note:	The meaning of no mobility is to be clarified by RAN in TR 38.769 [8].

NOTE 3: 	Coordination with RAN is required to determine the Ambient IoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies etc.).

NOTE 4:	The security aspects for Ambient IoT requires coordination with SA WG3.

NOTE 5:	The charging aspects for Ambient IoT will be studied by SA WG5.

NOTE 6:	The NAS based Congestion control is not in the scope of this study.

NOTE 7:	A dedicated Ambient IoT core network can be deployed by the operator to handle the traffic produced by ambient IoT devices.

-	An Ambient IoT device cannot always be assumed to be able to store dynamic protocol-related state information (e.g., counters, temporary identifiers) if the device is not energized and does not have any energy anymore (see R1-2407364 [X]).

# >>>>END OF CHANGES<<<<