3GPP TSG-WG SA2 Meeting #165	S2-2410552

Hyderabad, India, 14-18 October 2024	(revision of S2-240xxxx)

Source:	Lenovo

Title:	Interim Conclusion on Architecture to support topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This paper proposes interim conclusions for topology 2 architecture.

# 1. Introduction/Discussion

There are two major types of solutions in TR 23.700-13:

-	the one type (e.g. solutions 3, 22) doesn’t require AIoT NAS protocol to the AIoT device, i.e. the AIoT message are send directly between the AIoT device and the AF.

-	the other type does require AIoT NAS protocol between the AIoT device and the AIoTF.

It is assumed that there is an AIoT NAS protocol between the AIoT device and the AIoTF. In topology 2, the AIoT NAS PDUs are carried over the UE reader transparently to the AIoTF.

Based on the solutions in TR 23.700-13, there are several categories of solutions for Topology 2 for the transport of the AIoT NAS PDUs between the UE reader and the AIoTF:

- Control Plane (CP) based solution where the AIoT messages are send encapsulated over the NAS protocol between the UE reader and the AIoTF.

- User Plane (UP) based where the AIoT messages are send encapsulated over an IP-based protocol between the UE reader and the AIoTF in the Core Network.

- UP-based where the AIoT messages are send encapsulated over an IP-based protocol between the UE reader and the AF outside the network domain.

It is proposed the AIoT NAS layer is terminated at the Core Network, i.e. AIoTF, in order to allow the network operator to authenticate or verify the AIoT device.

In the cases where a single or few AIoT devices are located in the coverage of a UE reader, it is expected that few interactions between the UE reader and the AIoTF are needed. In such case, the CP-based solution may be efficient, i.e. as per bullet 1) above.

Furthermore, in the cases where multiple (e.g. dozens) of AIoT devices are located in the coverage of a UE reader, it is expected that may interactions between the UE reader and the AIoTF are needed. In such case it is more efficient to use the UP-based solution, i.e. as per bullet 2) above. The AIoTF determines whether to use CP-based and UP-based solutions for the transport or the AIoTF messages based on the expected traffic amount (e.g. if many interactions towards the UE reader are expected for many AIoT devices or multilpe AIoT operations, the AIoTF selects to use the UP-based solution).

This paper proposes conclusions for topology 2.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

### 8.1.2	Architecture to Support Topology 2

The following principles are agreed for the architecture to support topology 2:

-	The new core network function (AIoTF) as introduced for topology 1 is also used for topology 2.

-	The UE reader indicates its AIoT capabilities to the AMF (during the Registration procedure). The AMF authorizes the UE to act as UE reader (e.g. based on subscription or policy information).

-	The AMF (in case of CP-based solution) or the SMF (in case of UP-based sollution) selects an AIoTF and registers the UE reader with the AIoTF.

-	The radio resources required by the UE reader for the communicaiton with the AIoT devices are allocated and controlled by NG-RAN.

-	Regarding the transport of the AIoT messages between the UE reader and AIoTF:

-	Both CP-based and UP-based solutions are supported.

-	The AIoTF determines whether to use CP-based and UP-based solutions for the transport or the AIoTF messages based on the expected traffic amount (e.g. if many interactions towards the UE reader are expected for many AIoT devices or multilpe AIoT operations, the AIoTF selects to use the UP-based solution).

-	The CP-based solution includes enhanecments to speify a new NAS container type to be carried between the UE reader and the AioTF via the AMF.

-	The UP-based solution includes the enhancements to configure the UE with the AIoTF address (e.g. IP address or FQDN). The UE reader initates the UP communication to register with the AIoTF.

NOTE 1:	Stage 3 will specify the NAS container format for the CP-based solution and application protocol for the UP-based solution.

# * * * * End of changes * * * *