3GPP TSG SA WG2 Meeting #165                                                        	S2-2410593

Hyderabad, India, 14th Oct – 18th Oct, 2024

Source:	Samsung

Title:	Discussion on stale PDN connections at PGW.

Document for:	Discussion

Agenda item:	5.1(SAES)

# 1. Introduction

This paper discusses an issue observed in the field when the UE moves from source to target MME along with the SGW change, it is observed that the sessions remain stale/unusable in PGW and never gets deleted even though UE and MME would have released them for the scenario described in discussion section. After some point PGW will stop accepting new session establishment requests.

Following similar steps when UE performs intersystem change from 5GS to EPS there is a possibility of stale/unusable PDU sessions in SMF+PGW NF.

# 2. Discussion

Consider below steps:

1)	Pre-conditions: PDN-1, PDN-2 and PDN-3 are active in the system.

2)	UE performs local PDN connection release, and enters new area.

3)	As UE is outside registration area, the UE performs TAU procedure but indicate active PDN connections as PDN-1 and PDN-2.

4)	MME-2 performs context transfer procedure, MME-2 identifies that PDN-1, PDN-2 and PDN-3 are active at network side but PDN-3 is not active in the UE. MME-2 releases any resources of PDN-3 and does not take any further action.

5)	MME-2 sends context acknowledgment message and indicate there is a change in SGW to MME-1.

6)	Given SGW has changed, the MME-1 and SGW-1 release the resources of all active PDNs with no interaction with P-GW.

7)	The MME-2 triggers creation session request individually for PDN-1 and PDN-2.

8)	SGW-2 sends modify bearer request to PGW to create new association between new SGW-2 and P-GW individually for PDN-1 and PDN-2.

9)	PDN-1 and PDN-2 are active in the system.

Issue: Only PGW is having active PDN-3 session. This starts occupying unnecessary resources in the PGW and after some point the PGW will stop accepting new session establishment requests.

Potential solutions:

Proposal-1)

a) Source MME triggers delete session request:

Target MME(MME-2) sends context request message including complete TAU request message received from the UE. The TAU request message includes EPS bearer status of the UE. Thus, source MME can determine that PDN-3 is not active on the UE side and can trigger delete session request for PDN-3.

Drawback of above proposal: Source MME has to read the full TAU message every-time there is a context transfer. This is not desirable. This solution creates an impact to normal case to handle not so frequent/abnormal situation.

b) Target MME can indicate EPS bearer status to Source MME outside TAU request message.

Source MME can trigger delete session request but this will need interface level changes. Thus it is not preferred option.

Proposal-2)	Target MME triggering delete session request:

After understanding that PDN-3 is not active at the UE, but active at network after step-4 in above figure. MME-2 performs the create session request and establishes all the resources normally in network as if the PDN-3 is active at the UE side. Once the session is established between MME-2, SGW-2 and P-GW. MME triggers delete session for PDN-3 to remove the resources gracefully in the network.

This option has to unnecessarily exchange, messages to create the session in MME-2 & SGW-2; and modify the session in P-GW for PDN-3. But such message exchanges will happen in abnormal situation and have less implementation impacts to the system.

# 3. Conclusion & proposal

We propose to discuss above scenario, proposed solutions and agree the changes proposed in CR S2-241059.

# 4. Reference

Please refer to below section, for above steps:

#### 5.3.3.1	Tracking Area Update procedure with Serving GW change

.