3GPP TSG SA WG2 #165	            S2-2410619

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-2410217)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1480 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#8 conclusion in TR 23.700-77, the architecture for avatar communications needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of IMS DC based avatar communication is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS DC based avatar communication cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

AC.X	Support of Avatar Communication

AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) and the associated metadata are provided to the UE, so that the user of the UE selects one Avatar ID among the received Avatar IDs that are allowed to the user. The Avatar ID List is provided from the Base Avatar Repository (BAR) to DCSF, and is transferred to the UE through bootstrap data channel.

-	Avatar ID List is the list of Avatar ID(s) that are identifying the Avatar Representation allowed for the user, and the associated metadata (e.g. the data required for the user to select one Avatar to use in a call such as thumbnails, location, time, country, etc).

After selection of one Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar ID is provided from the BAR to MF via DCAS, and it is transferred to the local UE and/or to the remote UE according to the rendering mode.

-	Avatar Representation is the data which is generated from the inputs such as captured data from camera and other sensor information, and it is used as a base of Avatar for rendering.

The avatar media is rendered by processing the Avatar Representation with the animation data. The avatar media rendering process is performed by the local UE, the remote UE, and/or the network according to the rendering mode.

-	Rendered Avatar media is generated by processing the Avatar Representation with the animation data (e.g. audio, video, and/or metadata such as action and facial expression data).

The details of enhanced IMS data channel architecture and procedures are described in following sub-clauses.

AC.X.2	Architecture

Figure AC.X.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Represenation and its associated Avatar ID. One ore more Avatar Representation(s) is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR provides Avatar ID List to the DCSF via DC Application Server. The Avatar ID List is the list of Avatar ID(s) and the associated metadata (e.g. the data required for the user to select one Avatar to use in a call) that is allowed to use for the user.

-	The BAR sends Avatar Representation to the MF via DC Application Server. The Avatar Representation is the data which is generated from the inputs such as captured data from camera and other sensor information, and it is used as a base of Avatar for rendering.

Editor's note:	The interface(s) between the BAR and the DC Application Server is FFS.

DCSF:

-	The DCSF receives event control notifications from the IMS AS with the indication that it is allowed to access to BAR for this user and the requests of delivery of Avatar ID List and/or Avatar Representation.

-	Upon receving the indication of BAR access allowance and the requests of delivery of Avatar ID List from the IMS AS, the DCSF retrieves the Avatar ID List from the BAR via DCAS,  and sends it to MF via MDC1.

-

IMS AS:

-	Based on the data channel media description information and the user subscription data, the IMS AS validates whether the user of the UE is allowed to use the data channel based avatar communication and whether it is allowed to access to the BAR for this user of the UE. If allowed, the IMS AS indicates to the DCSF about the BAR access allowance and the requests of delivery of Avatar ID List and/or Avatar Representation.

MF:

-	The MF receives the Avatar ID List from the DCSF via MDC1 and sends it to the UE through bootstrap data channel.

-	The MF receives the Avatar Representation from the BAR via DCAS by using MDC2.

-	The MF supports Avatar media processing. In case of UE based rendering mode, the MF transfers the Avatar Representation to the UE. In case of Network based rendering mode, the MF processes the Avatar Representation with the animation data (e.g. audio, video, and/or metadata such as action and facial expression data) to generate the avatar media. The MF transfers the rendered avatar media to the UE.

DC AS:

-	The DC AS retrieves the Avatar ID List from BAR and sends it to DCSF via DC3 or DC4 during bootstrap data channel setup procedure for Avatar communication as described in AC.X.3.1.

-	The DC AS retrieves the Avatar Representation from BAR and sends it MF via MDC2.

-	The DC AS provides the controls of Avatar media processing at MF in case of Network based rendering mode.

# * * * * End of changes * * * *