3GPP TSG-WG SA2 Meeting #165	S2-2410778

# 14 - 18 October, 2024, Hyderabad, India									 									(revision of S2-2410083)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5714 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Enhancement for BDT with Energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, TOYOTA MOTOR CORPORATION |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-66 v19.0.0 has documented to make policy decisions based on operator policy for key issue#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. The following information is stored as part of the subscription data in the UDM/UDR: - The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy. This contribution provides to enhance BDT considering the Energy Consumption information, based on the solution #25. Note that the detail of Energy consumption information is described in CR5712. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an Enhancement for BDT considering the Energy Consumption information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for BDT with Energy Consumption information in R19 Energy Efficiency study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new), 5.X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5055 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***1st Changes (All new texts)***

5.X	Support for Energy Efficiency and Energy Saving

5.X.Y	BDT based on network energy related information

5GS supports BDT (background data transfer) policy selection and re-negotiation process by introducing network energy related information. The AF may requests energy related criteria for BDT policy. The PCF may retrieve energy related information from Energy Information Function (EIF) to make policy decisions. The PCF may also trigger the re-negotiation of BDT policy with the AF based on the notification of energy related information.

# *** End of Changes ***