3GPP TSG-WG SA2 Meeting #165	S2-2410962

# Hyderabad, India, October 14-18, 2024			(revison of S2-2410640, 0015)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0589 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reports |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, Nokia, Nokia Shanghai Bell, CEWiT |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_DLPMR |  |  |  |  |  |  | Date: |  | 2024-10-04 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | TEI19_DLPM WID has been agreed for Rel-19. Changes add the feature to Rel-19. Since Release 16, the LMF can request the NG-RAN to report periodic measurements over NRPPa by using the Measurements procedures specified in TS 38.455 sections 8.5.2 and 8.2.3. LMF could use this periodical reporting of measurements to generate location estimates of the UE’s location periodically. However, there is no SA2 procedure which could leverage this periodical received location measurement to generate location estimates periodically. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new procedure Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reportrs |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature is not added based on TEI19_DLPM. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.x (New), 6.3.x.1(New), 6.3.x.2 (New), 6.4.x (New) |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 6.3.x	Deferred 5GC-MT-LR Procedure for Periodic Location Events based on NRPPa Periodic Measurement

### 6.3.x.1	Initiation and Reporting of Periodic Events based on NRPPa Periodic Measurement

Figure 6.3.x.1-1: Deferred 5GC-MT-LR for Periodic Events based on NRPPa Periodic Measurement

1. Deferred 5GC-MT-LR steps 1-14 are executed as described in clause 6.3.1. AF, LCS clients or NF triggers periodic location event.

2. LMF performs one or more of the positioning procedures as it is described in step 15 in clause 6.3.1. LMF based on UE capability information received in this step and based on configuration decides to perform NRPPa Periodic Measurements. Once LMF determines to use NRPPa periodic measurement, it may perform Positioning Information Exchange based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN (e.g: as described in clause 8.2.6.2 in TS 38.455)

3. Deferred 5GC-MT-LR steps 18-21 are executed as described in clause 6.3.1 with the following changes

- The NRPPa periodic indication is added to step 18-20 to indicate Periodic Location Estimates are generated based on NRPPa Periodic Measurement. (H)GMLC stores NRPPa periodic indication.

4a-b. LMF performs NRPPa Periodic E-CID Measurement Initiation Request (clause 8.2.1.2. in TS 38.455[15]) or LMF performs NRPPa Periodic Measurement Request (clause 8.5.1.2. in TS 38.455[15]). LMF uses the time interval between successive location reports and the total number of reports received in step 1 to setup NRPPa periodical report procedure.

5. The AMF forwards the message to NG-RAN.

6-7a. NG-RAN returns with E-CID Measurement Initiation Response (clause 8.2.1.2. in TS 38.455[15]).

6-7b. NG-RAN returns with Measurement Response (clause 8.5.1.2. in TS 38.455[15]).

8-9a. LMF is receiving E-CID Measurement Report from NG-RAN periodically (clause 8.2.3.2 in TS 38.455[15]). LMF uses Measurement Information to calculate Location Result.

8-9b. LMF is receiving Measurement Report from NG-RAN periodically (clause 8.5.2.2 in TS 38.455[15]). LMF uses Measurement Information to calculate Location Result.

10. LMF invokes Nlmf_Location_EventNotify service operation as described in step 28 in clause 6.3.1 in order to transfer location information to VGMLC or (H)GMLC.

11. Deferred 5GC-MT-LR steps 29 is executed as described in clause 6.3.1.

12a. Deferred 5GC-MT-LR steps 30a is executed as described in clause 6.3.1.

12b-1. Deferred 5GC-MT-LR steps 30b-1 is executed as described in clause 6.3.1.

12b-2. Deferred 5GC-MT-LR steps 30b-2 is executed as described in clause 6.3.1.

12c. Deferred 5GC-MT-LR steps 30c is executed as described in clause 6.3.1.

Steps between 8 and 12c are executed periodically.

LMF before step 4 invokes Namf_EventExposure_Subscribe to receive events from serving AMF about NG-RAN change using “RAN_Node” as LocationFilter.

### When LMF receives from NG-RAN error causes (e.g. due to UE mobility to another NG-RAN node or UE is released to CM-IDLE mode) that NG-RAN node is unable to provide the measurement results then LMF considers that the NRPPa periodic reporting is terminated by NG-RAN. The LMF may re-trigger a request for NRPPa periodical reporting, considering the event notification from AMF which is subscribed by the LMF as described above.

### 6.3.x.2	Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC

Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC based on NRPPa Periodic Measurement executed as decribed in 6.3.3 with the following changes:

- Steps from 6 to 9 are not executed based on NRPPa periodic indication provided by VGMLC in step 4. (ie.: UE’s related procedures are not executed)

- When in step 5 LMF receives Namf_Location_CancelLocation, LMF stops NRRPa periodic measurement report by requesting E-CID Measurement Termination (clause 8.2.4.2 in TS 38.455[15]) or Measurement Abort (clause 8.5.4.2 in TS 38.455[15]) and unsubscribe for AMF event.

* * * * Next Change * * * *

### 6.4.x	LMF Change Procedure for NRPPa Periodic Measurement

The LMF Change procedure supports change of a serving LMF during a deferred 5GC-MT-LR procedure for Periodic Location Events based on NRPPa Periodic Measurement. LMF change may be initiated due to new NG-RAN or new serving AMF.

Figure 6.4.x-1: Change of serving LMF for Periodic Events based on NRPPa Periodic Measurement

1.	NG-RAN or AMF change happens, e.g. due to UE mobility in CM-CONNECTED state. The (new) AMF may trigger the event notification towards LMF (e.g. based on the event subscription as described in clause 6.3.x.1 above).

When there is a change of AMF, the new AMF receives all event subscriptions from old AMF. The new AMF informs LMF about the new AMF address if the LMF has subscribed for the NG-RAN change event as described in clause 6.3.x.1 and the LMF reinitiates NRPPa Periodic Measurement Initiation towards new serving AMF with the remaining number of reports.

2.	[Conditional] LMF1 may evaluate and determine that it is unsuitable or unable to support location for the current UE NG-RAN and determines LMF2 as being a more suitable LMF. LMF1 may already have LMF2 information e.g. from previous NRF discovery or locally configured, otherwise LMF1 queries. LMF1 selects new LMF based on NG-RAN location based and other information as listed in clause 5.1.

3.	[Conditional] LMF1 invokes an Nlmf_Location_LocationContextTransfer Request service operation towards LMF2 to provide the current location context of the UE. The service operation includes the AMF identity and all the information originally received by LMF1 for the periodic location request either from the AMF or from an earlier serving LMF according to this procedure. LMF2 invokes Namf_EventExposure_Subscribe to receive events from serving AMF about NG-RAN change.

4.	[Conditional] LMF2 informs LMF1 of the location context transfer operation results. LMF2 performs NRPPa Periodic Measurement Initiation.

5. 	[Conditional] LMF1 releases all resources for the procedure and unsubscribe from AMF event.

* * * * End of Changes * * * *