3GPP TSG-WG SA2 Meeting #165	S2-2411052

# 14 - 18 October, 2024, Hyderabad, India										(revision of S2-2410781)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5713 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Enhancement for AF influence on traffic routing with Energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, TOYOTA MOTOR CORPORATION, China Mobile, CATT, ZTE, InterDigital, Nokia, ETRI, Deutsche Telekom, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-66 v19.0.0 has documented the UP path adjustment in.the conclusion for key issue#3: 5GS enhancements for network energy saving and efficiency. 2) For enhancements on existing operations and procedures for energy saving and energy efficiency: - UP path of PDU session may be adjusted. This contribution provides to UP path adjustment considering energy related information. Note that the detail of energy related information is described in CR5628(S2-2409638). This paper is merge version of S2-2410258, S2-2410306, S2-2410369 and S2-2410346. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UP path adjustment considering energy related information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for UP path adjustment considering energy related information in R19 Energy Efficiency study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***1st Changes ***

5.X.Y	UP path adjustment based on network energy related information

SMFs may consider energy related information with NRF profile to select suitable UPF(s) for a PDU session.

Editor’s note: Further details of UP path adjustment consider energy related information are FFS.

Editor’s note: Considering energy related information from EIF and/or OAM for UP path adjustment of PDU session is FFS.

# *** End of Changes ***