3GPP TSG-WG2 Meeting #165	S2-2411158

# Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0144 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding C2 Communication Reliability with Redundant User Plane Paths to Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion in clause 8.1.3 of TR 23.700-59, it is agreed to capture Solution#14 for C2 communication reliability as Informative Annex in TS 23.256 without details on application layers aspects. This paper introduces the respective changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added content on C2 communication reliability with redundant user plane paths to Annex in TS 23.256. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | C2 communication reliability with redundant user plane paths is missing and the specification is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

# A.X	C2 Communication Reliability with Redundant User Plane Paths

Annex F in TS 23.501 [2] specifies the mechanism that can be used to achieve redundant user plane paths. The similar mechanism can also be used to enchance the End-to-End C2 communication reliability in the scenarios where:

-	a UAV has more than one SIM from a single operator, and the UAV use them to set up redundant C2 connections to the UTM;

-	a UAV is present in the area where RAN coverage allows the UAV to connect to multiple gNBs simultaneously. The selection of gNBs can be distinct from each other.

NOTE 1:	No impact on RAN specifications is expected from the reuse and enhancements of the existing mechanisms.

NOTE 2:	It is expected that UAV’s application layer handles the redundant connections by means outside the scope of the 3GPP.

NOTE 3:	If this mechanism is being used, the operator needs to ensure that a UAV with multi-SIM single operator is supported, and the same UAV ID been used in multiple SIM with single operator.

For a UAV with redundant connections to different gNBs to request the establishment of PDU Sessions that use independent RAN and CN network resources, the following per-conditions needs to be met:

-	separate gNBs nodes are required to achieve user plane redundancy over the 3GPP system. It is, however, up to operator’s deployment and configuration to ensure that separate gNBs are available and used;

-	a UAV is able to establish two (or multiple) independent connections to two (or multiple) independent gNBs;

-	RAN coverage is so that the UAV is able to connect to multiple gNBs located in the target area. To ensure that the UAV connects to different gNBs, the gNBs need to operate in such a way that allows the UAV to select and connect to different gNBs (e.g. gNB frequency allocation supports such configurations/scenarios);

-	deployment of the CN and UPF in particular supports configuration/scenarios with redundant user plane paths;

-	the underlying transport network’s topology is aligned with the RAN and UPF deployments and supports configurations/scenarious with redundant user plane paths;

-	the physical network topology and geographical distribution of network functions also supports configuration/scenarios with redundant user plane paths to the extent deemed necessary by the operator;

-	operation aspects of the redundant user plane paths (for instance, independent power supplies) are made independent to the extent deemed necessary by the operator.

Figure A.X-1 provides the architecture view. The UAV is connected to gNB1 and gNB2, respectively. Connection1 sets up a PDU Session via gNB1 to UPF1, while Connection2 sets up another PDU Session to UPF2 via gNB2. UPF1 and UPF2 interface with SMF1 and SMF2, respectively, and UPF1 and UPF2 provide the UAV’s connections to the same UTM.

Figure A.X-1: Architecture for achieving C2 communication reliability via redundant (disjoint) user plane paths

To ensure that the UAV selects different gNBs, UAV’s connections and cells of the gNBs are grouped into more than one group (so-called Reliability Group) and the UAV selects the cells from the same realiability group as the connection. Figure A.X-2 shows an example, where Connection 1 and the cells of gNB1 belong to Reliability Group A and Connection 2 and the cells of gNB2 belong to Reliability Group B.

Figure A.X-2: Reliability group-based redundancy concept in RAN

To determine Reliability Groups for the UAV, the methods shown in Figure F-2 in Annex F of TS 23.501 [2] can be used. A Reliability Group of each of the connections is represented using the existing parameters and sent from the AMF to the RAN during the RAN context establishment; thus, each of the gNBs knows Reliability Groups of the UAV’s connections. Reliability groups of gNB cells are pre-configured via the O&M, and gNBs can either learn the Reliability Groups of the neighbouring cells as the Xn connectivity is set up or also configured by the O&M.

If the UAV leaves the cells of its currently connected Reliability Groups or if the quality of the connection drops below a certain threshold, the connections can be transferred to cells that belong to another Reliability Group. In such case, the gNB initiates the handover to cells in an appropriate Reliability Groups whenever such cells are available.

>>>> End of Changes <<<<